PLEXTALK Linio Pocket

version 2

User's Guide

©2012 Shinano Kenshi Co., Ltd.

# IMPORTANT NOTICES

## FCC NOTICE

Declaration of Conformity

Model Number: PTP1/LINIO

Trade Name:  PLEXTALK

Responsible party: Shinano Kenshi Co., Ltd.

Address: 1078, Kami-maruko, Ueda-shi, Nagano-ken 386-0498 JAPAN

This device complies with Part 15 of the FCC Rules; Operation is subject to the following two conditions:

(1) This device may not cause harmful interference, and

(2) This device must accept any interference received, including interference that may cause undesired operation.

This equipment has been tested and found to comply with the limits for a Class B digital device, pursuant to part 15 of the FCC Rules. These limits are designed to provide reasonable protection against harmful interference in a residential installation. This equipment generates, uses and can radiate radio frequency energy and, if not installed and used in accordance with the instructions, may cause harmful interference to radio communications. However, there is no guarantee that interference will not occur in a particular installation. If this equipment does cause harmful interference to radio or television reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by one or more of the following measures:

•Reorient or relocate the receiving antenna.

•increase the separation between the equipment and receiver.

•Connect the equipment into an outlet on a circuit different from that to which the receiver is connected.

•Consult the dealer or an experienced radio / TV technician for help.

CAUTION: Any changes or modification not expressly approved by the party responsible for compliance could void the user's authority to operate the equipment.

## Canadian Department of Communications Statement

This digital apparatus does not exceed the Class B limits for radio noise emissions from digital apparatus set out in the Radio Interference Regulations of the Canadian Department of Communications.

This class B digital apparatus complies with Canadian ICES-003.

Cet appareil numerique de la classe B est conforme a la norme NMB-003 du Canada.

CAUTION:  Use of controls or adjustments or performance of procedures other than those specified herein may result in hazardous radiation exposure.

# Table of Contents

IMPORTANT NOTICES	2

FCC NOTICE	2

Canadian Department of Communications Statement	2

Table of Contents	4

1. Safety precaution and legal information	11

1.1. Important safety information	11

1.2.  Legal and copyright information	15

1.2.1. Copyright information about this guide	15

1.2.2. Restrictions on copying or recording	16

1.3. Licenses and trademarks	16

2. Getting started	18

2.1. Terminology and conventions used	18

2.2. What you get in the box	18

2.3. Description of the unit	18

2.3.1. Front face	19

2.3.2. Top edge	20

2.3.3. Left edge	20

2.3.4. Right edge	21

2.3.5. Bottom edge	21

2.3.6. Back face	21

2.4. Inserting the battery	21

2.5. Charging PLEXTALK	22

2.6. Maintenance and battery care	23

2.6.1. Unit care	23

2.6.2. Battery care	23

2.6.3. Crossed-out dustbin mark	24

2.7. Turning PLEXTALK on and off	24

2.8. Key Describer function	25

2.9. Key Lock function	25

2.10. Inserting and ejecting SD card	25

3. Introduction to the menu system	27

3.1. Main menu	27

3.2. Accessing and navigating the menu	27

3.3. Shortcut for the menu selection	28

4. Setting up your PLEXTALK	29

4.1. Setting the date and time	29

4.2. Selecting your preferred synthetic voice	29

4.3. Setting the text to speech pitch	29

4.4. Selecting a preferred waiting sound	30

4.5. Selecting the Key Press Confirmation Sound	30

5. Power management features	31

5.1. Battery low warning	31

5.2. Automatic power off function	31

5.3. Compulsory power off function	31

5.4. Sleep timer function	31

5.4.1. Setting the sleep timer	32

5.4.2. Cancelling the sleep timer	32

6. Playing features common to all formats	33

6.1. Start and stop playback	33

6.2. Fast forward or fast rewind	33

6.3. Changing the playback volume	34

6.4. Changing the playback speed	34

6.5. Changing the audio tone	34

6.6. Changing the guide volume	35

6.7. Changing the guide speed	35

6.8. Accessing the Navigation menu	35

6.9. Selecting the media and title	36

6.10. Title navigation	36

6.10.1. How to navigate the title	36

6.10.2. How to play music	37

6.11. Go to a title directly	37

6.12. Switching sorting order of titles	38

6.13. Getting information about media and title	38

6.14. Getting information on the current status	39

6.15. Selecting a playback mode	39

6.16. Using bookmarks	39

6.16.1. Types of bookmark	39

6.16.2. Inserting bookmarks	40

6.16.3. Navigating to bookmarks	40

6.16.4. Clearing bookmarks	41

6.16.5. Turning on and off Voice Bookmark speech	42

6.17. Understanding how PLEXTALK groups files and folders	42

7. Playing DAISY titles	43

7.1. Heading navigation	43

7.2. Moving to the beginning or end of the title	43

7.3. Page navigation	44

7.4. Group navigation	44

7.5. Phrase navigation	44

7.6. 10 minutes skip, 30 seconds skip	45

7.7. Navigating by screen, paragraph, line, sentence, spell, word or character in case of DAISY text mode	45

7.8. Navigating by percentage	45

7.9. Navigating by keyword	45

7.10. Playback mode	46

7.11. Multimedia DAISY playback setting	47

8. Playing "Recordings"	48

9. Playing Audio Files or Music CDs	49

9.1. Track navigation	49

9.2. Album navigation	50

9.3. 10 minutes skip, 30 seconds skip	50

9.4. Playback mode	50

9.5. Announcing albums and tracks by name or number	51

9.6. About playing sequences	51

9.7. Restrictions on Audio Files	51

10. Playing Text Files	52

10.1. Navigating by screen, paragraph, line, sentence, spell, word or character	52

10.2. Navigating by percentage	52

10.3. Navigating by keyword	53

10.4. Navigating by page	54

10.5. Navigating by heading	54

10.6. Playback mode	54

11. Playing Audio Book	55

12. Recording a DAISY title	56

12.1. Basic information about recording with PLEXTALK	56

12.2. Recording hardware	56

12.3. Recording media	57

12.4. Recording states	57

12.5. DAISY editing mode	57

12.6. Basic recording technique	58

12.7. Muting the guide speech when recording	58

12.8. Connecting and selecting an input source	59

12.9. Selecting a recording mode and sound quality setting	60

12.9.1. About recording modes and sound quality	60

12.9.2. Selecting a preset recording mode	61

12.9.3. Selecting the Custom recording mode	62

12.10. Selecting the title to record into	63

12.11. Setting the recording level	63

12.12. Using the level monitor	64

12.13. Turning the recording monitor on and off	65

12.14. Getting recording information	65

13. Editing a DAISY title	66

13.1. Editing heading markers during recording	66

13.2. Setting a heading after recording	66

13.3. Cancelling a heading setting after recording	67

13.4. Removing a section or multiple sections	67

14. Media management features	69

14.1. Erasing an SD card, USB device or Internal Memory	69

14.2. Creating a new title	69

14.3. Deleting a title	70

14.4. Deleting a file	70

14.5. Preparing to edit a DAISY title	70

14.6. Building a DAISY title	71

15. Using PLEXTALK's Voice Memo functions	73

15.1. Recording a new Voice Memo	73

15.2. Reviewing existing Voice Memos	73

15.3. Back up Voice Memos to SD card	74

15.4. Deleting Voice Memos	74

16. Backup	75

16.1. General backup information	75

16.2. Back up Voice Memos	75

16.3. Backup a title	76

16.4. Backup time for music CD	76

17. Entering text	78

17.1. Entering alphabet by ABC mode	78

17.2. Entering numbers by number mode	79

17.3. Entering symbols by Symbol table	79

17.4. Moving entering position	80

17.5. Enter space, linefeed and erase characters	81

18. Editing text files	82

18.1. Creating a new text file	82

18.2. Editing an existing text file on the SD card, USB device or Internal Memory	82

18.3. Saving the edited text	82

18.4. Exiting text editing	83

18.4.1. Saving and exiting text editing	83

18.4.2. Exiting text editing without saving the text	83

18.5. Renaming the file	84

18.6. Deleting current line	84

18.7. Deleting all text	84

19. Calendar function	85

19.1. Select the date in Calendar	85

19.2. Add an event	86

19.3. Reminder of the event	87

19.4. Edit the event	87

19.5. Delete the selected event	87

19.6. Calendar Menu	87

19.6.1. Delete all events	87

19.6.2. Delete the events of more than three months ago	88

20. Alarm function	89

20.1. Configure alarm notification	89

20.1.1. Setting or changing alarm	89

20.1.2. Set alarm on or off	90

20.1.3. Clear alarm setting	90

20.2. Reminder of the alarm	90

21. Network settings	92

21.1. Connecting to the wireless network	92

21.1.1. Enable wireless network device	92

21.1.2. Confirm available wireless network and connect	92

21.2. How PLEXTALK connects to a network	94

21.3. Advanced Network settings	94

21.3.1. Confirming network information	94

21.3.2. Loading network setting	95

21.3.3. Saving the network setting	95

21.3.4. Adding wireless network setting	96

21.3.5. Change or delete wireless network setting	96

21.3.6. Network folder setting	98

21.3.7. Initializing network settings to default	99

22. Podcast receiver	100

22.1. Registering podcast feeds	100

22.1.1. Registering podcast feeds using the "PLEXTALK Transfer" software	100

22.1.2. Registering podcast feeds using podcast receiver software	101

22.2. Listening to Podcasts	102

22.3. Navigating by Episode, 10 minutes or 30 seconds	102

22.4. Obtaining the latest episode list	103

22.5. Download all Episodes of the feed	103

22.6. Notice during the download	103

23. Web radio streaming	105

23.1. Adding web radio station to the playlist	105

23.2. Listening to web radio	105

24. Network folder function	106

24.1. Set LAN manager authentication level of windows PC	106

24.2. Start Network Folder	107

24.3. Log on to the Network Folder at your computer	108

24.4. Finish Network Folder	108

25. DAISY Online	109

25.1. Online service setting	109

25.1.1. Loading online service setting	109

25.1.2. Confirming and changing online service setting	110

25.1.3. Deleting online service setting	111

25.1.4. Announcing by recorded voice or synthetic voice	111

25.2. Bookshelf	112

25.2.1. Playing a title in Bookshelf	113

25.2.2. Playback options: streaming and downloading	113

25.2.3. Downloading a title	113

25.2.4. Returning a title	114

25.2.5. Selecting a title by category	114

25.2.6. Transferring titles category from "New titles" to "Issued titles"	114

25.3. Service Announcements	115

25.3.1. Listening to Service Announcement	115

25.3.2. Deleting Service Announcements	116

25.4. Bookshelf and Announcement menu	116

25.4.1. Updating all bookshelves	116

25.4.2. Selecting online service and updating Bookshelf	117

25.4.3. Updating Service Announcements	117

25.5. Online service menu	118

26. Using PLEXTALK as an external drive	120

26.1. What can PLEXTALK be used for when connected to a personal computer?	120

26.2. Selecting SD card or Internal Memory as an external drive	120

26.3. Connecting PLEXTALK to a computer	120

26.4. Disconnecting PLEXTALK from a computer	121

27. Other useful features	122

27.1. Accessing the built in help file	122

27.2. Checking system properties	122

27.3. "Skippable" settings	122

27.4. Network update	123

27.5. Deleting authorization keys	123

27.6. Reset PLEXTALK to factory defaults	124

Appendix A: Main Specifications	126

General specifications	126

Audio features	127

Main functions	128

Battery specifications	129

Memory Card Drive of a personal computer via USB cable	130

Wireless communication function	130

Appendix B: Menu Hierarchy	131

Appendix C: Troubleshooting	135

General operating problems	135

Spoken errors during DAISY playback	136

Errors using the Voice Memo	136

Errors when using PLEXTALK as an SD card drive on a computer	136

Appendix D: Technical data about recording modes and sound qualities	138

1. Recording mode settings	138

2. Settings for preset recording modes	138

3. Available settings for Custom mode	139

4. Available recording time for the specified sound qualities	139

Appendix E: Questions and Answers on Network setting	141

Appendix F: List of entering characters	143

# 1. Safety precaution and legal information

## 1.1. Important safety information

Failure to follow these safety instructions could result in fire, electric shock, or other injuries, or damage to PLEXTALK or other property. Read all the safety information below before using PLEXTALK.

The symbols below differentiate the level of danger.

! DANGER

This symbol denotes that death or serious injuries may directly result from improper use.

! WARNING

This symbol denotes that death or serious injuries may result from improper use.

! CAUTION

This symbol denotes that bodily injuries or damage to property may result from improper use.

< Traffic accident >

! DANGER

Do not use while walking outdoor. Otherwise, it may cause traffic accident.

! DANGER

Do not use while riding or driving. Otherwise, it may cause traffic accident.

< Emergency >

! DANGER

If the equipment smokes or gives off a strange smell, immediately turn off the power, disconnect the power plug from the power outlet, remove the battery and contact your dealer. Be careful burn injury because the equipment and battery may still be high-temperature.

< Battery >

! WARNING

If the battery pack leaks, immediately remove it from power outlet and fire, and contact your dealer. The vapors from leaking battery pack fluid may ignite or explode. If battery fluid gets into your eyes, wash them with water and consult a physician immediately. If the fluid of battery contacts your skin, immediately flush the contacted area thoroughly with clean water and conslut a phisician. If the fluid gets on your clothing, flush the area with water.

! WARNING

Do not throw the equipment or battery pack into the fire. Do not put the equipment or battery pack into the microwave ovens or other heating appliance. Otherwise, it may cause fire, electric shock or malfunction.

! WARNING

Do not give hard shock to it or drop it from high place. Otherwise, it may cause fire, electric shock or malfunction.

! WARNING

If not using extended period, disconnect the power plug from the power outlet and remove the battery. Otherwise, it may cause the battery pack to leak.

< Do not insert foreign objects >

! WARNING

Do not insert liquids or foreign objects. Do not allow liquids or foreign objects to come in contact with the PLEXTALK. Otherwise, it may cause fire, electric shock or malfunction.

If liquids or foreign object enter the PLEXTALK, turn off the power immediately, disconnect the power plug from the power outlet, remove the battery and contact your dealer.

Especially be careful the followings.

- Do not use in a bath room, in a rain or under moist conditions.

- Do not spill liquids into the equipment.

- When you clean the equipment, do not spill or spray the detergent liquid.

- Do not put a metal piece into the inside of SD card slot or battery housing.

- Do not use the SD card which is stuck with labels or stickers.

< Power >

! WARNING

Only use the specified battery, specified AC adaptor and specified power cable that was supplied with your PLEXTALK. Otherwise, it may cause fire, electric shock or malfunction.

! WARNING

Do not connect with the unspecified power source and voltage. Specification is AC100-240V  50/60Hz. Otherwise, it may cause fire, electric shock or malfunction.

! WARNING

Do not insert or remove the power plug with wet hands. Otherwise, electrical shock may occur.

! WARNING

Do not damage the power cable. Do not remove the power cable with pulling the adapter cable or do not place heavy object on the adapter cable and power cable. If the cable get damaged, it may cause fire or electric shock.

! WARNING

Make sure that the PLEXTALK, mains adapter and mains plug are securely connected. Otherwise, it may cause fire or electric shock.

! WARNING

Wipe off any dust that accumulates on the power plug regularly. When wiping, turn off the power and then disconnect the power plug from the power outlet. Otherwise, it may cause fire or electric shock.

! WARNING

Do not put too many plugs in one outlet. Otherwise, it may cause fire or electric shock.

! WARNING

If thunderous, disconnect the power plug from the power outlet. When the lightning strikes, it may cause fire or electric shock.

< Repair >

! WARNING

Don't disassemble, repair or remodel the equipment. Otherwise, it may cause fire, electric shock or malfunction. Ask your dealer for the repair.

! WARNING

If the equipment broken down, turn off the power immediately and contact your dealer. It may cause fire or electric shock if you continue to use.

< Usage Environment >

! WARNING

Do not use, store, or leave the equipment in high-temperature environment. Otherwise, it may cause fire, electric shock or malfunction.

Do not leave in areas under strong direct sunlight.

Do not leave in sun-heated vehicle.

Do not leave in hot areas such as near fire or appliance which emits high heat.

! CAUTION

Do not use the equipment in extremely humid, dusty or hot areas.  Otherwise, it may cause malfunction.

< Electric wave / Electromagnetic wave >

! WARNING

Do not use the PLEXTALK near wearers of pacemaker.

! WARNING

If the medical facility or airplane has specified zones where use or possession of signals of mobile devices is prohibited, do not use the PLEXTALK.

! CAUTION

Do not use the PLEXTALK near machinery occurring with a strong electromagnetic wave. Otherwise, it may cause malfunction.

< Adverse effect on human body >

! WARNING

If the headphone cord or the strap squeezes the neck, it may cause suffocation.

! WARNING

Store the SD card or small accessories out of reach of small children to avoid the the drinking by mistake. In addition, be careful to the drinking by mistake by your  pets.

! CAUTION

When you use the equipment for a long time or while you charge the equipment, the equipment might rise in temperature. Do not keep the equipment in touch with the same place of the skin for a long time because there is fear of the low temperature burn.

! CAUTION

Do not hear at full blast more than needs. Otherwise, it may cause impair hearing.

! CAUTION

Do not use the headphone for a long time. Otherwise, it may cause impair hearing.

! CAUTION

The USB cable terminal has sharp edge. Be careful when you touch the sharp edge. It may cause injury.

< Cleaning >

! WARNING

Before cleaning, turn off the power and then remove the AC adaptor and the USB cable from the PLEXTALK. Otherwise, it may cause fire, electric shock or malfunction.

! WARNING

Do not spray solvent directly onto the case. Otherwise, it may cause fire, electric shock or malfunction.

! CAUTION

Wipe dust and dirt from the PLEXTALK with a soft cloth dampened slightly with a diluted neutral detergent, then wipe dry with a cloth. If you wipe the equipment with alcohol, paint thinner or benzine,  the surface of the equipment may be worn or torn .

< Other malfunction factors >

! CAUTION

The PLEXTALK's SD card slot is only compatible with SD or SDHC cards. Inserting other memory cards will cause malfunction.

! CAUTION

Insert the SD card in the right direction. Do not use excessive force. Otherwise, it may cause malfunction.

< Disposal >

! WARNING

Dispose the equipment and the battery pack following the disposal method which local government established.

## 1.2.  Legal and copyright information

This chapter contains important information regarding copyright, licenses and trademarks. It also outlines the restrictions, which apply when using the PLEXTALK for copying or recording digital sound. You are strongly advised to read this chapter thoroughly before continuing.

### 1.2.1. Copyright information about this guide

Shinano Kenshi reserves the right to modify the products described in this guide at any time without prior notice.

Shinano Kenshi makes no representation or warranties with respect to the contents hereof and specifically disclaims any implied warranties of merchantability or fitness for any particular purpose. Further, Shinano Kenshi Co., Ltd. reserves the right to revise this guide and to make changes in its content, without obligation to notify any person or organization of such revision or change.

This guide is copyright. It may not be copied, photocopied, translated, or reduced to any electronic medium or machine-readable form without prior permission from Shinano Kenshi.

Manual copyright 2012 Shinano Kenshi Co., Ltd.

### 1.2.2. Restrictions on copying or recording

Before using your PLEXTALK, you should review the following important information to ensure that you understand the legal restrictions that apply to its use for certain operations.

< General information >

This PLEXTALK device is designed to help you record and reproduce sound works, to which you own the copyright, or where you have gained permission from the copyright owner or the rightful licenser. Unless you own the copyright or have gained the appropriate permission from the copyright owner or the rightful licenser, your unauthorized recording, reproduction or distribution thereof, may result in severe civil and criminal penalties under copyright laws and international copyright treaties. If you are uncertain about your rights, contact your legal advisor.

Under no circumstances will Shinano Kenshi be responsible for the consequences of any illegal copying performed using this PLEXTALK recorder.

< Music CDs >

Copies of Music CDs are for individual enjoyment and, under the Copyright Act, cannot be used otherwise, without the authorization of the copyright holder.

## 1.3. Licenses and trademarks

All licenses and trademarks are the property of their respective owners.

Windows 2000, Windows XP, Windows Vista, Windows 7 and Windows 8 are the trademarks of Microsoft Corporation.

Foxit Embedded PDF SDK Copyright (c)2012, Foxit Corporation

www.foxitsoftware.com

Copyright information about Ogg Vorbis

This software is provided by the copyright holders and contributors "as is" and any express or implied warranties, including, but not limited to, the implied warranties of merchantability and fitness for a particular purpose are disclaimed. In no event shall the foundation or contributors be liable for any direct, indirect, incidental, special, exemplary, or consequential damages (including, but not limited to, procurement of substitute goods or services; loss of use, data, or profits; or business interruption) however caused and on any theory of liability, whether in contract, strict liability, or tort (including negligence or otherwise) arising in any way out of the use of this software, even if advised of the possibility of such damage.

# 2. Getting started

## 2.1. Terminology and conventions used

This section details the terminology and conventions used throughout this guide. The details are as follows:

•WARNING:  A warning notice denotes a hazard that could result in personal injury.

•CAUTION:  A caution notice denotes a hazard that could damage or destroy the PLEXTALK product, destroy the recording media, destroy or affect your data or computer configuration.

•Note:  A note is used to give additional important information about a feature that has not already been mentioned.

•SD Card:  The PLEXTALK is able to use both "SD" and "SDHC" cards. In this User's Guide, both cards are called "SD cards".

## 2.2. What you get in the box

Below is a list of all the items you should find in the box with your PLEXTALK. Please make sure that all items are included. If there are any missing items, please contact your local agent.

•1 PLEXTALK Linio Pocket

•1 SD Card (Note:  The SD card will come already inserted in the unit.)

•1 AC adaptor

•1 Rechargeable battery

•1 Set of stereo headphones

•1 USB cable (long) for connecting to a computer

•1 USB cable (short) for connecting to a USB peripheral

•1 Pouch

•1 Pocket guide

Note:  It is strongly recommended that you retain all packaging, in case you need to return your PLEXTALK for any reason in the future.

## 2.3. Description of the unit

This section describes the physical look of the PLEXTALK and the layout of the keys and other controls to enable you to locate them when referred to in later chapters. The names and main purposes are also detailed here.

While carefully holding the unit, take a few moments to locate the front of the unit. The front has the microphone and speaker grill at one end and 23 keys arranged across the rest of the surface. Turn the unit so that you are holding it with the keys uppermost and with the grill away from you. The PLEXTALK should now be set up in its normal operating position. Keep the unit in this position as the physical layout is described.

### 2.3.1. Front face

The grill that you used to orientate the unit is where the internal speaker and the built in microphone are located. The speaker is on the right and the microphone is in the center.

Just below the grill, in the center, there are 2 LEDs. The one on the left is the Power LED. It should be off when the unit is turned off. It becomes orange while booting up. It shows green when the unit is on. The one on the right is the Battery LED and indicates the charging status. It is off when the PLEXTALK is operating on battery power. During charging Phase 1, it will show a red light. During charging Phase 2, it will alternate between red and green, and when the battery is fully charged, it turns solid green.

Below the grill, to the left and right of the LEDs, there are two oval buttons. The key on the left is colored red and is the Record key, used for DAISY recording, Voice memo recording and for recording Voice Bookmarks. The key on the right is colored green and is the Power key, used for power on/off and for setting the sleep timer. In the center of this key there are two prominent tactile dots.

Below the Record key and the Power key, there are 4 function keys: 2 on the left and 2 on the right of the unit. The upper left key is the GoTo key, used for page and heading navigation. The lower left key is the Bookmark key, used for bookmark navigation, insertion and deletion. The upper right key is the Menu key, used to access the main menu. Press and hold this key to turn on or off the Key Describer mode. The lower right key is the Title key, used for changing the title or media.

In between the 4 function keys, at the center of the unit, is the circular Play/Stop key surrounded by four arrow keys: Up, Down, Left and Right. The Play/Stop key is slightly indented and is colored blue. It is used to start/stop playback and acts as the Enter key for some operations. The Up key is used for moving up in the list of navigation options. It is also used for increasing the speed and tone settings or to cancel an operation. The Down key is used for moving down in the list of navigation options. It is also used for decreasing the speed and tone settings or to act as the Enter key for some operations. The Left key is used for moving left in menus or to move back in a title. Press and hold the key for fast rewind. The Right key is used for moving right in menus or to move forward in a title. Press and hold the key for fast forward.

Below the function and arrow keys are a further 12 small circular keys, arranged in 4 rows of 3 keys. These are generally called the Number Pad. The Number Pad is laid out exactly like a push button telephone keypad. The first row has numbers 1, 2 and 3. The second row has numbers 4, 5 and 6. The third row has numbers 7, 8 and 9 and the fourth row has the Star key, Number 0 key and the Pound/Hash key. The number 5 key has a small raised dot on it to help you locate it.

Some of the keys have secondary functions and these are listed below:

•Number 1 = Change speed or tone in combination with the Up or Down key

•Number 5 = Information key. Press once for a list of information. Press repeatedly to step quickly through the list of information. Press and hold to get the current date and time.

•Number 9 = Backup

•Number 0 = Help. Press once to enter or exit the built in help file.

•Star = Undo/Cancel

•Pound/Hash = Redo/Enter

Throughout this guide, the term "Enter key" refers to the Play/Stop key, the Down key or the Pound/Hash key. The term "Cancel key" refers to the Up key or the Star key.

### 2.3.2. Top edge

On the top edge of the PLEXTALK, there are 2 sockets - one towards the left side and one in the center. The socket on the left is for an external microphone and line in and the center socket is for headphones. All sockets take 3.5mm diameter stereo mini jack plugs.

In the top left corner, there is a small hole which is designed for fitting a carrying strap if required.

### 2.3.3. Left edge

On the left edge of the unit, roughly level with the Record key, there is a slot for the SD card.

### 2.3.4. Right edge

On the right edge of the unit, there are 2 switches. The upper switch is for increasing and decreasing the volume. Pushing the switch towards the top of the unit, increases the volume. Pulling the switch towards the bottom of the unit, decreases the volume.

The lower switch is the Key Lock key. This is to prevent accidental activation of any other key, including the Power key. Pushing the switch toward the top of the unit locks all keys. Pulling the switch toward the bottom of the unit releases the lock.

### 2.3.5. Bottom edge

There are 2 items on the bottom edge of the unit. On the left is a small round hole in a square-shaped housing. This is the DC input terminal. On the right is a slightly smaller rectangular slot. This is the USB socket.

### 2.3.6. Back face

Carefully turn over your PLEXTALK and hold it face down in one hand, keeping the end with the grill away from you.

In the center of the Back face, there are 2 raised ridges. These are at the top end of the battery cover. Towards the bottom edge of the unit, but still on the Back face, there is a rectangular button which is flush with the surface. This button is at the bottom edge of the battery cover and is used to release the cover.

To remove the battery cover, press down firmly on the rectangular button with the tip of a finger. At the same time, use the thumb of your other hand, on the 2 ridges, to slide the battery cover towards the bottom edge of the unit. It should release fairly easily but you may need to practice a couple of times to master the technique.

## 2.4. Inserting the battery

In order to insert or replace the battery, do the following:

•Turn off the power and remove the power adaptor.

•Remove the battery cover, as described in the previous section, and lay the PLEXTALK and battery cover to one side.

•Locate the new battery and orientate it so that the one edge, which is completely smooth, is facing away from you and the edge, which has 2 obvious projections on it, is facing to your right. These two projections are the battery terminals.

•Assuming that you have done this correctly, there should be 2 further projections, one either side of the bottom left corner. Note these as these are useful when trying to remove the battery later.

•Pick up the PLEXTALK, ensuring that the front face is downwards and the battery housing is towards you.

•Carefully insert the battery, keeping the two battery terminals on the leading edge. This should mean that you are introducing the battery from left to right across the battery housing.

•Push the battery terminals gently into place and then press down gently on the left side of the battery and it should slip easily into the housing. Do not use force to insert the battery as this should not be necessary.

•Place the battery cover over the battery housing, around 5 millimeters from its normal position. While pressing the battery cover towards the battery housing, gently slide the battery cover back up towards the top of the unit until the rectangular button pops up into the locked position.

WARNING:  You should only use the recommended battery with this unit. Failure to do so may cause an explosion or fire.

Note:  To remove an old battery, gently lift the bottom left corner of the battery using a fingernail under one of the 2 projections that are on the bottom left corner and then lift it carefully out of the housing. This takes a bit of practice but is simple once you understand the technique.

## 2.5. Charging PLEXTALK

In order to charge the internal battery, you need to connect the PLEXTALK to the AC adaptor or to a computer using the supplied USB cable. Do this as follows:

•Place the PLEXTALK face up in its normal operating position.

•To charge the battery using the AC adaptor, plug the supplied AC adaptor into the power input terminal on the bottom left edge of the unit. Then plug the power adaptor into the AC wall socket. Charging will start.

•To charge the battery using the USB connection, turn the power off.

•Then plug the smaller end of the long USB cable into the USB socket on the bottom right edge of the PLEXTALK. Then plug the wider end of the cable into a spare USB socket on a computer. Ensure that the computer is turned on and charging will start.

CAUTION:  Never use force when connecting the USB cable as you may damage the cable or connector in the process.

Note: The charging via a USB cable works during power off.

## 2.6. Maintenance and battery care

### 2.6.1. Unit care

Carefully wipe any dust and dirt from the PLEXTALK with a soft cloth that has been slightly dampened with a diluted, neutral detergent, then wipe dry with a cloth.

WARNING:  Always turn the power off and disconnect PLEXTALK from the AC adaptor before cleaning.

### 2.6.2. Battery care

In order that you get the most out of your rechargeable battery, you should take careful note of the following points:

•The battery will normally take 4 hours to fully charge. However, when using your PLEXTALK for the first time, or after one month of storage, you should connect to a mains power supply for at least 24 hours.

•If the PLEXTALK is not to be used for over a month, remove the battery and store it in a cool place with low humidity.

•The battery life depends on how frequently it is used and recharged. Its capacity to store charge reduces over time. When the usable time following a charge becomes too short for practical use, replace the battery with a new one. These can be purchased from your local agent. Your agent should also be able to give you advice about recycling and disposal of used batteries.

•The usable time may be affected by the ambient temperature and working environment.

In order to maintain battery performance, the following procedure should be carried out every six months:

•Fully charge the battery.

•Use on battery power until dead.

•Recharge.

### 2.6.3. Crossed-out dustbin mark

Symbol mark called "crossed-out dustbin mark" is printed on the surface of the battery and on the inside of the battery housing.

Note:  This symbol mark is valid only for EU countries.

This symbol mark is according to the directive 2006/66/EC. This symbol means that batteries and accumulators, at their end-of-life, should be disposed of separately from regular municipal waste. If a chemical symbol is printed beneath the symbol shown above, this chemical symbol means that the battery or accumulator contains a heavy metal at a certain concentration listed below.

Hg: mercury (0.0005%)

Cd: cadmium (0.002%)

Pb: lead (0.004%)

These substances may cause serious hazards to human and the global environment if not cared in an appropriate manner. In the European Union there are separate collection systems for used batteries and accumulators. Please dispose of batteries and accumulators properly at your local community waste collection or recycling center. Our product is designed and manufactured with battery and accumulator components which can be recycled or reused.

## 2.7. Turning PLEXTALK on and off

To turn the unit on or off, do the following:

•Press and hold down the Power key to turn the unit on. When the power is on, the left LED will turn orange and then green.

•Press and hold the Power key to turn on power saving mode. Release after the "Power save" announcement. Keep pressing and holding another few seconds until the "Power off" announcement in order to turn the power off.

Note: When PLEXTALK is in the power saving mode, PLEXTALK resumes by pressing any key. Then time will be announced.

Note: When PLEXTALK is in the power saving mode, battery drainage is reduced.

Note: The booting up process should take approximately 40 seconds after turning the power off or after the battery has been replaced.

Note:  When you turn off the power, if you do not hold the key down long enough, the Sleep Timer will be activated.

Note:  If Key Lock switch is on, the Power key will not function.

Caution: If you turn off the power, the PLEXTALK does not notice the events of calendar and the alarms.

## 2.8. Key Describer function

As its name implies, the Key Describer function allows the user to get a description of any key on the PLEXTALK.

•Turn on the Key Describer by pressing and holding down the Menu key for around 5 seconds. Now press any key on the unit and a description of its function will be spoken.

•Turn off the Key Describer function by pressing and holding down the Menu key for around 5 seconds.

## 2.9. Key Lock function

Key Lock can be used to lock the keys and controls so that they can't be accidentally operated.

•To apply the lock, push the Key Lock switch towards the top of the unit.

•To unlock the PLEXTALK, pull the Key Lock switch towards the bottom of the unit.

## 2.10. Inserting and ejecting SD card

To insert the SD card, do the following:

•Turn off the power.

•Hold the SD card so that the cut off corner may be located at the front right corner. In other words, the cut off corner should be nearest to the bottom of the unit.

•Insert the SD card into the card slot on the left edge of the unit.

•Press the card all the way into the slot ensuring that it engages properly, with the outer edge flush with the body of the PLEXTALK. Do not use excessive force when doing so.

To eject the SD card, do the following:

•Turn off the power.

•Push the SD card inwards and then release it. The card should pop out leaving the outside edge of the card protruding.

•Carefully grip the card and pull it out of the slot.

CAUTION:  The PLEXTALK's SD card slot is only compatible with SD or SDHC cards. Inserting other memory card such as a Memory Stick or Multi Media Card, will cause a malfunction and should not be used.

Note:  SD cards have a locking switch which enables you to protect your data if you want. This is located on the left side of the card away from the cut off corner. If the switch is towards the leading edge of the card, you will be able to record and delete titles. If the switch is away from the leading edge, your data will be protected.

# 3. Introduction to the menu system

A number of the features of the PLEXTALK are accessed through the built in menu system. This chapter aims to give you an understanding of how the menu is structured and how you access it.

## 3.1. Main menu

At the highest level there is the Main Menu. These are:

•Online service: the menu services provided by the online service provider.

•Edit: allows simple editing of a title.

•Recording Settings: allows you to select from a variety of recording settings or to set your own customized settings.

•Playback setting: allows you to repeat setting and to change the method of reading name of album and track.

•Bookmark: allows you to specify how Voice Bookmarks play and allows you to delete all bookmarks.

•Voice Memo: allows you to delete Voice Memos.

•Podcast: allows you to update the episode list and download episodes.

•Calendar: allows you to delete events of Calendar.

•Alarm: allows you to set or clear the alarm setting.

•Media Management: allows you to get media information, build recorded DAISY titles, erase SD card data and so on.

•System settings: allows you to change settings for date and time, synthetic voice, network and so on.

•Bookshelf and Announcement: allows you to update Bookshelf and Announcement.

## 3.2. Accessing and navigating the menu

To navigate around the menu, use the following methods:

•Access the menu by pressing the Menu key once.

•To select a different option on the same level of the menu, press either the Left key to move to the left or the Right key to move to the right. As you move to a new menu option, it will be spoken.

•On all menus, if you continue in the same direction, you will eventually return to the starting point as the menus are designed to loop round.

•To select a menu option, press the Down key or the Pound/Hash key. Assuming that there is a sub menu, and it is accessible at that time. This will take you down one level in the menu structure and leave you on the relevant sub menu.

•To move up one level in the menu, press the Up key.

•To exit immediately from the menu system, from any level in the structure, press the Menu key.

## 3.3. Shortcut for the menu selection

You can register and select a specific menu item with Number Pad.

To register the specific menu item, do the following:

•Press the Menu key and go to the menu item you want to register.

•While pressing and holding the Menu key, press and hold your favorite Number Pad until confirmation sounds. Then the PLEXTALK will register the specific menu item with this number.

To select the specific menu item directly, do the following:

•While pressing and holding the Menu key, press and release the registered Number Pad. Then you can select the specific menu item.

# 4. Setting up your PLEXTALK

## 4.1. Setting the date and time

To set the date and time for the internal clock, do the following:

•Press the Menu key once to access the Main Menu.

•Use the Left or Right key to go to the "System settings" option and select it by pressing the Enter key.

•Select the "Set the system date and time" option from the menu and press Enter.

•Use the Number Pad to enter the current year (with four digits), month and day. Then press the Enter key.  Or just press Enter to accept the current setting.

•Use the Number Pad to enter the current time with four digits and press Enter. Use leading zeros where necessary.

•By way of confirmation, the date and time that you have just entered will be spoken back to you.

Note:  When entering an incorrect number, press the Cancel key to clear the entered number.

Note:  To check the current date and time, press and hold down the Number 5 key.

## 4.2. Selecting your preferred synthetic voice

You are given the choice of two synthetic voices for the text to speech voice. To select your preferred text to speech voice, do the following:

•Press the Menu key once to access the Main Menu.

•Use the Left or Right key to go to the "System settings" option and select it by pressing the Enter key.

•Use the Left or Right key to go to the "Select the text to speech voice" option and select it by pressing the Enter key.

•Use the Left or Right key to select your preferred voice and confirm it by pressing the Enter key. PLEXTALK will make your selection the new default setting and will exit the menu system.

## 4.3. Setting the text to speech pitch

To adjust the pitch of the built in text to speech voice, do the following:

•Press the Menu key once to access the Main Menu.

•Use the Left or Right key to go to the "System settings" option and select it by pressing the Enter key.

•Use the Left or Right key to go to the "Set the text to speech pitch" option and select it by pressing the Enter key.

•Use the Left or Right key to choose your preferred pitch setting, in the range from -5 to +5, with 0 being the normal setting, and select it by pressing the Enter key.

## 4.4. Selecting a preferred waiting sound

When the PLEXTALK is busy, such as when it is accessing a media, it plays a sound to indicate that it is carrying out that action. To select your preferred waiting sound, do the following:

•Press the Menu key once to access the Main Menu.

•Use the Left or Right key to go to the "System settings" option and select it by pressing the Enter key.

•Use the Left or Right key to go to the "Select the waiting sound" option and select it by pressing the Enter key.

•Use the Left or Right key to listen to the available options and press the Enter key when you are on the waiting sound you prefer.

## 4.5. Selecting the Key Press Confirmation Sound

The PLEXTALK gives you two choices of Key Press Confirmation Sound, on or off. To select the Key Press Confirmation Sound, do the following:

•Press the Menu key once to access the Main Menu.

•Use the Left or Right key to go to the "System settings" option and select it by pressing the Enter key.

•Use the Left or Right key to go to the "Select the Key Press Confirmation Sound" option and select it by pressing the Enter key.

•Use the Left or Right key to select either "Key Press Confirmation Sound On" or "Key Press Confirmation Sound Off", and select it by pressing the Enter key. The PLEXTALK will make your selection the new default setting and will restart.

# 5. Power management features

## 5.1. Battery low warning

When the PLEXTALK is operating on battery power and the power level drops to a level where it needs to be recharged, the PLEXTALK will say "Low battery." Then player will shut down in one minute. If you wish to continue to use the PLEXTALK, you need to plug in the AC adaptor.

## 5.2. Automatic power off function

In order to save battery power, the PLEXTALK will automatically turn on the power saving mode after 2 minutes of inactivity even if the battery level is still OK.

When the PLEXTALK is connected to the AC adaptor or USB device, the PLEXTALK automatically turns on power saving mode after 15 minutes of inactivity.

## 5.3. Compulsory power off function

If for any reason, the PLEXTALK will not shut down or you are not able to navigate with any keys, you can try using the compulsory power off function, as an alternative to removing the battery. To use the compulsory power off feature, do the following;

•Turn the Key Lock on.

•Press and hold down the Power key for more than 10 seconds, then release the Power key. The PLEXTALK should shut down.

•Turn the Key Lock off.

Note:  If this method does not work, and your PLEXTALK is still powered on, you should unplug the AC adaptor, remove the battery and insert the battery again.

## 5.4. Sleep timer function

The Sleep Timer feature enables you to specify an amount of time, after which, the PLEXTALK will shut down.

### 5.4.1. Setting the sleep timer

To set the sleep timer, do the following:

•Quickly press and release the Power key. The PLEXTALK will say, "Sleep timer. 15 minutes. Set."

•Press and release the Power key again to increase the time for shutting down. The Sleep Timer setting will increase as 30 minutes, 45 minutes, 1 hour, 1 hour 30 minutes, and 2 hours for each press.

### 5.4.2. Cancelling the sleep timer

To cancel the sleep timer, do the following:

•Quickly press and release the Power key. The PLEXTALK will tell you that the sleep timer is currently on and the number of minutes remaining before shut down.

•Keep pressing the Power key repeatedly until the PLEXTALK says "Off".

# 6. Playing features common to all formats

The PLEXTALK is able to play following formats.

•DAISY book

•Music:  MP3, Wave, WMA, AMR-WB+ and Ogg Vorbis.

•Text files: plain text, Microsoft Word .doc, .docx, HTML, EPUB2 and PDF.

•Audio books: Audible, MP3, Wave, WMA, AMR-WB+, Ogg Vorbis and LKF.

•Podcast

•Web radio

Note:  If a CD has multiple kinds of content, for example, DAISY files and Audio files, the PLEXTALK will recognize one kind of content only.

Note:  Word files and WMA files on external USB CD/DVD media can not be played.

## 6.1. Start and stop playback

To start or stop playback, do the following:

•To start playback, press the Play/Stop key. The PLEXTALK will start playback from the position where it was stopped previously.

•To stop playback, press the Play/Stop key.

## 6.2. Fast forward or fast rewind

The fast rewind and fast forward features use the Left and Right keys either side of the Play/Stop key.

•To fast rewind, press and hold down the Left key. Playback will start when you release the key.

•To fast forward, press and hold down the Right key. Playback will start when you release the key.

When you play a DAISY book with audio or Audio file, as you hold down the Left and Right key, the fast rewind or fast forward will skip 5 seconds, 10 seconds, 20 seconds, 30 seconds, 1 minute, 2 minutes, 3 minutes and so on. If the key is released before the "2 minutes" option is spoken, playback will start at the place the key is released. If the key is released after this point, the fast rewind or fast forward will continue until the Play/Stop key is pressed to stop the process.

Note: There are no functions of fast-forward and fast-rewind for playing web radio.

## 6.3. Changing the playback volume

The volume can be set on a scale from 0 to 25, in steps of 1, with 0 being silent and 25 being the Highest possible setting. The volume of the guide speech is also adjusted in conjunction with the playback volume.

•To increase the playback volume, either push the Volume switch up repeatedly, increasing the level by 1 setting each time, or push and hold the Volume switch up until the volume reaches the desired level.

•To decrease the playback volume, either pull the Volume switch down repeatedly, decreasing the level by 1 setting each time, or pull and hold the Volume switch down until the volume reaches the desired level.

Note: You can set the headphone volume and speaker volume independently.

## 6.4. Changing the playback speed

There are 12 speed settings ranging from minus 3 to plus 8, which can be adjusted 1 level at a time. The normal speed is at the 0 position. Minus 3 is half of normal speed, plus 4 is double speed and plus 8 is triple speed.

•To increase the playback speed, press the Number 1 key once and then press the Up key.

•To decrease the playback speed, press the Number 1 key once and then press the Down key.

Note:  Pressing and holding the Up or Down key, will cause the speed setting to change rapidly in the selected direction.

Note: Playback speed of web radio cannot be changed.

## 6.5. Changing the audio tone

There are 13 tone settings ranging from minus 6 to plus 6, which can be adjusted 1 level at a time.

•To increase the tone, press the Number 1 key twice and then press the Up key.

•To decrease the tone, press the Number 1 key twice and then press the Down key.

Note:  Pressing and holding the Up or Down key, will cause the tone setting to change rapidly in the relevant direction.

## 6.6. Changing the guide volume

The volume of the guide speech can be changed. There are 11 volume settings ranging from minus 5 to plus 5, which can be adjusted 1 level at a time. To set the guide volume, do the following:

•Press the Number 1 key three times.

•Press the Up key to increase the guide volume or press the Down key to decrease the guide volume.

Note:  Pressing and holding the Up or Down key, will cause the guide volume setting to change rapidly in the relevant direction.

Note: When the playback volume is changed, the volume of the guide speech is also changed proportionally corresponding to the playback volume.

## 6.7. Changing the guide speed

The speed of the guide speech can be changed independently from the playback speed. There are 12 speed settings ranging from minus 3 to plus 8, which can be adjusted 1 level at a time. To set the guide speed independently, do the following:

•Press the Number 1 key four times.

•Press the Up key to increase the guide speed or press the Down key to decrease the guide speed.

Note:  Pressing and holding the Up or Down key, will cause the guide speed setting to change rapidly in the relevant direction.

## 6.8. Accessing the Navigation menu

One of the most useful features of the PLEXTALK is the ability to choose how you want to navigate. To use the Navigation Menu, do the following:

•Use the Up or Down keys to move upwards or downwards in the navigation menu. The items you hear spoken are the options available to you from the Navigation Menu at that time.

•Having selected your preferred navigation method, you can now use your Left or Right key to move backwards or forwards at your chosen level of navigation.

•For example, if you go to the "Level 1" option with your Up or Down key, and then use your Left or Right key, you will be able to move backwards or forwards to the next Level 1 heading.

## 6.9. Selecting the media and title

At times you may wish to switch from using an SD card to the USB media or vice versa. You can do this as follows:

•Press the Title key repeatedly until your desired media is announced. PLEXTALK will move sequentially through the available options: "SD card", "USB", "Internal Memory", "Voice Memo", "Bookshelf", "Service Announcement" and "Calendar".

•After your desired media is announced, use the Left or Right key to move through the available contents on that media.

•When you are on the item you wish to play, press the Play/Stop key and playback will start at the last playing position in that item.

Note: PLEXTALK does not announce non-existent media.

## 6.10. Title navigation

In situations where there are multiple items on a media, you may wish to switch from the item you are currently reading to different one.

### 6.10.1. How to navigate the title

You can select your favorite title by using "category", "folder", "title" and "file" levels. In the "category" level, there are "DAISY title", "Recordings", "Music", "Text", "Audio book", "Podcast" and "Web radio". To search the title by title navigation, do the following:

•Press the Title key repeatedly until you hear the media you want to go to.

•Press the Up or Down key to select the level of title navigation from "category", "folder", "title" or "file".

•Press the Left or Right key to move sequentially through the selected level.

•When selecting the "folder" or "category" level, if this folder or category contains the title or the album you wish to play, press the Down key once or twice to switch to the "title" level. The PLEXTALK will say "Select title". Use the Left or Right key to search the title you wish to play.

•When selecting the "title" or "file" level, you are able to select a favorite title or file.

•If you wish to play the selected title, press the Play/Stop key. Playback will start from the position where it was stopped previously.

Note: Navigation by file is available in "Music", "Audio book" and "Podcast", but not available in other categories.

Note: Audio book from Audible cannot be available file navigation.

### 6.10.2. How to play music

This section shows how to seek for your favorite music.

< Playing all music >

There are two ways to play all music. Use the Up or Down key to select "category" level, then use the Left or Right key to select "Music" category and press the Enter key. Or, use the Up or Down key to select "title" or "folder" level, then use the Left or Right key to select "All music" and press the Enter key.

< Playing a single album >

Select your favorite album at "title" level and then press the Enter key. Only this album will be played.

< Playing a single track >

Select your favorite track at "file" level and then press the Enter key. Only this track will be played.

< Playing albums in a folder >

Select your favorite folder at "folder" level and then press the Enter key. After pressing the Enter key, all albums in this folder become available to play.

< "Repeat" or "Shuffle" playback >

You can use "Repeat" or "Shuffle" playback in the folder or album which you selected.

## 6.11. Go to a title directly

To go to a title directly, do the following:

•Press the Title key repeatedly until you hear the media you want to go to.

•Press the Up or Down key to select the "category" level.

•Use the Left or Right key to select favourite category.

•Type in the number of the title you wish to go to, and press the Play/Stop key or Pound/Hash key.

•Press the Enter key again to play the title.

## 6.12. Switching sorting order of titles

When selecting a title, you can change the sorting order of the titles. There are three types of the sorting order that are "All titles list", "Recent titles list" and "New titles list".

•All titles list: All files and folders are sorted by their names.

•Recent titles list: The titles are sorted by date played.

•New titles list: The titles are sorted by time stamp of the file.

To play the recently played title, do the following:

•Press the Title key once.

•Press the GoTo key repeatedly to select the "Recent titles list".

•Use the Up or Down key to select either "category" or "title" and use the Left or Right key to select the title you prefer. And then press the Enter key to play.

•Or using the Number Pad, enter the number of the title you prefer and press the Enter key to select. Then press the Enter key once more to start playback.

Note: PLEXTALK memorizes up to 10 titles in each Category.

## 6.13. Getting information about media and title

To get a variety of information about the current media and title, do the following:

•Press the Menu key once to access the Main Menu.

•Use the Left or Right key to go to the "Media Management" option and select it by pressing the Enter key.

•Use the Left or Right key to go to the "Media and title information" option and select it by pressing the Enter key.

•PLEXTALK will announce the information on the current media and title.

•When you have noted the relevant information, press the Enter key to exit the menu system.

## 6.14. Getting information on the current status

You can get a range of information on the current status, about heading, page, bookmark and so on. Whether PLEXTALK is playing or not, do the following:

•Press the Number 5 key once.

•A range of information will be spoken.

•If you do not wish to listen to all the information, and wish to step through individual items more quickly, you can do this by repeatedly pressing the Number 5 key.

•If you have heard what you want to hear, you can stop the spoken information by pressing the Enter key.

•Press and hold down the Number 5 key to hear the current date and time.

## 6.15. Selecting a playback mode

To select the shuffle and repeat settings from playback modes, do the following:

•Press the Menu key once to access the Main Menu.

•Use the Left or Right key to go to the "Playback Settings" option and select it by pressing the Enter key.

•Use the Left or Right key to go to the "Shuffle and repeat settings" option and select it by pressing the Enter key.

•Use the Left or Right key to go to your preferred playing mode and select it by pressing the Enter key.

## 6.16. Using bookmarks

### 6.16.1. Types of bookmark

There are 3 types of bookmark you can use in PLEXTALK as follows:

•Numbered Bookmarks: You can insert up to 10000 Numbered Bookmarks. Each bookmark is assigned a unique number in the range 1 to 65000.

•Voice Bookmarks: These are Numbered Bookmarks that have accompanying voice annotations. Voice Bookmarks use memory. The total memory available gives about 30 minutes of recording time. If you try to exceed this allocation, the oldest bookmark that has not been used recently is automatically deleted before the new one is created.

•Resume Bookmarks: For every title, album or text file, the current playing location will be stored so playback can be resumed from that location the next time the title, album or text file is used. Resume Bookmarks are inserted automatically by PLEXTALK when you move to a different title, album or file, or when you shut down the player. No action is required from you.

Note:  PLEXTALK will remember the Resume Bookmark for up to 1000 titles, albums or text files.

### 6.16.2. Inserting bookmarks

To insert a Numbered Bookmark, whether PLEXTALK is playing or not, do the following:

•Press the Bookmark key twice.

•Using the Number Pad, enter the number you wish to assign to this bookmark. If you allow PLEXTALK to assign a number, do not enter the number or enter the number "0".

•Press the Enter key to confirm or the Cancel key to clear your entry.

Note:  When you allow PLEXTALK to assign the number, PLEXTALK selects the smallest number from the unused numbers.

To insert a Voice Bookmark, whether PLEXTALK is playing or not, do the following:

•Press the Bookmark key twice.

•Using the Number Pad, enter the number you wish to assign to this Voice Bookmark. If you allow PLEXTALK to assign a number, do not enter the number or enter the number "0".

•Press the Record key once to confirm or the Cancel key to clear your entry.

•Press and hold down the Record key and speak your voice annotation.

•Release the Record key when you have finished. PLEXTALK will confirm that the Voice Bookmark has been set.

### 6.16.3. Navigating to bookmarks

To jump to a Numbered Bookmark or a Voice Bookmark, whether PLEXTALK is playing or not, do the following:

•Press the Bookmark key once.

•Using the Number Pad, enter the number of the bookmark you wish to go to.

•Press the Enter key to confirm or the Cancel key to clear your entry.

•When a valid bookmark number has been entered, the PLEXTALK will go to the location of the bookmark and playback will start automatically.

Once you have entered bookmarks into a title, album or file, a "Bookmark" option will become available on the Navigation Menu. In order to navigate to the next bookmark with a lower or higher number, do the following:

•Use the Up or Down key to go to the "Bookmark" option on the Navigation Menu.

•Press the Left key to move to the next bookmark with a lower number. Playback will start automatically.

•Press the Right key to move to the next bookmark with a higher number. Playback will start automatically.

### 6.16.4. Clearing bookmarks

To clear a Numbered Bookmark or a Voice Bookmark, the procedure is identical, as follows:

•Press the Bookmark key 3 times.

•Using the Number Pad, enter the number of the bookmark you wish to remove.

•Press the Enter key to confirm or the Cancel key to clear your entry.

•When you confirm the action, PLEXTALK will remove the specified bookmark from the current title or album and will confirm this with a spoken message.

If you wish to clear all bookmarks in the current DAISY title or Text file, or clear all bookmarks in all albums, do the following:

•Press the Menu key to access the Main Menu.

•Use the Left or Right key to go to the "Bookmark" option and select it by pressing the Enter key.

•Use the Left or Right key to go to the "Clear all bookmarks in current title" option and select it by pressing the Enter key.

•PLEXTALK will ask you to confirm this action. If you wish to proceed, press the Enter key and PLEXTALK will say "Deleted".

Note:  The procedure will only clear bookmarks in the current DAISY title or Text file and will not affect bookmarks in other titles or files.

### 6.16.5. Turning on and off Voice Bookmark speech

When you are using Voice Bookmarks, you are able to select whether you want to hear them spoken or not during playback. To turn Voice Bookmarks On or Off, do the following:

•Press the Menu key to access the Main Menu.

•Use the Left or Right key to go to the "Bookmark" option and select it by pressing the Enter key.

•Use the Left or Right key to go to the "Playback setting for Voice Bookmark" option and select it by pressing the Enter key.

•Use the Left or Right key to select either "Voice Bookmark on" or "Voice Bookmark off" and press the Enter key to confirm your choice.

## 6.17. Understanding how PLEXTALK groups files and folders

When PLEXTALK accesses the SD card or USB media, it automatically assesses the contents and groups items into "DAISY title", "Recordings", "Music", "Text", "Audio book", "Podcast" and "Web Radio", regardless of where these items are stored on the media. When you subsequently navigate through the titles, they will be already ordered for you regardless of where they are physically located on the media.

For example, if you have 3 Text Files in the root folder and 2 more in a sub folder, PLEXTALK will view these as 5 Text Files stored in 2 folders.

If there are multiple types of contents such as DAISY titles, Audio Files or Text Files, PLEXTALK will automatically sort them by their type and name.

# 7. Playing DAISY titles

This chapter covers additional features that can be used when you are playing a DAISY title.

## 7.1. Heading navigation

There are two ways of navigating to headings within a DAISY title. You can either go to headings direct or you can move through the available headings one at a time.

To go direct to a specific heading, do the following:

•Press the GoTo key twice.

•Using the Number Pad, type in the number of the heading you wish to go to.

•Press the Enter key to confirm or the Cancel key to clear your entry.

To navigate headings sequentially, using heading levels, do the following:

•Use the Up or Down key to go to your desired level of navigation, for example Level 1, Level 2 etc.

•Press the Left key to move backwards to the previous heading on the selected level.

•Press the Right key to move forwards to the next heading on the selected level.

Note:  When navigating in this way, PLEXTALK will stop at all headings that have a level the same as or higher than the level you have selected. For example, if you chose to navigate on Level 3, PLEXTALK will find all Level 1, Level 2 and Level 3 headings as you move backwards or forwards.

## 7.2. Moving to the beginning or end of the title

To move to the beginning of the title, do the following:

•Press the GoTo key twice.

•Press the Enter key and the playing position will move to the start of the current title.

To move to the end of the title, do the following:

•Press the GoTo key twice.

•Press the Number 0 key followed by the Enter key and the playing position will move to the end of the current title.

## 7.3. Page navigation

Navigating to pages is very similar to heading navigation.

To go direct to a specified page, do the following:

•Press the GoTo key once.

•Using the Number Pad, type in the number of the page you wish to go to.

•Press the Enter key to confirm or the Cancel key to clear your entry.

To navigate pages sequentially, do the following:

•Use the Up or Down key to go to the "Page" option on the Navigation Menu.

•Press the Left key to move backwards to the previous page.

•Press the Right key to move forwards to the next page.

Note:  If there are no page markers in the title, the "Page" option will not be listed on the Navigation Menu.

## 7.4. Group navigation

To move backwards or forwards a group at a time, do the following:

•Use the Up or Down key to go to the "Group" option on the Navigation Menu.

•Use the Left key to move backwards to the previous group item in the title.

•Use the Right key to move forwards to the next group item in the title.

Note:  If there are no group markers in the title, the "Group" option will not be listed on the Navigation Menu.

## 7.5. Phrase navigation

To move backwards or forwards one phrase, do the following:

•To jump back one phrase, press the Up or Down key to select the "Phrase" option on the Navigation Menu and then press the Left key.

•To jump forwards one phrase, press the Up or Down key to select the "Phrase" option on the Navigation Menu and then press the Right key.

## 7.6. 10 minutes skip, 30 seconds skip

To skip by 10 minutes or 30 seconds at the Audio DAISY formats, do the following:

•Use the Up or Down key to select 10 minutes or 30 seconds.

•To skip backward 10 minutes or 30 seconds, press the Left key.

•To skip forward 10 minutes or 30 seconds, press the Right key.

## 7.7. Navigating by screen, paragraph, line, sentence, spell, word or character in case of DAISY text mode

In addition to DAISY navigation options, DAISY text file will always give you the navigation options "Screen", "Paragraph", "Line", "Sentence", "Spell", "Word" and "Character". These options will automatically appear on the Navigation Menu when you access this type of file. To navigate by any of these options, the process is identical as follows:

•Use the Up or Down keys to select your preferred navigation method that is screen, paragraph, line, sentence, spell, word or character.

•Use the Left key to jump back in the file by the selected amount.

•Use the Right key to jump forward in the file by the selected amount.

Note:  "Screen" means 40 lines.

Note:  The spell navigation announces each word then spells the word.

## 7.8. Navigating by percentage

You can navigate by a percentage of the file. To navigate by percentage, do the following:

•Press the GoTo key three times.

•Type in a percentage between 0 percent and 100 percent, and press the Enter key. PLEXTALK will move the playback position to that position.

## 7.9. Navigating by keyword

You can move to a position using a keyword when playing a text DAISY file. To search by keyword, do the following:

•Press the GoTo key four times.

•Using the Number Pad, enter the keyword you wish to search. Refer to 17 "Entering text" to learn  the methods of entering characters.

•Press the Play/Stop key to listen to the keyword you entered.

•After you finish entering the keyword, press the Down key.

•PLEXTALK will search for the keyword from the current position to the end of the file. If the keyword is found, PLEXTALK will play the line that contains the keyword.

•If you press the Down key without entering the keyword,  PLEXTALK will regard the current word as keyword, and search for this word.

•The keyword will automatically be registered on the Navigation Menu.

To navigate the same keyword sequentially, do the following:

•Press the Up or Down key several times until the PLEXTALK announces the keyword.

•Press the Left key to search the keyword toward the beginning of the file.

•Press the Right key to search the keyword toward the end of the file.

Note: If you press the Play/Stop key when playing the line containing the keyword, the PLEXTALK starts to play from the position of the keyword.

Note: If the keyword does not exist, PLEXTALK announces that the keyword was not found and the playback position will remain the current position.

Note: When the PLEXTALK searches the keyword backwards, the searching process will be finished at the beginning of the file. When the PLEXTALK searches forwards, the searching process will be finished at the end of the file.

Note: To cancel searching for the keyword, press the Up key or Star key. The playback position will remain at the current position.

Note: You can register up to three keywords. When three keywords have been registered, adding one more keyword will remove the oldest keyword.

Note: You can use numeric characters or symbols as the keyword.

Note: The PLEXTALK does not distinguish between lowercase and uppercase characters when searching for the keyword.

Note: If the playback mode of multimedia DAISY title is set to "Text", you are able to use the searching process.

## 7.10. Playback mode

You can select from two playback modes when using a DAISY title. The available options are as follows:

•Normal playback: it plays through the title and stops when finished.

•Repeat title: the current title will repeat continuously until playback is stopped.

## 7.11. Multimedia DAISY playback setting

Multimedia DAISY is DAISY format that is contained both audio and text files. You can select which format is played if you play the Multimedia DAISY.

•Select Multimedia DAISY title.

•Press the menu key.

•Use the Left or Right key to go to "Playback settings" option and press the Enter key.

•Use the Left or Right key to go to "Multimedia DAISY playback setting" option and press the Enter key.

•Use the Left or Right key to select "Auto", "Audio" or "Text" and press the Enter key.

Note: You can change this optoin only when playing multimedia DAISY.

# 8. Playing "Recordings"

DAISY titles placed in the "PlexRecording" folder are placed into the category called "Recordings".

A title recorded with this PLEXTALK is automatically stored into the "PlexRecording" folder.

Playback method of "Recordings" is the same as a DAISY title.

Note: If you back up a "Recordings" title from SD card to USB device, it will be stored in the "PlexBackup" folder of the USB device. Therefore the backed up title is not categorized into "Recordings" but "DAISY title".

# 9. Playing Audio Files or Music CDs

This chapter covers features that are specific to playing Audio Files or Music CDs.

PLEXTALK is able to play following types of audio file:

•MP3 Files: MPEG1 Layer 3 from 32kbps to 320kbps and MPEG2 Layer 3 from 16kbps to 160kbps.

•WAVE Files.

•WMA Files: PLEXTALK does not support DRM protected WMA files.

•AMR-WB+ Files.

•Ogg Vorbis Files: from 45 kbps to 320 kbps

CAUTION:  PLEXTALK may not be able to play a Music CD that is not compliant with the CD digital audio (CD-DA) standard, such as copy-protected Music CDs.

Note: PLEXTALK cannot be used to record or edit Audio Files on either SD cards or USB media.

Note: WMA file on external USB CD/DVD media are unable to play.

## 9.1. Track navigation

You can either go to tracks direct or you can move through the available tracks one at a time.

To go direct to a specified track, do the following:

•Press the GoTo key once.

•Using the Number Pad, type in the number of the track you wish to go to.

•Press the Enter key to confirm or the Cancel key to clear your entry.

To navigate tracks sequentially, do the following:

•Use the Up or Down key to go to the "Track" option on the Navigation Menu.

•Press the Left key to move backwards to the previous track.

•Press the Right key to move forwards to the next track.

Note:  When moving back a track at a time, if you allow the track to play for more than 3 seconds, the next time you move back a track, you will move to the start of the current track. If you move back a second time within 3 seconds, you will move to the previous track.

## 9.2. Album navigation

You can either go to albums direct or you can move through the available albums one at a time.

To go direct to a specified album, do the following:

•Press the GoTo key twice.

•Using the Number Pad, type in the number of the album you wish to go to.

•Press the Enter key to confirm or the Cancel key to clear your entry.

To navigate albums sequentially, do the following:

•Press the Up or Down key and select "Album" from the Navigation Menu.

•Press the Left key to move backwards to the previous album.

•Press the Right key to move forwards to the next album.

## 9.3. 10 minutes skip, 30 seconds skip

To skip by 10 minutes or 30 seconds, do the following:

•Use the Up or Down key to select 10 minutes or 30 seconds.

•To skip backward 10 minutes or 30 seconds, press the Left key.

•To skip forward 10 minutes or 30 seconds, press the Right key.

## 9.4. Playback mode

When playing Audio Files, you can select a number of different playback modes. The available options are as follows:

•Normal playback: It plays through the current album and stops when finished.

•Repeat track: The current track will repeat continuously until playback is stopped.

•Repeat album: The current album will repeat continuously until playback is stopped.

•Repeat entire albums: Repeat all of the albums in the folder which you selected.

•Shuffle repeat: Shuffle and repeat all of the music in the folder or album which you selected.

## 9.5. Announcing albums and tracks by name or number

When navigating albums or tracks, PLEXTALK announces albums and tracks by name or number. Either setting to read the name or the number can be selected. For this setting, do the following:

•Press the Menu key once to access the Main Menu.

•Use the Left or Right key to go to the "Playback Settings" option and select it by pressing the Enter key.

•Use the Left or Right key to go to the "Reading audio file name" option and select it by pressing the Enter key.

•Use the Left or Right key to select between "Read audio file name" and "Read audio track number", and select it by pressing the Enter key.

## 9.6. About playing sequences

This section gives you information about the playing sequence used for playing files and folders.

•Playback of files: PLEXTALK will playback the files in the order of their file names.

•Playback of folders: If more than one folder exists on the SD card or USB media, the audio files will be played in the order of the folder names.

## 9.7. Restrictions on Audio Files

Restrictions on numbers of files and folders:

•The maximum number of files that can be stored in a folder is 1024. Files in excess of 1024 are not recognized.

•The maximum number of files that can be stored in a media is 6000. Files in excess of 6000 are not recognized.

•The maximum number of folders is 1024, including a root folder. Folders in excess of 1024 are not recognized.

•PLEXTALK treats file folders as albums and recognizes up to 8 hierarchies including root.

•If a folder contains a DAISY book, PLEXTALK does not search the other contents under the folder.

Note:  When there are a large number of folders, PLEXTALK may take longer to read the SD card or USB information.

# 10. Playing Text Files

PLEXTALK is able to play the Text Files by using synthetic voices. PLEXTALK supports following types of text files:

•Plain text

•Microsoft Word .doc, .docx

•HTML

•EPUB2

•PDF

Note: Word file on external USB CD/DVD media are unable to play.

## 10.1. Navigating by screen, paragraph, line, sentence, spell, word or character

A Text File will always give you the navigation options "Screen", "Paragraph", "Line", "Sentence", "Spell", "Word" and "Character". These options will automatically appear on the Navigation Menu when you access this type of file. If the files that  is Word, HTML, EPUB2 and PDF had edited with headings, you will be given an additional navigation option on the Navigation Menu. This will be called "Level". The navigation options of PDF files are "Level"(if existance), "page", "line", "sentence", "spell", "word" and "character".

To navigate by any of these options, the process is identical as follows:

•Use the Up or Down key to select your preferred navigation method that is screen, paragraph, line, sentence, spell, word or character.

•Use the Left key to jump back in the file by the selected amount.

•Use the Right key to jump forward in the file by the selected amount.

Note:  "Screen" means 40 lines.

Note:  The spell navigation announces each word then spells the word.

Note: You cannot select the heading navigation which did not edited with headings in a document.

## 10.2. Navigating by percentage

In Text Files, you can navigate by a percentage of the file. To navigate by percentage, do the following:

•Press the GoTo key repeatedly to select Percentage navigation.

•Type in a percentage between 0 percent and 100 percent, and press the Enter key.

•PLEXTALK will move the playback position to that position in the Text File.

## 10.3. Navigating by keyword

You can move to a position using a keyword prefer when playing a text file. To search by keyword, do the following:

•Press the GoTo key repeatedly to select Keyword navigation.

•Using the Number Pad, enter the keyword you wish to search. Refer to 17 "Entering text" to learn the methods of entering characters.

•Press the Play/Stop key to listen to the keyword you entered.

•After you finish entering keyword, press the Down key.

•PLEXTALK will search the keyword from the current position to the end of the file. If the keyword is found, PLEXTALK will play the line that contains the keyword.

•If you press the Down key without entering the keyword, PLEXTALK will regard the current word as the keyword, and search this word.

•The keyword will automatically register on the Navigation Menu.

To navigate the same keyword sequentially, do the following:

•Press the Up or Down key several times until the PLEXTALK announces the keyword.

•Press the Left key to search the keyword toward the beginning of the file.

•Press the Right key to search the keyword toward the end of the file.

Note: If you press the Play/Stop key when playing the line containing the keyword, the PLEXTALK starts to play from the position of the keyword.

Note: If the keyword does not exist, PLEXTALK announces that the keyword was not found and the playback position will remain the current position.

Note: When the PLEXTALK searches the keyword backwards, searching process will be finished at the beginning of the file. When the PLEXTALK searches forwards, searching process will be finished at the end of the file.

Note: To cancel searching the keyword, press the Up key or Star key. The playback position will remain the current position.

Note: You can register up to three keywords. When three keywords have been registered, adding one more keyword will remove the oldest keyword.

Note: You can use numeric characters or symbols as the keyword.

Note: The PLEXTALK does not distinguish between lowercase and uppercase characters when searching the keyword.

Note: If the playback mode of multimedia DAISY title is set to "Text", you are able to use the searching process.

## 10.4. Navigating by page

•Press the GoTo key repeatedly to select page navigation.

•Using the Number Pad, type in the number of the page you wish to go to.

•Press the Enter key to confirm or the Cancel key to clear your entry.

Note: Page navigation is only available in EPUB2 or PDF file.

## 10.5. Navigating by heading

•Press the GoTo key repeatedly to select heading navigation.

•Using the Number Pad, type in the number of the heading you wish to go to.

•Press the Enter key to confirm or the Cancel key to clear your entry.

Note: Heading navigation is only available in EPUB2 or PDF file.

## 10.6. Playback mode

When playing Text Files, you can select a preferred playback mode. The available options are as follows:

•Normal playback: It plays through the title and stops when finished.

•Repeat title: the current title will repeat continuously until playback is stopped.

# 11. Playing Audio Book

PLEXTALK is able to play the "Audio Book" format. PLEXTALK supports the following types of audio files:

•.aa files from Audible.com.

•MP3, Wave, WMA, AMR-WB+, Ogg Vorbis and LKF

Create an "Audible" folder at the root of the SD card and store the ".aa" files in this folder. If there are other types of audio files than ".aa" in the "Audible" folder, PLEXTALK does not recognize these files. PLEXTALK handles an ".aa" file as one title. Therefore, an ".aa" file can be available through "title" navigation, but not available through file navigation.

Meanwhile, you should create a "PlexAudioBook" folder at the root of the SD card and store Audio Book files other than ".aa" in this folder.

All audio files stored in the "PlexAudioBook" folder are played as "Audio Book", not as "Music". PLEXTALK handles a folder in "PlexAudioBook" folder as one title.

Navigations of audio book are exactly similar to Music files. Refer to section 9 "Playing Audio Files or Music CDs" for details.

Note: PLEXTALK recognizes ".aa" files in folders other than the "Audible" folder, but it is recommended that you should store the ".aa" files in the "Audible" folder.

Note: PLEXTALK does not recognize ".aa" files in the "PlexAudioBook" folder.

Note: ".aa" audio files support heading and percent navigation.

Note: Two playback modes, "Normal playback" and "Repeat title" options, can be selected from shuffle and repeat settings.

# 12. Recording a DAISY title

PLEXTALK is able to record DAISY titles to an SD card, USB device or Internal Memory. This chapter shows you the recording method and recording settings.

Note: PLEXTALK is also able to record Voice Bookmarks and Voice Memos to internal memory. Refer to section 6.16 "Using bookmarks" and section 15 "Using PLEXTALK's Voice Memo functions" for details of Voice Bookmarks and Voice Memos.

## 12.1. Basic information about recording with PLEXTALK

Before you start recording with PLEXTALK, you should be clear about several important points as detailed below.

•PLEXTALK always records in the DAISY format.

•Recorded DAISY titles are stored in the "PlexRecording" folder of SD card, USB device or Internal Memory.

•While you are recording, PLEXTALK automatically insert phrase information in the recording, which can help you move easily to important portions of the material.

## 12.2. Recording hardware

Your PLEXTALK is supplied with the basic accessories that you require to make an initial voice recording. You may, however, wish to consider purchasing additional accessories to enhance the quality of your recordings and to enable you to record from external sources such as tape recorders, CD players, radios and other similar devices. The basic items that you require, or may wish to consider purchasing, are as follows:

•SD card or USB device: You can record to and edit information on an SD card or USB device.

•External Microphone (Optional): Dynamic microphones and plug-in power stereo microphones can be used. Select any suitable microphone that uses a 3.5 millimeter diameter mini jack plug.

CAUTION:  When a condenser microphone is used, it must be plug-in power compatible and must be used correctly, otherwise it may be damaged. Please read the microphone manual carefully before use.

•Audio cable (Optional): Assuming that other devices have a suitable connection, you can use an audio cable with 3.5 millimeter diameter stereo mini jack plugs to record direct from a cassette player, CD player, radio or other similar device.

•Headphones: A set of stereo headphones with 3.5 millimeter diameter stereo mini jack plug is included.

## 12.3. Recording media

This section highlights a number of issues that you should be aware of before recording.

•With PLEXTALK, you can use an SD card, USB device or Internal Memory to record and edit DAISY titles and you can erase and rerecord as many times as you want.

•PLEXTALK will work with SD cards with a range of capacities up to a maximum of 32 gigabytes. Before purchasing an additional SD card for use with PLEXTALK, you should ensure that it is compatible. Contact your PLEXTALK dealer for advice.

CAUTION:  Whenever you erase an SD card, USB device or Internal Memory using a personal computer, make sure you select "FAT 16" or "FAT 32".

## 12.4. Recording states

In terms of recording, PLEXTALK has three main states. These are "Recording stand by", "Recording pause" and "Recording".

•In recording stand by, you can adjust the recording level and select a recording mode and sound quality setting. No recording is taking place at this time. The Power LED will turn orange and will flash on and off constantly.

•In recording pause, you can adjust the recording level but cannot alter the recording mode or sound quality settings. Again, no recording is taking place at this time. The Power LED will turn orange and will flash on and off constantly.

•During recording, audio input is being recorded to the SD card, USB device or Internal Memory. You can adjust the recording level but cannot adjust the recording mode or sound quality setting. The Power LED will be orange and will be on constantly.

## 12.5. DAISY editing mode

DAISY editing mode is the mode for recording or editing a DAISY title. Before recording or editing a DAISY title, the PLEXTALK automatically starts the DAISY editing mode. PLEXTALK also starts this mode when you create a title or prepare to edit a DAISY title.

When you want to finish the DAISY editing mode, do the following;

•Select "Exit DAISY editing mode" in the "Edit" menu.

•Or switch media by pressing the Title key.

Note: While in the DAISY editing mode, you cannot use network functions.

## 12.6. Basic recording technique

This section outlines the normal steps used in the recording process. The basic steps for recording with PLEXTALK are as follows:

•If you use SD card or USB device, ensure that it is unlocked.

•If you use external microphone or line-in, connect it to the PLEXTALK.

•Press the Title key to select a media you wish to record into.

•Press the Record key once to go into "DAISY editing mode."

•Press the Left or Right key to select a title you wish to record into.

•Set the recording level either manually or using the Auto Gain Control.

•Press the Record key again to start recording.

•Press the Record key again to pause recording at any time.

•Press the Record key to start recording again.

•Press the Play/Stop key to stop recording.

Note:  You can pause recording as often as you need to.

Note: Refer to the section 12.8 "Connecting and selecting an input source" for details on selecting input source.

## 12.7. Muting the guide speech when recording

For best quality recordings, it is recommended that you mute the internal guide speech when you are recording through the built in microphone or through an external microphone. This is to avoid the guide speech being heard in your final recording. You can mute the guide speech in the following three ways:

•Turn down the guide volume to zero; This option works well for those that can see the recording status light but is more difficult for blind users who require the guide speech to confirm whether PLEXTALK is recording or paused.

•Plug in headphones; All output from PLEXTALK is heard through the headphones and no guide speech is recorded.

•Turn off the guide speech during recording; PLEXTALK provides you with the ability to automatically turn off guide speech when you start recording. When you are recording, no guide speech is heard but guide speech returns automatically when you pause or stop recording.

Guide speech during recording can be turned off or on as follows:

•Press the Menu key once to access the Main Menu.

•Use the Left or Right key to go to the "Recording settings" option and select it by pressing the Enter key.

•Use the Left or Right key to go to the "Turn guidance on or off when recording" option and select it by pressing the Enter key.

•Use the Left or Right key to select "Guidance on" or "Guidance off" and press the Enter key to confirm your selection.

## 12.8. Connecting and selecting an input source

When making live recordings of voices, conferences, lectures, presentations and concerts, you may find that you get better quality by connecting an external microphone. To record from devices such as tape recorders, Music CD players and radios, you should connect to PLEXTALK through the external microphone and line in socket, using a suitable audio cable. To connect either an external microphone or an audio cable, do the following:

•Insert the microphone or audio cable jack plug into the combined external microphone and line in socket.

•PLEXTALK will confirm that it is connected by saying "External Microphone mode" or "Line in mode".

•If PLEXTALK says "External Microphone mode", this indicates that the "Select external audio input jack" setting is set for a microphone.

•If PLEXTALK says "Line in mode", this indicates that the setting is set for a line in input.

•If you have connected a microphone and PLEXTALK says "Line in mode", or if you have connected an audio cable and PLEXTALK says "External Microphone mode", you need now to change the setting in the menu. See later in this section for details on how to do this.

Note:  At this stage, PLEXTALK does not know if you have connected a microphone or an audio cable. The speech that you hear only indicates what the "Select external audio input jack" setting is currently set to.

To change the external input source setting, do the following:

•Press the Menu key to access the Main Menu.

•Use the Left or Right key to go to the "Recording settings" option and select it by pressing the Enter key.

•Use the Left or Right key to go to the "Select external audio input jack" option and select it by pressing the Enter key.

•Use the Left or Right key to select either "Microphone mode" or "Line in mode" and press the Enter key to confirm your selection.

CAUTION:  When you connect from the line output of a CD player to PLEXTALK's Line Input socket, the recorded sound may sometimes be distorted, even if you adjust the recording level. If this happens, you should connect from the CD player's headphone socket to PLEXTALK's Line Input socket and then adjust the CD player's headphone output volume for recording.

## 12.9. Selecting a recording mode and sound quality setting

### 12.9.1. About recording modes and sound quality

When recording, PLEXTALK gives you the ability to select from 6 recording modes. The 6 modes are as follows:

•Default: for making general recordings and voice recordings where there is relatively low background noise.

•Conference: for recording environments, such as conferences, lectures and classrooms, where the background noise is expected to be high.

•Audio: for recording from audio devices such as CD players.

•Analog cassettes: for recording from analogue cassettes.

•Radio: for recording from radio broadcasts.

•Custom: if none of the above options gives you the kind of recording that you want, this recording mode allows you to set all the parameters manually.

In addition PLEXTALK allows you to choose from a number of sound quality settings for each recording mode. By choosing a high quality setting, you reduce the available recording time. By choosing a low sound quality, you can maximize the available recording time but the audio quality will not be so good. The available sound quality settings are as follows:

•PCM 44.1kHz stereo

•PCM 22.05kHz mono

•MP3 256kbps 44.1kHz stereo

•MP3 128kbps 44.1kHz stereo

•MP3 64kbps 44.1kHz mono

•MP3 32kbps 22.05kHz mono

Note: Technical data about recording modes and sound qualities are detailed in Appendix D.

### 12.9.2. Selecting a preset recording mode

You can either select a recording mode and sound quality setting before or after going into recording stand by. To choose a recording mode and sound quality setting before going into recording stand by, do the following:

•Press the Menu key once to access the Main Menu.

•Use the Left or Right key to go to the "Recording settings" option and select it by pressing the Enter key.

•Use the Left or Right key to go to the "Select recording mode" option and select it by pressing the Enter key.

•Use the Left or Right key to go to the mode you want and select it by pressing the Enter key.

•PLEXTALK will then offer you a chance to select a sound quality setting. Use the Left or Right key to move through the available options and select the one you want by pressing the Enter key. PLEXTALK will confirm that your selection has been set.

To choose a recording mode and a sound quality setting after going into recording stand by, do the following:

•Press the Record key once and PLEXTALK will go into recording stand by.

•Press the Menu key once to access the Main Menu.

•Use the Left or Right key to go to the "Select recording mode" option and select it by pressing the Enter key.

•Use the Left or Right key to go to the mode you want and select it by pressing the Enter key.

•PLEXTALK will then offer you a chance to select a sound quality setting. Use the Left or Right key to move through the available options and select the one you want by pressing the Enter key.

•PLEXTALK will confirm that your selection has been set. You will be returned to recording stand by when you have made your selection.

Note:  In both the situations above, the option you select will remain the default setting until you change it in the future.

### 12.9.3. Selecting the Custom recording mode

When you select the Custom recording mode, you have to specify what settings you want for each of the 5 recording parameters. To set the Custom recording mode, do the following:

•Press the Menu key once to access the Main Menu.

•Use the Left or Right key to go to the "Recording settings" option and select it by pressing the Enter key. As above, this is not necessary if you started this process from recording stand by.

•Use the Left or Right key to go to the "Select recording mode" option and select it by pressing the Enter key.

•Use the Left or Right key to go to the "Custom" option and select it by pressing the Enter key.

•PLEXTALK will then offer you a chance to select a sound quality setting. Use the Left or Right key to move through the available options and select the one you want by pressing the Enter key.

•You will now be given the chance to choose your preferred setting for each of the remaining 4 recording parameters in turn. These 4 parameters are "Automatic section setting", "Time to auto stop recording when no sound", "Trigger of starting recording" and "Background noise level of input sound".

•PLEXTALK will speak the name of the parameter. Use the Left or Right key to go to the option you want and select it by pressing the Enter key.

•PLEXTALK will then speak the name of the next parameter. Repeat this process until you have selected a setting for the "Background noise level of input sound" parameter.

•When you have made this last selection, PLEXTALK will confirm that the settings have been set. It will then exit the menu system and return you to the status you started the process from.

## 12.10. Selecting the title to record into

This section details how you can select to record into an existing title or into a new title. To record into an existing DAISY title, do the following:

•Navigate to an existing DAISY title and stop playback.

•Press the Record key to go into recording standby.

•Alternatively, you can select which title you wish to record into by using the Left and Right keys while in recording standby.

•Press the Record key again to start recording. A new section will be created at the end of the current title. All new audio recorded will be appended to the end of the current title, regardless of where you were in the recording when you started this process.

To record into a new title, you can either create a new title first and then record into it or you can create a new title during the recording process. This section details how to create a new title during the recording process. Create a new DAISY title during recording as follows:

•Select SD card, USB device or Internal Memory with the Title key.

•Press the Record key to go into recording standby.

•Whilst in recording standby, use the Left and Right keys to select which title you wish to record into. As you move right you will find that PLEXTALK generates a title called "New title".

•Highlight the "New title" item and press the Record key. PLEXTALK will start recording into the new title.

Note: "New title" is created under the "PlexRecording" folder.

## 12.11. Setting the recording level

When making any recording, it is important to ensure that the recording level is set correctly. PLEXTALK gives you two methods of adjusting the recording level. You can either choose to have the level set automatically or you can set it manually.

You can switch between Automatic Gain Control and manual recording level setting as follows:

•Press the Menu key to access the Main Menu.

•Use the Left or Right key to go to the "Recording settings" option and select it by pressing the Enter key.

•Use the Left or Right key to go to the "Auto Gain Control" option and select it by pressing the Enter key.

•Use the Left or Right key to select "Auto gain control off" or "Auto gain control on" and press the Enter key to confirm your selection.

Note:  If you have turned on the Auto Gain Control, in recording standby, you will hear PLEXTALK say "Auto" repeatedly to indicate that the level will be set automatically.

You can manually adjust the recording level as follows:

•Set the Automatic Gain Control to "Off", using the method detailed above. This means that you have enabled manual setting of the recording level. The recording level can be set in the range 0 to 20 in increments of 1.

•Press the Record key once to go into recording stand by.

•Press the Up key. Each time you press the key, the recording level will increase by one step.

•Press the Down key. Each time you press the key, the recording level will decrease by one step.

•For larger adjustments of the recording level, you can press and hold down the key. PLEXTALK will continue to increase or decrease the level until you release the key or the maximum or minimum setting is reached.

•As you make the adjustments, PLEXTALK will announce the new recording level and will say either "Low", "Good" or "High" to indicate that the recording level is too low, about right or too high. You should aim to get PLEXTALK to say "Good" more often than "Low" or "High".

Note:  If the level monitor is off, you will not hear the "Low", "Good" or "High" announcements.

Note:  When you record audio material with the Auto Gain Control ON, you may not get the recording quality you want. In this situation, redo your recording with the auto gain control off and adjust the recording level manually.

## 12.12. Using the level monitor

The level monitor is PLEXTALK's way of indicating if the recording level is low, good or high or if it is being set automatically. PLEXTALK's default setting is for the level monitor to be on when you enter recording standby. The level monitor will be automatically muted when you subsequently start recording.

To turn the level monitor on or off at any time during recording standby, recording pause or during recording, do the following:

•Press the Title key once.

•PLEXTALK will either say, "Level monitor on" or "Level monitor off".

•Each press of the Title key will toggle between these two settings.

Note:  If the guidance during recording is turned off, PLEXTALK will not announce your key presses during recording. If you require this facility, you are advised to use headphones and turn on the guidance to avoid the level monitor being picked up in the recording.

## 12.13. Turning the recording monitor on and off

PLEXTALK enables you to monitor any recording as it is being made, so you can gauge how the recording will sound in the end. You can turn the recording monitor on or off in recording stand by, recording pause or during recording. This is done as follows:

•Connect a set of stereo headphones to the headphone socket on the top edge of PLEXTALK.

•Press the Bookmark key once to turn the recording monitor off.

•Press the Bookmark key again to turn the recording monitor on.

Note: You do not require headphones when recording from a line in source.

Note:  For all recordings, PLEXTALK will automatically turn on the recording monitor by default.

## 12.14. Getting recording information

You can get information about the current status of the recording by pressing the Number 5 key.

Note:  If voice guidance during recording is set to "Guidance off", you will not hear any of the above information during recording. You will still hear the information in the recording stand by and recording pause states.

# 13. Editing a DAISY title

## 13.1. Editing heading markers during recording

Heading markers can be set either when recording is paused or during recording. Both methods are detailed below:

•To set a heading during recording, press the GoTo key once. The new heading will be set immediately.

•To set a heading when recording is paused, press the GoTo key once. The heading will be inserted when recording recommences.

•Whilst still in recording pause, you can cancel the heading by pressing the GoTo key a second time.

Note:  PLEXTALK will automatically number headings sequentially from the beginning of the title.

## 13.2. Setting a heading after recording

To set a heading after completing the recording, do the following:

•Navigate through your recording to the approximate location where you wish to set the heading.

•During playback or when the playback is stopped, press the Menu key once to access the Main Menu.

•Use the Left or Right key to go to the "Edit" option and select it by pressing the Enter key.

•Use the Left or Right key to go to the "Set heading" option and select it by pressing the Enter key.

•PLEXTALK will then tell you that you are about to set Heading XX and will ask you to set the position.

•You will then hear 3 seconds of audio played repeatedly from the location you are in the recording.

•Adjust the heading position by using your Left or Right key.

•Use these keys to move the heading position backwards or forwards by 0.1 second increments.

•When you are happy that the heading is positioned where you want it, press the Enter key to finish the process. PLEXTALK will confirm that the heading has been set.

Note:  Headings in the recording will be renumbered automatically from the beginning.

## 13.3. Cancelling a heading setting after recording

To cancel a heading after completing the recording, do the following:

•Navigate to the location of the heading you wish to cancel.

•During playback or when the playback is stopped, press the Menu key once to access the Main Menu.

•Use the Left or Right key to go to the "Edit" option and select it by pressing the Enter key.

•Use the Left or Right key to go to the "Cancel heading" option and select it by pressing the Enter key.

•PLEXTALK will then ask you to confirm the cancellation of the specified heading. If you wish to cancel this heading, press the Enter key. PLEXTALK will confirm that the heading has been cancelled.

Note:  Remaining headings will be renumbered automatically from the beginning.

## 13.4. Removing a section or multiple sections

PLEXTALK enables you to remove one or more sections from a DAISY title. Do this as follows:

•Navigate to the title from which you want to remove one or more sections.

•Move back or forward in the title to the approximate position of the first section you wish to remove.

•Press the Menu key once to access the Main Menu.

•Use the Left or Right key to go to the "Edit" option and select it by pressing the Enter key.

•Use the Left or Right key to go to the "Remove section" option and select it by pressing the Enter key.

•PLEXTALK will now ask you to select the first section to be removed. Use the Left or Right key to move back or forward in the title until you hear the section and select it by pressing the Enter key.

•PLEXTALK will now ask you to select the last section that you want to remove. Use the Left or Right key to move back or forward in the title until you hear the section and select it by pressing the Enter key.

•Finally, PLEXTALK will ask you to confirm that you wish to proceed. Press Enter to confirm the action or Cancel to abort.

Note:  If you only wish to remove a single section, simply press Enter on the same section for both the start section and end section and proceed as for removing multiple sections.

Note:  You can only remove multiple sections that are contiguous. To remove multiple sections that are not contiguous, you need to remove each section individually.

Note:  PLEXTALK will automatically reorder the heading numbering for the remaining sections in the title.

# 14. Media management features

## 14.1. Erasing an SD card, USB device or Internal Memory

PLEXTALK allows you to erase an SD card, USB device and Internal Memory.

To erase an SD card, USB device or Internal Memory, do the following:

•Press the Menu key once to access the Main Menu.

•Use the Left or Right key to go to the "Media Management" option and select it by pressing the Enter key.

•Use the Left or Right key to select "Delete all files on the SD card", "Delete all files in the USB device" or "Delete all files in Internal Memory" and press the Enter key.

•PLEXTALK will ask you to confirm that you wish to continue with the formatting process. Press the Enter key again to continue or the Cancel key to abort.

•If you choose to continue, the formatting process will start. PLEXTALK will announce when the process is complete.

Note:  If the SD card, USB device or Internal Memory is erased, all data on the media will be deleted.

Note: Bookshelf will not be deleted if you erase the Internal Memory. If you want to delete the Bookshelf contents, return the DAISY title. Refer to the Section 25.2.4, "Returning a title" for details.

Caution: If you execute the Menu item "Delete all files in Internal Memory", all your files included in the "Alarm" folder will be deleted.

## 14.2. Creating a new title

To create a new title on the SD card, USB device or Internal Memory when not in recording standby, do the following:

•Press the Title key to select SD card, USB device or Internal Memory.

•Press the Menu key once to access the Main Menu.

•Use the Left or Right key to go to the "Media management" option and select it by pressing the Enter key.

•Use the Left or Right key to go to the "Create a new title" option and select it by pressing the Enter key.

•PLEXTALK will then ask you to confirm that you wish to proceed. Press the Enter key to confirm or the Cancel key to abort.

Note: New title is created under the "PlexRecording" folder.

## 14.3. Deleting a title

PLEXTALK enables you to delete a title on an SD card, USB device or Internal Memory.

To delete a title, do the following:

•Go to the title that you wish to delete.

•Press the Menu key once to access the Main Menu.

•Use the Left or Right key to go to the "Media management" option and select it by pressing the Enter key.

•Use the Left or Right key to go to the "Delete the selected title" option and select it by pressing the Enter key.

•PLEXTALK will then ask you to confirm that you wish to proceed. Press the Enter key to confirm or the Cancel key to abort.

## 14.4. Deleting a file

You can delete a file of "Music", "Audio book" or "Podcast". To delete a file, do the following:

•Go to the file that you wish to delete.

•Press the Menu key once to access the Main Menu.

•Use the Left or Right key to go to the "Media management" option and select it by pressing the Enter key.

•Use the Left or Right key to go to the "Delete a file" option and select it by pressing the Enter key.

•PLEXTALK will then ask you to confirm that you wish to proceed. Press the Enter key to confirm or the Cancel key to abort.

Note: If you delete the Audio Book from "Audible", use the menu option "Delete the selected title".

## 14.5. Preparing to edit a DAISY title

If you have already built a recorded title or, if you have received a title from elsewhere and you wish to edit it, you first need to prepare the title for editing. To prepare a title for editing, do the following:

•Go to the title you wish to prepare for editing.

•Press the Menu key once to access the Main Menu.

•Use the Left or Right key to go to the "Media Management" option and select it by pressing the Enter key.

•Use the Left or Right key to go to the "Prepare for editing" option and select it by pressing the Enter key.

•PLEXTALK will speak a message asking you if you wish to proceed. If you wish to continue, press the Enter key or press the Cancel key to abort the process.

•During the process, PLEXTALK will speak progress messages and will announce when it is complete. If you wish to cancel the process whilst it is running, press the Cancel key.

CAUTION: When the battery level is low, the process may be aborted. Therefore, it is recommended that you connect to a mains power source before performing this process.

Note: If you prepare a title for editing, you will be able to record or edit this title.

Note: If you prepare for editing a title in the category of "DAISY book", it remains in this category.

## 14.6. Building a DAISY title

The Build book process converts a DAISY title, recorded on the PLEXTALK, into a DAISY book that can be played on other DAISY hardware players and on a personal computer using DAISY playback software. To perform the Build book process, do the following:

•Go to the title you wish to build.

•Press the Menu key once to access the Main Menu.

•Use the Left or Right key to go to the "Media Management" option and select it by pressing the Enter key.

•Use the Left or Right key to go to the "Build book" option and select it by pressing the Enter key.

•Use the Left or Right key to select DAISY type "DAISY 2.02" or "DAISY 3" and press the Enter key.

•PLEXTALK will speak a message asking you if you wish to proceed. If you wish to continue, press the Enter key or press the Cancel key to abort the process.

•PLEXTALK will speak progress messages and will announce when it is complete. If you wish to cancel the process whilst it's running, press the Cancel key.

CAUTION: When the battery level is low, the process may be aborted. Therefore, it is recommended that you connect to a mains power source before performing this process.

Note: If you build a title, you will be unable to record or edit this title. When you would like to record or edit this title once more, prepare this title for editing.

Note: If you build a title in the category of "Recordings", it remains in this category.

Note: The Build book process, for a DAISY title, will require between 30MB and 40MB of space on the media.

# 15. Using PLEXTALK's Voice Memo functions

In addition to using PLEXTALK as a DAISY player, music player and as a DAISY recorder, you are also able to use it for recording simple Voice Memos.

## 15.1. Recording a new Voice Memo

To record a new Voice Memo, do the following:

•Press and hold down the Record key and speak the memo.

•Release the key when you are finished.

•PLEXTALK will insert a new Voice Memo into the internal memory and will then return you to the title you were in before recording the new Voice Memo.

Note:  You cannot record a continual Voice Memo that lasts for more than 1 minute. When you exceed 1 minute, PLEXTALK automatically stops recording.

Note:  The total amount of available recording time for Voice Memos is 30 minutes. The maximum number of individual memos is 200.

Note:  If the recorded time exceeds 30 minutes, or the number of memos exceeds 200, you will hear a warning message. You should then delete any unnecessary Voice Memos or backup your Voice Memos to an SD card.

Note:  For Voice Memos, the audio compression is fixed at MP3 32kbps monaural.

## 15.2. Reviewing existing Voice Memos

You can review your existing Voice Memos either sequentially or by going direct to a specific Voice Memo. To listen to your existing Voice Memos sequentially, do the following:

•Press the Title key repeatedly until PLEXTALK says "Voice Memo".

•Use the Left or Right key to move sequentially through the existing Voice Memos.

•If you wish to hear the current Voice Memo again, press the Play/Stop key.

To go direct to a specific Voice Memo, do the following:

•Press the GoTo key once.

•Use the Number Pad to enter the number of the required Voice Memo, followed by the Enter key.

Note:  You can go direct to the last Voice Memo by pressing the GoTo key and entering 0 followed by the Enter key.

## 15.3. Back up Voice Memos to SD card

Please refer to Section 16.2. for information about Back up Voice Memos.

## 15.4. Deleting Voice Memos

If you wish to delete a single Voice Memo, do the following:

•Navigate to the Voice Memo you wish to delete, as detailed previously.

•Press the Menu key once to access the Main Menu.

•Use the Left or Right key to go to the "Voice Memo" option and select it by pressing the Enter key.

•Use the Left or Right key to go to the "Delete the selected Voice Memo" option and press the Enter key.

•PLEXTALK will ask you to confirm that you wish to continue. Press the Enter key again to continue or the Cancel key to abort.

If you wish to delete all Voice Memos, do the following:

•Press the Menu key once to access the Main Menu.

•Use the Left or Right key to go to the "Voice Memo" option and select it by pressing the Enter key.

•Use the Left or Right key to go to the "Delete all Voice Memos" option and press the Enter key.

•PLEXTALK will ask you to confirm that you wish to continue. Press the Enter key again to continue or the Cancel key to abort.

# 16. Backup

## 16.1. General backup information

Before backing up any data, the following points should be noted:

•When backing up data from a media to another media, the former is called "source media" and the latter is called "target media."

•You can backup a variety of data from SD card to external USB media or Internal Memory, or vice versa. However, an external USB CD/DVD drive can only be used as a source for backup purposes.

•In the back up process, PLEXTALK gives a verbal warning if there is insufficient space available on the target media.

•If you backup from a Music CD using an external USB CD/DVD drive, you can choose the audio compression to be used. It will be backed up as Audio Files and not in DAISY format.

•When the battery level is low, the backup process may be aborted. Therefore, it is recommended that you connect to a mains power source before backup.

•If you wish to cancel the backup process whilst it is running, press the Cancel key.

•Podcast episodes and Web radio cannot be backed up.

•Data will be backed up into these folders:

DAISY: "PlexBackup" folder

Recordings: "PlexBackup" folder

Music: "PlexBackup" folder

Text: "PlexBackup" folder

".aa" Audio book: "Backup" folder under the "Audible" folder

Other Audio book: "Backup" folder under the "PlexAudioBook" folder

## 16.2. Back up Voice Memos

This section details how to backup all Voice Memos from the internal memory to SD card.

•Insert an SD card with sufficient free space.

•Press the Title key to Navigate to Voice Memo and press the Enter key.

•Press the Number 9 key once to access the Backup feature.

•PLEXTALK will ask you to confirm that you wish to continue with the backup. Press the Enter key to continue or the Cancel key to abort.

•If you choose to continue, the backup process will start. PLEXTALK will announce when the backup is complete.

Note:  The internal memory for Voice Memos can only be backed up into "PlexBackup" folder of SD card.

Note:  Voice Memos are saved as a DAISY title with level 1 navigation. Each memo is saved as a separate section.

Note:  When backing up Voice Memos, all the Voice Memos are backed up. You cannot back up a particular Voice Memo.

## 16.3. Backup a title

To backup a title, do the following:

•If necessary, insert an SD card or USB device with sufficient free space and ensure that it is unlocked.

•Navigate, on the source media, to the title that you wish to backup, and press the Enter key.

•Press the Number 9 key once to access the Backup feature.

•If the PLEXTALK says "Select a target media", press the Left or Right key to select the target media and then press the Enter key.

•When backing up from a Music CD, press the Left or Right key to go to your preferred audio compression and press the Enter key to confirm your selection.

•When backing up from a Music CD, PLEXTALK will ask you to which category you are going to backup, "Music" or "Audio Book". Use the Left or Right key to select your preferred category and press the Enter key.

•PLEXTALK will ask you to confirm that you wish to continue with the backup. Press the Enter key to continue or the Cancel key to abort.

•If you choose to continue, the backup process will start. PLEXTALK will announce when the backup is complete.

## 16.4. Backup time for music CD

For the range of available sample rates or bit rates, the approximate backup times are given below:

Backing up about 50 minutes of audio from a music CD to SD card:

•PCM 44.1kHz Stereo: 35 minutes approximately.

•MP3 256kbps 44.1kHz Stereo High Quality: 50 minutes approximately.

•MP3 256kbps 44.1kHz Stereo: 40 minutes approximately.

•MP3 128kbps 44.1kHz Stereo High Quality: 55 minutes approximately.

•MP3 128kbps 44.1kHz Stereo: 40 minutes approximately.

Note:  All times are approximate. The backup time will depend on the size of the source title and numbers of files. It will also depend on the writing speed of the SD card.

# 17. Entering text

This chapter describes how to enter characters.

## 17.1. Entering alphabet by ABC mode

Multiple characters are mapped to each numeric key and one of the character is selected each press.

Order of character mapping.

Characters mapped to a numeric key appear in order as "lower case, number, upper case". For example, pressing 5 key to switch the characters in order of "j, k, l, 5, J, K, L".

Character is confirmed after two seconds automatically.

After selecting a character, the character is automatically confirmed after two seconds inactivity and PLEXTALK announces the character.

Entering multiple characters mapped to a key continuously.

To enter multiple characters mapped to a key without waiting two seconds, press the Right key to enter next character. For example, to enter "ab", enter "a" by pressing the number 2 key once, and press the Right key, then enter "b" by pressing 2 key twice.

Example of operation.

Entering "http".

h: Press the 4 key twice.

t: Press the 8 key once.

t: Press the 8 key once.

p: Press the 7 key once.

Characters and numbers available for input.

The characters mapped to each numeric key are as follows:

1 contains ".", "@", "/", ":", "1"

2 contains a, b, c, 2, A, B, and C

3 contains d, e, f, 3, D, E, and F

4 contains g, h, i, 4, G, H, and I

5 contains j, k, l, 5, J, K, and L

6 contains m, n, o, 6, M, N, and O

7 contains p, q, r, s, 7, P, Q, R, and S

8 contains t, u, v, 8, T, U, and V

9 contains w, x, y, z, 9, W, X, Y, and Z

0 contains ?, _, =, and 0

Confirming entered characters.

Press the Play/Stop key to listen to the entered text.

Changing character type.

To change type of characters, press the bookmark key to switch between "ABC mode", "Number mode" and "Symbol table".

Note: If the input allows number only, the mode is not switched even if the bookmark key is pressed.

## 17.2. Entering numbers by number mode

Number mode is the mode to enter numbers by pressing numerical keys. Press the bookmark key several times to switch to the Number mode, and then enter numbers.

Enter numbers by pressing numeric keys. The 1 to 0 keys correspond to the numbers.

## 17.3. Entering symbols by Symbol table

Symbol table is the mode to enter symbols by using the number 2, 4, 6 and 8 keys to scroll characters on the symbol table below and pressing the Pound/Hash key to enter the character. Press the bookmark key several times to switch to the symbol table, and then enter symbols.

To scroll the characters on the symbol table, press the number 2 key to scroll up, press the number 8 key to scroll down, press the number 4 key to scroll left and press the number 6 key to scroll right. To enter a symbol several times, press the Pound/Hash key repeatedly.

Symbol table is described below.

| . | , | ' | " | : | ; | ? | ¦ |
| --- | --- | --- | --- | --- | --- | --- | --- |
| + | - | = | / | \ | _ | [ | ] |
| ! | @ | # | $ | % | ` | { | } |
| ^ | & | * | ( | ) | ~ | < | > |

## 17.4. Moving entering position

Move back a character.

Press the Left key to move back a character.

Move ahead a character.

Press the Right key to move ahead a character.

Move back a word.

Press the Left key and number 0 key together to move back a word.

Move ahead a word.

Press the Right key and number 0 key together to move ahead a word.

Move back to the beginning of the line.

Press and hold the Left key to move back to the beginning of the line. Alternatively press the Star key and 5 key together.

Move ahead to the end of the line.

Press and hold the Right key to move ahead to the end of the line. Alternatively press the Star key and 8 key together.

Move back a line.

Press the Up key to move back a line.

Move ahead a line.

Press the Down key to move ahead a line.

Move back a screen (40 lines up).

Press the Star key and 6 key together to move back 40 lines.

Move ahead a screen (40 lines down).

Press the Star key and 9 key together to move ahead 40 lines.

Move back a paragraph.

Press the Up key and number 0 key together key to move back a paragraph.

Move ahead a paragraph.

Press the Down key and number 0 key together to move ahead a paragraph.

Move back to the beginning of the text file.

Press and hold the Up key to move back to the beginning of the text file.

Move ahead to the end of the text file.

Press and hold the Down key to move ahead to the end of the text file.

## 17.5. Enter space, linefeed and erase characters

Space (Blank character).

Press the Hash key to enter Space (Blank character).

Backspace (Delete the previous character).

Press the Star key to delete the previous character.

Deleting a character at current position.

Press the Star key and 7 key together to delete the current character.

Deleting all characters in the Setting menu.

Press and hold the Star key.

Deleting all characters during editing text file.

Refer to the section 18.7. "Deleting all text".

Deleting current line.

Refer to the section 18.6. "Deleting current line".

New line.

Press the Star key and Hash key together to begin a new line.

Tab character.

Press the Star key and 2 key together to enter a tab character.

Switching "insert" mode and "overwrite" mode.

Press the Star key and 4 key together to switch the "insert" mode and "overwrite" mode.

# 18. Editing text files

PLEXTALK lets you write a document and save it as a text file. This chapter gives the information on creating and saving a text file.

## 18.1. Creating a new text file

To create a new text file, do the following;

•Press the Menu key to access the Main Menu.

•Use the Left or Right key to select the "Media management" option and press the Enter key.

•Use the Left or Right key to select "Create a new text file" option and press the Enter key.

•PLEXTALK will ask you to confirm that you wish to continue. Press the Enter key to continue. A new text file will be created and now you are in the Text editing mode.

## 18.2. Editing an existing text file on the SD card, USB device or Internal Memory

To start editing an existing text file, do the following;

•Go to the file that you wish to edit.

•Press the Menu key.

•Use the Left or Right key to select "Edit" option and press the Enter key.

•PLEXTALK will ask you to confirm that you wish to continue. Press the Enter key to continue. Now you are able to edit the text file.

## 18.3. Saving the edited text

To save the edited text, do the following;

•Press the Menu key.

•Use the Left or Right key to select "Save the edited text" option and press the Enter key.

•PLEXTALK will ask you to confirm that you wish to continue. Press the Enter key to continue or Cancel key to abort.

•When the Enter key is pressed, PLEXTALK saves the edited text then returns to text editing mode.

Note: File name will be the first line of the text. If the text has nothing at the time of saving file, current date and time is applied as the file name.

## 18.4. Exiting text editing

### 18.4.1. Saving and exiting text editing

To save and exit text editing, do the following;

•Press the Title key.

•PLEXTALK will ask you to confirm that you wish to save and exit text editing. Press the Enter key to exit or Cancel key to continue text editing.

•When the Enter key is pressed, PLEXTALK saves the text and exits text editing.

Alternatively, do the following:

•Press the Menu key.

•Use the Left or Right key to select "Save and exit text editing" option and press the Enter key.

•PLEXTALK will ask you to confirm that you wish to proceed. Press the Enter key to proceed or Cancel key to abort.

•When the Enter key is pressed, PLEXTALK saves the text and exits text editing.

### 18.4.2. Exiting text editing without saving the text

To exit the text editing without saving the text, do the following;

•Press the Menu key.

•Use the Left or Right key to select "Exit text editing without saving" option and press the Enter key.

•PLEXTALK will ask you to confirm that you wish to continue. Press the Enter key to continue or Cancel key to abort.

•If the Enter key is pressed, PLEXTALK discards the edited information then the text is restored.

## 18.5. Renaming the file

To rename the file, do the following;

•Press the Menu key.

•Use the Left or Right key to select "Rename the file" option and press the Enter key.

•Edit the file name.

•Press the Down key and PLEXTALK will ask you to confirm that you wish to continue.

•Press the Enter key to continue or Cancel key to abort.

Note: Press the Play/Stop key to listen to the new filename.

## 18.6. Deleting current line

To delete the current line, do the following;

•Press the Menu key.

•Use the Left or Right key to select "Delete the current line" option and press the Enter key.

•PLEXTALK will ask you to confirm that you wish to continue. Press the Enter key to continue or Cancel key to abort.

## 18.7. Deleting all text

To delete all text, do the following;

•Press the Menu key.

•Use the Left or Right key to select "Delete all text in the file" option and press the Enter key.

•PLEXTALK will ask you to confirm that you wish to continue. Press the Enter key to continue or Cancel key to abort.

# 19. Calendar function

You can add events to the Calendar in your PLEXTALK. Also you can check to the events in your PLEXTALK. When you turn on a reminder function, the PLEXTALK will notify the event at set time.

## 19.1. Select the date in Calendar

To select the date in the Calendar, do the followings:

•Press the Title key to select "Calendar". Now you are in today on the Calendar and the navigation level is "day".

•Press the Left or Right key to go to the previous date or next date. The PLEXTALK announces the date and the existence of the event.

•To check the date and the existence of the event, press the Play/Stop key.

•To check the start time of the events, press the 5 key.

To select the date one week ahead, one month ahead or one year ahead, do the followings:

•Press the Up key to go up from the "day" level  to the "week" level. Then press the Left or Right key to select weeks ago or weeks ahead.

•Press the Up key to go up from the "week" level  to the "month" level. Then press the Left or Right key to select months ago or months ahead.

•Press the Up key to go up from the "month" level  to the "year" level. Then press the Left or Right key to select years ago or years ahead.

•If you are in the "day" level, press the Down key to go down to the "event" level. Then press the Left or Right key to select the previous or next event.

Note: You can move quickly to the past or future by pressing and holding the Left or Right key.

Note: Press the 5 key to check the detail of the event when in the "event" level.

To select the date directly, do the followings:

•Press the Title key to select "Calendar".

•Press the GoTo key.

•Enter the year (with 4 digits), month and day of the event with numeric keys and press the Enter key to move to the specified date. If you press the Enter key without entering the date, you will move to "today".

## 19.2. Add an event

To add an event, do the followings:

•Select the date which you want to add an event. Then press the Record key to add the event.

•You can entitle the event. Press and hold the Record key to record the title. Or enter the title by numeric keys and press the Down key.

•If you immediately press the Down key, the event will have no title.

•The PLEXTALK asks when the event starts. Enter the starting date with numeric keys.

•The PLEXTALK asks the event is an all day event or not. Press the Left or Right key to select "All day" or "Input starting time" and press the Enter key.

•If you select "All day", enter the "ending date" of the event.

•If you select "Input starting time", enter the "starting time", "ending date" and "ending time" of the event.

•Press the Left or Right key to select repeat setting from "Non repeating", "Daily", "Weekly", "Monthly" and "Yearly".

•If you select except "Non repeating" option, the PLEXTALK ask you whether you set ending date of repeat. Press the Left or Right key to select "Never ending" or "Set ending date of repeat". If you select "Set ending date of repeat", enter the ending date of repeat by numeric keys.

•Select reminders. If you did not select "All day",  press the Left or Right key to select reminders from "No reminder", "At time of event", "5 minutes before", "30 minutes before" and "1 hour before". If you selected "All day",  select reminders from "No reminder" or "17 o'clock of the day before". When you select "No reminder", the menu will be closed.

•Press the Left or Right key to select reminder sound from "Long chime", "Short beep" or "Title".

•Press the Left or Right key to adjust the volume of the reminder from 5 volumes. Pressing the Left key turns the volume down or the Right key turns the volume up.

Note: Refer to the section 17, "Entering text" for the detail of entering text with numeric keys.

Note: Available period is year 2000 to 2080.

## 19.3. Reminder of the event

The PLEXTALK reminds you the event at the scheduled time.

•To stop the reminder, press any key.

Note: The PLEXTALK will not remind you the event on the following conditions:

- Connecting to the computer via USB.

- Connecting to the computer via network.

- The power is turned off.

## 19.4. Edit the event

To edit the event, do the followings:

•Select the event that you want to edit.

•Press the Bookmark key once and press the Enter key. Refer to the section 19.2. "Add an event" to edit in detail.

•If you abort editing, press and hold the Up key.

## 19.5. Delete the selected event

To delete the selected event, do the followings:

•Select the event you want to delete.

•Press the Bookmark key twice and then press the Enter key.

•The PLEXTALK ask you whether you want to delete or not. Press the Enter key to delete or the Cancel key to abort.

## 19.6. Calendar Menu

### 19.6.1. Delete all events

To delete all events, do the followings:

•Press the Menu key.

•Press the Left or Right key to select "Calendar" and press the Enter key.

•Press the Left or Right key to select "Delete all events" and press the Enter key.

•The PLEXTALK ask you whether you want to delete or not. Press the Enter key to delete or the Cancel key to abort.

### 19.6.2. Delete the events of more than three months ago

To delete the events of more than three months ago, do the followings:

•Press the Menu key.

•Press the Left or Right key to select "Calendar" and press the Enter key.

•Press the Left or Right key to select "Delete the events of more than three months ago" and press the Enter key.

•The PLEXTALK ask you whether you want to delete or not. Press the Enter key to delete or the Cancel key to abort.

# 20. Alarm function

## 20.1. Configure alarm notification

The PLEXTALK notices you at a scheduled time using alarm function. You can set 5 alarms.

### 20.1.1. Setting or changing alarm

To set or edit an alarm, do the followings:

•Press the Menu key.

•Press the Left or Right key to select "Alarm" and press the Enter key.

•Press the Left or Right key to select the alarm you want to set and press the Enter key.

•Press the Left or Right key to select "Change the setting" and press the Enter key.

•Enter the time by numeric keys and press the Enter key.

•Press the Left or Right key to select from "Non repeating", "Daily" or "Set day of the week" and press the Enter key.

•If you select "Set day of the week", press the Left or Right key to select the day of the week and then press the Bookmark key. Press the Bookmark key once more to cancel it. Finally press the Enter key to set days.  You can set multiple day in the week.

•Press the Left or Right key to select either "Snooze off" or "Snooze on" and press the Enter key. If you select "Snooze on", the PLEXTALK beeps every 5 minutes up to 5 times.

•You can entitle the alarm. Press and hold the Record key to record the title. Or enter a title by numeric keys and press the Down key. If you immediately press the Down key, the alarm will have no title.

•Press the Left or Right key to select alarm sound from among "Bell", "Beep", "Chicken", "Title" or "Audio in Alarm folder" and press the Enter key.

•Press the Left or Right key to select the volume from among 5 volume levels. Press the Left key to turn down the volume and the Right key to turn up the volume. After selecting volume level, press the Enter key to confirm. Then the alarm setting is finished and the PLEXTALK sets the alarm ON and returns alarm menu.

Note: If you select "Audio in alarm folder", the PLEXTALK sounds the audio file which is found first.

Note: "Alarm" folder exists in the root of the Internal memory.

### 20.1.2. Set alarm on or off

To set or off the selected alarm, do the followings:

•Press the Menu key.

•Press the Left or Right key to select "Alarm" and press the Enter key.

•Press the Left or Right key to select the alarm you want to set and press the Enter key.

•Press the Left or Right key to select "Select On or Off" and press the Enter key.

•Press the Left or Right key to select either "Alarm Off" or "Alarm On" and press the Enter key. After the alarm is set, the PLEXTALK returns alarm menu.

Note: If you select the "Alarm Off", the alarm does not beep at the set time.

### 20.1.3. Clear alarm setting

To clear the selected alarm setting, do the followings:

•Press the Menu key.

•Press the Left or Right key to select "Alarm" and press the Enter key.

•Press the Left or Right key to select the alarm you want to clear and press the Enter key.

•Press the Left or Right key to select "Clear alarm setting" and press the Enter key.

•The PLEXTALK asks you to clear the setting. Press the Enter key to clear or the Cancel key to abort. After the alarm is cleared, the PLEXTALK returns the alarm selection.

Note: If you delete the alarm setting, the alarm will be turned off and the time is set at 0:00am.

## 20.2. Reminder of the alarm

PLEXTALK beeps the alarm at the set time.

To stop the alert, press any key.

To stop the "Snooze" function, press and hold the Menu key.

Note: If "Alarm 1" and "Alarm 2" are set exactly the same, the PLEXTALK will not beep "Alarm2" but "Alarm 1".

Note: If an event of calendar and an alarm are set at the same time, the PLEXTALK notices the Calendar event.

Note: The PLEXTALK does not notice on the following conditions:

- Connecting to the computer via an USB.

- Connecting to the computer via the network.

- The power is turned off.

# 21. Network settings

PLEXTALK has a network capability using wireless LAN.

WARNING: Refrain from using the wireless LAN function in an aircraft or a hospital according to the instructions of an airline company or a hospital.

## 21.1. Connecting to the wireless network

### 21.1.1. Enable wireless network device

To enable the network device, do the following:

•Press the Menu key.

•Use the Left or Right key to go to "System settings" option and press the Enter key.

•Use the Left or Right key to go to "Network settings" option and press the Enter key.

•Use the Left or Right key to go to "Enable or disable wireless network device" option and press the Enter key.

•Use the Left or Right key to select "Enable" and press the Enter key.

### 21.1.2. Confirm available wireless network and connect

Confirm currently available wireless network from "Available wireless network" list.

•Press the Menu key.

•Use the Left or Right key to go to "System settings" option and press the Enter key.

•Use the Left or Right key to go to "Network settings" option and press the Enter key.

•Use the Left or Right key to go to "Wireless network settings" option and press the Enter key.

•Use the Left or Right key to go to "Available wireless network" option and press the Enter key.

•PLEXTALK announces a name, link status and signal strength of one of the currently available networks. Press the Left or Right key to confirm information on another network

•If you select the network which is connecting and press the enter key, PLEXTALK disconnects the network and closes the Menu. From this time, PLEXTALK will not connect to this network automatically. And then, PLEXTALK tries to connect to a network in order of priority set in PLEXTALK.

•If you select the network which was disconnected, PLEXTALK tries to connect to it. When PLEXTALK establishes connection, the menu will be closed.

•If the selected network has not configured yet, do the following.

•PLEXTALK automatically detects the security mode of the network.

•Enter the password with Numeric keys. If "No security" is selected, entering the password is not required. After you have finished, press the Down key to confirm.

•If the PLEXTALK cannot detect the security mode, select the security mode with the Left or Right key and enter the password with Numeric keys and then press the Down key.

•Select "stealth setting". If the wireless network uses stealth function, select "ON". If it does not, select "OFF". Then press the Enter key.

•Select "DHCP setting". Use the Left or Right key to select "Enable" for automatic setting or "Disable" manual setting and press the Enter key. If "Enable" is selected, setup menu proceeds for proxy setting.

•Enter IP address. One IP address consists of 4 areas. PLEXTALK reads numbers of the first area. If you want to change the numbers, enter new numbers by numerical keys then press the Right key to go to the next area. If you do not want to change the numbers, simply press the Right key.

•Enter numbers from the second area to the fourth area. Press the Right key to go to the next area. To go back to the previous area, press the Left key. Once entering is completed, press the Down key.

•Enter subnet mask. Do the same method as entering of IP address.

•Enter default gateway. Do the same method as entering of IP address.

•Enter DNS setting. Primary and secondary DNS settings are available. Do the same method as entering of IP address.

•Select "proxy server setting". To use a proxy server, press the Left or Right key to select "Enable" and then press the Enter key. If you do not use a proxy server, select "Disable" and press the Enter key.

•If "Disable" is selected and the Enter key is pressed, menu is closed.

•If "Enable" is selected, enter the address of the proxy server and press the Down key when the entering is finished.

•Then enter a port number of the proxy server and press the Down key. PLEXTALK tries to connect to the wireless network.

•The menu automatically closes when connection is successful. If the connection failed, PLEXTALK automatically returns to the menu to select an available wireless network.

Note: Refer to the section 17 "Entering text" for details of Entering text.

Note: Network settings such as a Network name (SSID) and a security mode depend on each network. Set up the network settings by following your network environment.

Note: If WEP is selected as security mode, a password is 5, 10, 13, or 26 characters. For a security mode besides WEP, a password is between 8 and 64 characters.

## 21.2. How PLEXTALK connects to a network

PLEXTALK automatically tries to connect to the network as long as "Enable or disable network device" in the menu is "Enable".

Note: Refer to 21.1.1. "Enable wireless network device" to enable or disable the network device.

PLEXTALK first tries to connect to the same network where it was connected the last time. If it has failed to connect, PLEXTALK tries to connect to another network in the order of priority set in PLEXTALK. Refer to 21.3.5 "Change or delete wireless network setting" for more detail about priority.

After the network function is not used for more than five minutes without AC adaptor power, it may take a few seconds to a few minutes at most for PLEXTALK to resume the network function. This is because PLEXTALK turned the network function into power saving mode and tries to reconnect to the network.

## 21.3. Advanced Network settings

### 21.3.1. Confirming network information

Information about current network connection can be confirmed by this menu. The following information is available:

•Access point

•Signal strength

•DHCP

•IP address

•Subnet mask

•Default gateway

•Primary DNS address

•Secondary DNS address

•MAC address of wireless network

•Leased date of IP address

•Expire date of leased IP adress

•Address of DHCP server

If PLEXTALK is not linked to network, the following information is available.

•Link status

•MAC address of wireless network

To confirm the above information, do the following:

•Press the Menu key.

•Use the Left or Right key to go to "System settings" option and press the Enter key.

•Use the Left or Right key to go to "Network settings" option and press the Enter key.

•Use the Left or Right key to go to "Confirm network information" option and press the Enter key.

•PLEXTALK announces information. Use the Left or Right key to select the information you want to listen to.

### 21.3.2. Loading network setting

PLEXTALK can configure multiple network settings at once by loading a network settings file, which is prepared on your personal computer. Application software writes the settings in a file called "NetworkSetting.ini". Place this file on to the root of the SD card or USB device, then load the settings using the "Load network setting" menu.

•Press the Menu key.

•Use the Left or Right key to go to "System settings" option and press the Enter key.

•Use the Left or Right key to go to "Network settings" option and press the Enter key.

•Use the Left or Right key to go to "Load network setting" option and press the Enter key.

•PLEXTALK will ask you whether to proceed in loading the file. Press the Enter key to proceed or the Cancel key to cancel.

Note: All old settings are deleted when new network settings are loaded.

Note: If SD card and USB device are connected to the PLEXTALK and both contain "NetworkSetting.ini" respectively, the file in the SD card is loaded.

Note: You can generate the network settings file by the "PLEXTALK Transfer" software. This software is contained in the attached CD.

### 21.3.3. Saving the network setting

Network settings in the PLEXTALK can be saved to the SD card or USB device for backup. To save settings, do the following:

•Connect SD card or USB memory to the PLEXTALK.

•Press the Menu key.

•Use the Left or Right key to go to "System settings" option and press the Enter key.

•Use the Left or Right key to go to "Network settings" option and press the Enter key.

•Use the Left or Right key to go to "Save network settings" and press the Enter key.

•PLEXTALK will ask you to confirm saving the setting file or not. Press the Enter key to proceed. The file is saved to the root of the SD card or USB device. To cancel, press the Cancel key.

Note: If both SD card and USB device are connected, the setting file is saved to the root of the SD card.

Note: If neither SD card nor USB device is connected, the setting file is saved to the root of Internal Memory.

### 21.3.4. Adding wireless network setting

To add the wireless network setting, do the following:

•Press the Menu key.

•Use the Left or Right key to go to "System settings" option and press the Enter key.

•Use the Left or Right key to go to "Network settings" option and press the Enter key.

•Use the Left or Right key to go to "Wireless network settings" option and press the Enter key.

•Use the Left or Right key to go to "Add wireless network setting" option and press the Enter key.

•Enter the SSID with Numeric keys. After you have finished, press the Down key to confirm.

•Select "Security mode". Steps for setting are the same as "Available wireless network".  Refer to the section 21.1.2. "Confirm available wireless network and connect".

•After all settings are completed, the added wireless network is registered as the highest priority network. Then the menu is closed and PLEXTALK will try to connect to the added network.

### 21.3.5. Change or delete wireless network setting

Following three options are available in this menu.

•Change the priority and automatic connection settings

•Change wireless network setting

•Delete wireless network setting

To change or delete wireless network setting, do the following:

•Press the Menu key.

•Use the Left or Right key to go to "System settings" option and press the Enter key.

•Use the Left or Right key to go to "Network settings" option and press the Enter key.

•Use the Left or Right key to go to "Wireless network settings" option and press the Enter key.

•Use the Left or Right key to go to "Change or delete wireless network setting" option and press the Enter key.

•Use the Left or Right key to select the wireless network whose setting you want to change and press the Enter key. The wireless network is listed in the order of priority of the connection.

•Use the Left or Right key to select "Change the priority and automatic connection setting", "Change wireless network setting" or "Delete wireless network setting" and press the Enter key.

If you select "Change the priority and automatic connection setting":

•Use the Left or Right key to select "Connect automatically at highest priority" or "Do not connect automatically" and press the Enter key.

•PLEXTALK will ask whether to proceed or not. Press the Enter key to proceed or the Cancel key to abort.

•If you select "Connect automatically at highest priority", the Menu is closed and PLEXTALK tries to connect to this network at highest priority.

•If you select "Do not connect automatically", PLEXTALK will not connect to this network automatically from this point.

•PLEXTALK closes the Menu and tries to connect to a network in order of the priority set in PLEXTALK.

If you select "Change wireless network setting":

•Select "Security mode". Steps for setting are the same as "Available wireless network". Refer to the section 21.1.2. "Confirm available wireless network and connect."

•After all settings are completed, PLEXTALK stores the updated settings, then closes the menu.

If you select "Delete wireless network setting":

•PLEXTALK will ask you whether to proceed for delete, then press the Enter key to delete. If the current setting is deleted, PLEXTALK will reconnect to the highest priority network.

Note: If you want to change SSID, add new configuration after delete current configuration.

### 21.3.6. Network folder setting

You can configure the Network device name, User ID and password. Default values are:

•Network device name: Linio-(Last 4 digits of serial number), for example "Linio-1234"

•User ID: No user ID

•Password: No password

To change the settings of network folder, do the following:

•Press the Menu key.

•Use the Left or Right key to go to the "System settings" option and press the Enter key.

•Use the Left or Right key to go to the "Network settings" option and press the Enter key.

•Use the Left or Right key to go to the "Network folder setting" option and press the Enter key.

•PLEXTALK announces the network devise name, user ID, and password. PLEXTALK will ask you whether you would like to change them. Press the Down key if you would like to change any one of them.

•PLEXTALK will announce the network device name. If you accept the current name, just press the Down key. If you give it a new name, use the Number pad to enter the new name. Enter the new name within 15 characters using 0-9, a-z, A-Z, and "-" (dash). At least one alphabet should be contained. After you have finished, press the Down key to confirm.

•PLEXTALK will announce user ID. If you accept the current user ID, just press the Down key. If you give it a new user ID, enter the new user ID using the Number pad. Enter the user ID within 32 characters using 0-9, a-z, A-Z,  "-" (dash) and "_" (line). After you have finished, press the Down key to confirm. If you delete the present user ID and press the Down key, menu will be closed after setting the default user ID and password.

•PLEXTALK will announce the password. If you accept the current password, just press the Down key. If you give it a new password, enter the new password up to 32 characters using the Number pad. After you have finished, press the Down key to confirm. The setting will be completed and PLEXTALK will exit the menu system.

Note: Refer to the section 17 "Entering text" for details of entering text.

### 21.3.7. Initializing network settings to default

The network settings can be reset to factory default. The execution of the menu brings following result:

•Enable or disable wireless network device: Enable

•Wireless network settings: deleted

•Network folder settings: default

To reset the network settings to default, do the following:

•Press the Menu key.

•Use the Left or Right key to go to "System settings" option and press the Enter key.

•Use the Left or Right key to go to "Network settings" option and press the Enter key.

•Use the Left or Right key to go to "Initialize all network settings to default" option and press the Enter key.

•PLEXTALK will ask you to confirm that you wish to reset the network setting. Press the Enter key to reset or the Cancel key to cancel.

# 22. Podcast receiver

Podcast is on demand technology that allows you to download audio files from various web sites such as music providers, news networks, or talk radio. After adding the list of your favorite Podcasts to PLEXTALK, you can download the updated audio files in the series. Audio files delivered from Podcast are hereinafter referred to as "Episode".

Note:  Only audio file Episodes can be downloaded to PLEXTALK.

## 22.1. Registering podcast feeds

### 22.1.1. Registering podcast feeds using the "PLEXTALK Transfer" software

To register podcast feeds using the "PLEXTALK Transfer" software, do the following:

•Insert the SD card into your PLEXTALK.

•Make sure that your computer is connected to the Internet.

•If the "PLEXTALK Transfer" software is not currently installed, install it from the attached CD.

•Run the "PLEXTALK Transfer" software.

•After the main screen appears, connect your PLEXTALK to your computer via USB. The "PLEXTALK Transfer" will automatically identify your PLEXTALK.

•Press the Left key or Right key to select the SD card or Internal memory to which you want to register podcast feeds.

•Select "Podcast Feed Manager" on the "Content type" drop down box.

•Select destination list. Go to "File" on the Menu and select "New Podcast Feed".

•Enter the URL of the podcast feed and press "OK" button. The URL should be "rss". The podcast feed is registered and the name of the podcast feed appears in the list box.

•After registering podcast feeds, disconnect your PLEXTALK from your computer and exit the software.

Note: Podcast feeds configurations are stored on the "feed.opml" file in the "PlexPodcast" folder.

Note: The "PLEXTALK Transfer" software can transfer the opml file to the PLEXTALK. Select opml file from source list, then press "Send" button to transfer into the PLEXTALK.

Note: You can also download the "PLEXTALK Transfer" software from the PLEXTALK web site. http://www.plextalk.com/

### 22.1.2. Registering podcast feeds using podcast receiver software

If you are already using podcast receiver software, you can register your favorite podcast feeds to PLEXTALK. Export the Podcast feeds into the opml file. Then, create "PlexPodcast" folder at the root of the SD card or USB device and store the opml file in this folder. You can store multiple opml files.

## 22.2. Listening to Podcasts

After registering podcast feeds, you need to obtain the "list of the episodes" first and then download the "episodes". To download the episodes, it is necessary to connect the PLEXTALK to the Internet.

Note:  Refer to the Section 21, "Network settings", for configuring and enabling network settings.

To download and listen to the episodes, do the following:

•Insert the media in which Podcast feeds are registered.

•Use the Left or Right key to select your favorite podcast feed from "Podcast" category and press the Enter Key.

•If the PLEXTALK will ask you to update the "episodes list", press the Enter key. Then the PLEXTALK starts downloading the latest list of the episodes.

•After obtaining the latest list of the episodes, you can select the episode you prefer. Use the Left or Right key to select your favorite episode and press the Play/Stop key.

•If the Episode has been already downloaded, the playback will start.

•If it has not been downloaded yet, you will be asked if you would like to download it. Press the Enter key to download it. Downloading usually takes longer. After completing the download, press the Play/Stop key to play.

•If it is still in the middle of downloading, the message will be announced.

Note:  If you turn off the power during the download, the download will be cancelled.

Note:  Do not remove the SD card or USB device during downloading. Note: You can check the progress of downloading by pressing the 5 key.

Note:  During downloading the "episodes list", you can cancel the download using the Cancel key.

## 22.3. Navigating by Episode, 10 minutes or 30 seconds

You can select your favorite Episode. Also, you can move to your favorite position of the Episode using the every 10 minutes or every 30 seconds scale.

To navigate by "Episode", "10 minutes" or "30 seconds" option, do the following:

•Use the Up or Down key to select a navigation method among "Episode", "10 minutes", or "30 seconds".

•If selecting "Episode", press the Right key to move forwards to the next Episode. Press the Left key to move backwards to the previous Episode.

•If selecting "10 minutes" or "30 seconds", press the Right key to move to 10 minutes or 30 seconds forwards. Use the Left key to move to 10 minutes or 30 seconds backwards. However, the destination always stays in an Episode.

Note:  Fast-forward and fast-rewind are also limited within an Episode.

Note: You can select a specific episode using the GoTo key and the Number pad.

## 22.4. Obtaining the latest episode list

You can access the website of the Podcast feed and obtain the latest episode list using "Update the episode list" option from the podcast menu.

•Select a podcast feed from which you would like to obtain the latest episode list.

•Press the Menu key.

•Use the Left or Right key to select "Podcast" and press the Enter key.

•Use the Left or Right key to select "Update the episode list", and press the Enter key.

•You will be asked if you would like to update the episode list. Press the Enter key to start updating the episode list. After completing the update, you can select Episode from the latest episode list.

## 22.5. Download all Episodes of the feed

You can download all episodes which are not downloaded yet.

•Select a podcast feed that you would like to download.

•Press the Menu key.

•Use the Left or Right key to select "Podcast" and press the Enter key.

•Use the Left or Right key to select "Download all episodes in the feed", and press the Enter key.

•You will be asked if you would like to download the Episodes. Press the Enter key to start updating the episode list at first. And then all Episodes will be downloaded at a time.

## 22.6. Notice during the download

•Do not remove the SD card or USB device during downloading.

•It takes time to download the episodes. However, you can play other content while downloading the episodes. Select other title by the Title key.

•To check the download progress, press the 5 key. You can confirm the number of Episodes left to download.

•If you turn off the power during the download, the download will be cancelled. If you want to download the episode again, you need to try again.

# 23. Web radio streaming

You can listen to web radio on various web sites with PLEXTALK. You need to add the URL of your favorite web radio station to the playlist and access the site for streaming.

## 23.1. Adding web radio station to the playlist

To listen to web radio, you have to register web radio stations in advance. First, you need to use your computer and obtain a playlist file, containing the URL of the web radio station, whose extension is "pls", "m3u", or "asx". Then, create the "PlexWebradio" folder at the root of the SD card, USB device or Internal Memory and store the file in this folder. You can add multiple web radio stations in this folder.

Example:

You can find play list files at Internet sites which introduce web radio stations such as "SHOUTcast".  http://www.shoutcast.com

Select the desired radio station and right click to activate the "Save" dialogue. Save it into the "PlexWebradio" folder with your desired station name. Then the "pls" file will be stored in the folder.

## 23.2. Listening to web radio

To listen to web radio, it is necessary to connect the PLEXTALK to the Internet first.

Note:  Refer to the Section 21, "Network settings", for configuring and enabling network settings.

To listen to web radio, do the following:

•Insert the media containing playlist files.

•Press the Left or Right key to select your favorite Web radio station. Press the Play/Stop key to start streaming.

Note: There are no functions of fast-forward, fast-rewind, or level navigation for playing web radio stations.

Note: You cannot add bookmarks for web radio. After listening to other stations or changing media, you cannot resume playback from where you stopped.

Note: Refer to the playable web radio format of Appendix A "Main Specifications".

# 24. Network folder function

By using the network folder function, your personal computer can access the SD card, USB device or Internal Memory in the PLEXTALK via Wireless LAN network as a network shared folder.

CAUTION:  Whenever you erase an SD card, USB device or Internal Memory using a personal computer, make sure you select "FAT 16" or "FAT 32".

Caution: When using network folder function, the PLEXTALK does not notice the events of calendar and the alarms.

## 24.1. Set LAN manager authentication level of windows PC

If you use Windows XP or 2000 on your computer, please skip this section.

•If not currently installed, install the "PLEXTALK Transfer" software from the attached CD.

•Run the PLEXTALK Transfer.

•After the main screen appears, select "Network Configuration" from "Network" menu.

•Select the "Local Security Setting" tab of "PLEXTALK setting utility" dialog box.

•Press the "Start security setting" button. The "Local Security Setting" dialog box will appear.

•Select "Send NTLM response only. "Press the OK button.

•You will see a confirmation message box. Press "OK" to confirm.

•You need to restart your computer to make your computer apply this new security setting. The Utility will ask you confirm to restart your computer. Press "Yes" to restart.

## 24.2. Start Network Folder

To start the network folder function, do the following:

•Press the Menu key.

•Use the Left or Right key to go to the "Media Management" option and press the Enter key.

•Use the Left or Right key to go to the "Network folder" option and press the Enter key.

•PLEXTALK connect to the network and network folder is started. PLEXTALK will announce the network device name when it connected. If you not configured network setting yet, configure the network setting before this step.

•By pressing the 5 Key, you can check the Network device name, user ID, password, IP address and network status with the SSID name.

## 24.3. Log on to the Network Folder at your computer

To log on to the network folder, do the following:

•Press the Start menu of your desktop. Choose Run. (Shortcut Cmd+R)

•In the Open box, type \\ your device name. For example \\Linio-1234. And then choose OK.

## 24.4. Finish Network Folder

To finish the Network folder function, do the following:

•Press the Cancel key of the PLEXTALK.

•PLEXTALK will ask you to confirm that you wish to finish.

•Press the Enter key to finish the function or the Cancel key to continue.

# 25. DAISY Online

DAISY Online is the library system which lends DAISY books and distributes Service Announcements to users via Internet. These services are called "Online service". There are a lot of "Online service providers" and the services contents differ from each service provider.

To use DAISY Online service, register as a member of the online service first. Then configure the settings for DAISY Online player such as the PLEXTALK. This section describes the Online service setting, method to borrow or return titles and method to listen to "Service Announcements".

## 25.1. Online service setting

To receive DAISY Online service, you have to register as a member of the online service provider. After registration, the online service provider will give you a user ID and a password.

Configure the ID and password into the PLEXTALK after registration. There are two methods for configuration.

•Load the online service setting file. Refer to the chapter 25.1.1. "Loading online service setting" for details.

•Set user ID and password manually if a online service setting file has already been installed in the PLEXTALK. Refer to the chapter 25.1.2. "Confirming and changing online service setting" for details.

Ask the online service provider which method you have to choose.

### 25.1.1. Loading online service setting

Obtaining the method of the online service setting file depends on each online service provider. For example, you may obtain the setting file from the provider's web site. Ask the online service provider how to obtain the online service setting file.

After obtaining the online service setting file, load the file into PLEXTALK.

To load the online service setting file, do the following:

•Store the online service setting file on the root of the SD card or USB memory. The file name must be "OnlineSetting.ini".

•Insert the SD card or USB memory to your PLEXTALK.

•Press the Menu key.

•Use the Left or Right key to select "System settings" and press the Enter key.

•Use the Left or Right key to select "Online service setting" and press the Enter key.

•Use the Left or Right key to select "Load the setting" and press the Enter key.

•PLEXTALK will ask whether you wish to proceed. Press the Enter key to proceed or the Cancel key to abort.

•PLEXTALK loads the online setting file from the SD card or USB memory.  After loading the online setting file, PLEXTALK reboots automatically.

Note: You can register multiple online service setting files in PLEXTALK.

Note: PLEXTALK does not delete or change current online service setting if you load new other online service settings.

Note: If you load the setting file of the same online service provider, the setting values are overwritten with new values.

### 25.1.2. Confirming and changing online service setting

You can register service provider's name, user ID and password in PLEXTALK manually. Do the following:

•Press the Menu key.

•Use the Left or Right key to select "System settings" and press the Enter key.

•Use the Left or Right key to select "Online service setting" and press the Enter key.

•Use the Left or Right key to select "Confirm and change the setting" and press the enter key. PLEXTALK announces the service provider's name, user ID and existence of password.

•Pressing the information key enables you to confirm the service provider's name, user ID, existence of password and the URL of the online service provider.

•Use the Left or Right key to select the online service provider and press the Enter key.

•Use the Left or Right key to select "Change the setting" and press the Enter key.

•To change the name of the online service, use the Number key to enter new name and press the Down key. If you do not change the name, press the Down key immediately.

•To change the user ID, enter new user ID and press the Down key. If you do not change the user ID, press the Down key immediately.

•To change the password, enter new password and press the Down key. If you do not change the password, press the Down key immediately.

•You can select Online playback mode between streaming or download. Press the Left or Right key to select "Play by streaming" or "Play by downloading" and press the Enter key. PLEXTALK closes the Menu and reboots automatically.

Note: Refer to the Section 17, "Entering text" for more detail about entering text.

Note: If you change the user ID or password, you should enter new user ID or password which is provided from the online service provider.

Note: If the service provider does not allow both "steaming" and "download", the online playback mode setting is not available.

Note: If you select "streaming", downloading title will be aborted. However, downloaded files will not be deleted.

Note: If you select "downloading", all the "Issued titles" will be downloaded.

### 25.1.3. Deleting online service setting

If you withdraw the online service provider, delete the setting of the online service provider.

Do the following:

•Press the Menu key.

•Use the Left or Right key to select "System settings" and press the Enter key.

•Use the Left or Right key to select "Online service setting" and press the Enter key.

•Use the Left or Right key to select "Confirm and change setting" and press the Enter key.

•Use the Left or Right key to select the online service provider and press the Enter key.

•Use the Left or Right key to select "Delete the setting" and press the Enter key.

•PLEXTALK will ask whether you wish to proceed. Press the Enter key to proceed or the Cancel key to abort.

•PLEXTALK deletes the setting of the online service provider. Moreover, the Bookshelf and Service Announcements of the online service provider will be deleted. Then PLEXTALK reboots automatically.

### 25.1.4. Announcing by recorded voice or synthetic voice

You can select an announce voice by "recorded voice" or "synthetic voice" when using Bookshelf, Announcement or Online service menu. If you select "Set priority to Audio Label" on a priority basis, PLEXTALK announces "recorded voice" if there is recoded voice. If there is not recorded voice, it announces synthetic voice. If you select "Text Label only", PLEXTALK announces by synthetic voice all guidance. For this setting, do the following:

•Press the Menu key once to access the Main Menu.

•Use the Left or Right key to select "System settings" and press the Enter key.

•Use the Left or Right key to select "Online service setting" and press the Enter key.

•Use the Left or Right key to select "Setting for label playback" and press the Enter key.

•Use the Left or Right key to select "Set priority to Audio Label" or "Text Label only" and press the Enter key.

Note: if you select "Set priority to Audio Label", PLEXTALK take a slightly longer time to announce.

Note: Depending on the Online service provider, they may not provide the "recorded voice" for announcements.

## 25.2. Bookshelf

An available title list will be sent from the online service provider via the Internet. The available title list is called "new titles." Select the title you want to listen to from the "new titles". When you select a title from "new titles" and press the Play/Stop key, the selected title is transferred to the category called "issued titles." "Issue" means that you borrow the title from the online service provider.

"Bookshelf" consists of two categories which are "new titles" and "issued titles." PLEXTALK handles "Bookshelf" as a "media" such as SD card or USB device. If your PLEXTALK has registered multiple online service providers, each Bookshelf appears as an individual media.

Note: Some online service providers automatically transfer titles from "new titles" to "issued titles."

To get titles from the online service provider to Bookshelf, the following methods are available.

•Search and receive titles to Bookshelf using the "Online service menu".

•Ask the online service provider to send the titles in your Bookshelf.

•Search titles on the Web site of the online service provider and receive the titles in your Bookshelf.

Some online service providers do not provide the methods which are shown above. Ask the online service provider in detail how to get titles to your Bookshelf.

### 25.2.1. Playing a title in Bookshelf

To play a title in the Bookshelf, do the following:

•Press the Title key to select a title in the Bookshelf.

•Press the Play/Stop key to play the title.

The operations such as fast forward, fast rewind, heading navigation for the titles in Bookshelf are the same as operating titles on the SD card.

### 25.2.2. Playback options: streaming and downloading

There are two options to play the title, which are streaming and downloading. On streaming, a part of audio of the title is transferred to PLEXTALK and it plays the audio data. To play the title by download, PLEXTALK downloads the entire title then plays it.

Note: Some online service providers provide only streaming or provide only downloading.

Note: When streaming a title, it takes time to start the playback or navigate the title.

Note: While downloading a title, you cannot play the title.

Note: Downloaded titles in Bookshelf are stored in Internal memory. But those files do not appear when you select "Internal Memory" by pressing the Title key.

Note: You can back up a downloaded title to another medium.

### 25.2.3. Downloading a title

To download a title, do the following:

•Press the Title key to select Bookshelf.

•Select a title you want to download in "Issued titles", and press the 9 key.

•PLEXTALK will ask to confirm if you wish to proceed or not.  Press the Enter key to proceed or the Cancel key to abort.

Note: Some online service providers provide only streaming. In this case, downloading is not available.

Note: If you want to download multiple titles, repeat the procedure shown above.

Note: Press the 9 key during downloading to abort the downloading.

Note: Downloading may take a long time. You can play another title while the title is being downloaded.

Note: The time period to download varies depending on network environment and the size of the title.

### 25.2.4. Returning a title

To return a title, do the following:

•Press the Title key to select Bookshelf.

•Select a title you want to return in "Issued titles" category.

•Press the Bookmark key. The title is marked to be returned.

•If you press the Bookmark key again, the mark is cancelled.

•If you want to return other titles, repeat the operation above.

•After selecting the titles you want to return, press the Play/Stop or the Pound/Hash key to return the titles.

•PLEXTALK confirms if you want to return the titles or not. Press the Enter key to return the selected titles. Downloaded data is deleted from the Internal memory when returning titles.

Note: When you return a title, backed-up data is not deleted.

Note: You cannot return a title in "New titles" category. If you want to return, you can return after issued to "Issued titles".

### 25.2.5. Selecting a title by category

If there are many titles in your Bookshelf, switching the category enables you to select titles quickly. To select a title by category, do the following:

•Press and hold the Right key to go to the first title in the next category.

•Press and hold the Left key to go to the first title in the present category.

•Press and hold the Left key once more to go to the first title in the previous category.

•Press the Left or Right key to select your favorite title.

Alternatively, do the following;

•When selecting title, press the Up key to select the level of "category".

•Press the Left or Right key to select the next category or previous category.

•Press the Down key to select the level of "title".

•Press the Left or Right key to select your favorite title.

### 25.2.6. Transferring titles category from "New titles" to "Issued titles"

To transfer category of a title from "New titles" to "Issued titles", do the following:

•Select a title in "New titles" category and press the Play/Stop key. PLEXTALK transfers the category to "Issued titles".

To transfer category of several titles at once from "New titles" to "Issued titles", do the following:

•Select a title in "New titles" category and press the Bookmark key. The title is marked to be transferred category.

•If you press the Bookmark key again, the mark is cancelled.

•If you want to transfer category of other new titles, select new titles and press the Bookmark key.

•After selecting titles, press the Play/Stop key or the Pound/Hash key.

•PLEXTALK will confirm if you want to transfer the selected titles or not. Press the Enter key to transfer titles to "issued titles".

Note: If you press the Bookmark key while selecting a title in "issued title" category, PLEXTALK will return the title.

## 25.3. Service Announcements

"Service Announcement" is the information distributed by each online service provider. You can listen to the "Service Announcement" by selecting "Service Announcement" media. The "Service Announcement" media is not separated for each online service provider such as the Bookshelf.

Note: Some online service providers do not distribute a "Service Announcement". In this case, PLEXTALK does not announce the "Service Announcement" when selecting titles.

Note: The "Service Announcement" is updated automatically after turning on the power and after a certain period of time.

### 25.3.1. Listening to Service Announcement

To listen to the "Service Announcement", do the following:

•Use the Title key to select the "Service Announcement" media. PLEXTALK reads the first Service Announcement.

•Press the Left or Right key to listen to the previous or next Service Announcement.

If your PLEXTALK had registered several online service providers, the Service Announcements are sorted by each online service provider.

•Press and hold the Left or Right key to listen to the Service Announcement of another service provider.

You can switch online service providers in the following method.

•You can select online service provider by pressing the Up key while selecting "Service Announcement".

•Use the Left or Right key to switch online service providers.

•Press the Down key. Then you can select each Service Announcement.

•Use the Left or Right key to select a Service Announcement you wish to listen to.

### 25.3.2. Deleting Service Announcements

After listening to a Service Announcement, the Service Announcement can be deleted. To delete Service Announcements at once, do the following:

•Press the Bookmark key to select the Service Announcement you want to delete. The Service Announcement is marked to delete.

•Press the Bookmark key once more to cancel marking to delete.

•To mark several Service Announcements to delete, repeat the same operation above.

•After selecting the Service Announcements you want to delete, press the Play/Stop key or the Pound/Hash key.

•PLEXTALK will confirm if you want to delete or not. Press the Enter key to delete the Service Announcements.

Note: Once you delete a Service Announcement, you cannot listen to it again.

## 25.4. Bookshelf and Announcement menu

### 25.4.1. Updating all bookshelves

You can update all the bookshelves that are registered in PLEXTALK manually.

Do the following:

•Press the Menu key.

•Use the Left or Right key to select "Bookshelf and announcement" and press the Enter key.

•Use the Left or Right key to select "Update Bookshelf" and press the Enter key.

•PLEXTALK will ask whether you wish to proceed. Press the Enter key to proceed or the Cancel key to abort.

Note: PLEXTALK updates the Bookshelf automatically after turning on the power and after a certain period of time.

### 25.4.2. Selecting online service and updating Bookshelf

To update a specified online service provider's Bookshelf, do the following:

•Press the Menu key.

•Use the Left or Right key to select "Bookshelf and announcement" and press the Enter key.

•Use the Left or Right key to select "Select online service and update Bookshelf" and press the Enter key.

•Use the Left or Right key to select the online service provider and press the Enter key.

•PLEXTALK will ask whether you wish to proceed. Press the Enter key to proceed or the Cancel key to abort.

### 25.4.3. Updating Service Announcements

You can manually update all the Service Announcements that are registered in PLEXTALK. Do the following:

•Press the Menu key.

•Use the Left or Right key to select "Bookshelf and announcement" and press the Enter key.

•Use the Left or Right key to select "Update announcements" and press the Enter key.

•PLEXTALK will ask whether you wish to proceed. Press the Enter key to proceed or the Cancel key to abort.

Note: PLEXTALK updates the Service Announcement automatically after turning on the power and after a certain period of time.

## 25.5. Online service menu

Some online service providers supply services such as searching titles. To use the service, you need to select the "online service" option at the top menu of PLEXTALK. Available services on the menu differ from each service provider and the operation of the menu differs accordingly. Therefore this section explains generally how to operate the menu. Contact the service provider to learn the details of the service.

Note: If the service provider does not provide the online service menu, this menu is not available.

To use the online service menu, do the following:

•Press the Menu key.

•Use the Left or Right key to select "Online service" menu and press the Enter key.

•Use the Left or Right key to select the online service provider and press the Enter key PLEXTALK starts the online service menu.

This menu provides interactive communication between the user and service provider. You need to respond to the request from the service provider. Types of requests are "multiple choice" and "text entry".

•To respond to the "multiple choice" request, use the Left or Right key to select your choice.

•To respond the "text entry" request, enter text with the Number keys.

•If there are several requests, press the Down key to proceed for next request or press the Up key to return to previous request.

Note: Some service providers do not allow you to return to the previous request.

Note: Refer to chapter 17, "Entering text" for more detail about entering text.

After answering a series of requests, press the Down key to submit your response. At the end of the interactive communication, the online service provides a result. If the online service provides a "title list" as a result of the interactive communication, select the title you wish to borrow. To select a title, do the following:

•Use the Left or Right key to select a title.

•Press the Down key to borrow the title.

•PLEXTALK will ask to confirm if you wish to proceed or not. Press the Enter key to proceed or the Cancel key to abort. If you press the Enter key, the selected title is added to your Bookshelf and PLEXTALK plays the title by streaming or downloads the title.

To borrow several titles at once, do the following:

•Use the Left or Right key to select a title and press the Bookmark key. The title is marked to be issued.

•Press the Bookmark key again to cancel the mark.

•To issue several titles, repeat the operation above.

•After selecting titles, press the Down key or Pound/Hash key to issue the titles.

•PLEXTALK will ask to confirm if you wish to proceed or not. Press the Enter key to proceed or the Cancel key to abort. If you press the Enter key, the selected title is added to your Bookshelf and PLEXTALK plays the last selected title by streaming or downloads each title.

Note: To preview the title before issuing, press the Play/Stop key.

Note: There are titles which do not allow preview.

Note: To quit from the online service menu, press the Menu key to close the whole menu.

# 26. Using PLEXTALK as an external drive

You can connect PLEXTALK to a personal computer, using the USB (Universal Serial Bus) 2.0 interface, to use it as an external card drive. This chapter describes how to connect and disconnect PLEXTALK to computers that have a range of operating systems.

Note:  To use PLEXTALK as an external memory card drive, your personal computer's operating system must be Windows 2000, XP, Vista, Windows 7 or Windows 8.

Caution: When connecting the PLEXTALK to the personal computer via USB cable, the PLEXTALK does not notice the events of calendar and the alarms.

## 26.1. What can PLEXTALK be used for when connected to a personal computer?

With PLEXTALK connected to a computer as a card drive, you can perform standard file management functions such as:

•Transfer DAISY titles, Audio Files and Text Files between the PLEXTALK and the computer.

•Create, delete and rename folders and files on the SD card or Internal Memory of the PLEXTALK.

•Transfer audio files from a computer to an SD card or internal memory, and listen to them using the computer, in other words, use PLEXTALK as a file store for music files.

## 26.2. Selecting SD card or Internal Memory as an external drive

Before connecting PLEXTALK to a computer, you can select the SD card or Internal Memory as an external drive. To select SD card or Internal Memory, do the following:

•Press the Menu key.

•Use the Left or Right key to select "System settings" and press the Enter key.

•Use the Left or Right key to select "USB client mode" and press the Enter key.

•Use the Left or Right key to select "Internal Memory" or "SD card" and press the Enter key.

## 26.3. Connecting PLEXTALK to a computer

To connect PLEXTALK to a computer, use the following procedure:

•Turn on your computer system.

•Connect the AC adaptor to the PLEXTALK and turn it on.

•Connect the USB cable (long) from PLEXTALK to the computer directly.

•The computer recognizes PLEXTALK as an external memory card drive automatically.

•You can switch the external drive between SD card and Internal Memory by pressing the Left or Right key while connecting.

CAUTION:  When PLEXTALK is connected to a computer as an external memory card drive, you should always avoid switching off PLEXTALK, unplugging the USB cable or removing the SD card. These operations may result in damage to or loss of data.

CAUTION:  Whenever you erase an SD card or Internal Memory using a personal computer, make sure you select "FAT 16" or "FAT 32".

Note:  As soon as PLEXTALK recognizes the connection to the computer, it becomes an external memory card drive for the computer and loses all its player and recorder functionality.

## 26.4. Disconnecting PLEXTALK from a computer

When disconnecting PLEXTALK from a computer, use the procedure below:

•Left click the "USB" or "Safely remove hardware" icon located in the System Tray at the lower right of the computer screen.

•Choose this drive at the Safely Remove Hardware option in the System Tray.

Note:  If you have difficulty finding or clicking the icon in the Task Tray, shut down the computer and PLEXTALK, and then disconnect the cable.

# 27. Other useful features

## 27.1. Accessing the built in help file

To access the built in help file, do the following:

•Press the Number 0 key once.

•Navigate the file in the same way that you navigate other DAISY text titles.

To exit the help file, do the following:

•Press the Number 0 key once.

## 27.2. Checking system properties

To find out the system version or the serial number of your PLEXTALK and PDTB key information, do the following:

•Press the Menu key once to access the Main Menu.

•Use the Left or Right key to go to the "System settings" option and select it by pressing the Enter key.

•Use the Left or Right key to go to the "System properties" option and select it by pressing the Enter key.

•The system version and serial number will be played repeatedly. Also, the PDTB key information will be played only if you have installed the key.

•When you have noted the information that you require, press the Enter key to exit the menu system.

## 27.3. "Skippable" settings

Some DAISY books have items that you are able to skip. PLEXTALK enables you to select whether you skip the item or not. This feature is called "Skippable."  Skippable items are as follows:

•Page number

•Footnote

•Note reference

•Annotation

•Line number

•Sidebar

•Producer note

To set the "Skippable" settings, do the following:

•Press the Menu key.

•Use the Left or Right key to select "Playback settings" and press the Enter key.

•Use the Left or Right key to select "Skippable" and press the Enter key.

•Use the Left or Right key to select "Skip" or "Play" for each Skippable item and press the Enter key to confirm. After deciding all the Skippable items to be skipped or not, PLEXTALK will confirm that your selections have been set.

## 27.4. Network update

Network update is the firmware update of PLEXTALK via Internet. If a later firmware exists when the power is turned on, PLEXTALK asks if you wish to update the firmware or not. Press the Enter key if you want to update and updating starts. Press the Cancel key if you want to update later.

To update later, do the following:

•If the PLEXTALK is not connected to the network, connect it to the network.

•Press the Menu key.

•Use the Left or Right key to select "System settings" and press the Enter key.

•Use the Left or Right key to select "Network update" and press the Enter key.

•When there are no available latest updates, PLEXTALK announces the confirmation message. Then press the Enter key to exit the menu.

•When there are some updates, use the Left or Right key to select the update that you would like to apply and press the Enter key. Downloading of the update starts.

•After downloading is completed, the update process starts automatically. Then PLEXTALK restarts at the end of the update process.

## 27.5. Deleting authorization keys

To delete all authorization keys that have been registered on PLEXTALK, do the following:

•Press the Menu key.

•Use the Left or Right key to select "System settings" and press the Enter key.

•Use the Left or Right key to select "Delete authorization keys" and press the Enter key.

•PLEXTALK will ask you to confirm that you wish to continue with the process. Press [1234] by number keys and press the Enter key to delete the keys.

Note: You need to press 1, 2, 3, and 4 in this exact order.

Note: "Authorization keys" are keys to play the DAISY books that are protected by PDTB format. PDTB is a digital encryption technology that is formulated by DAISY consortium.

## 27.6. Reset PLEXTALK to factory defaults

To reset all settings back to the factory defaults, do the following:

•Press the Menu key once to access the Main Menu.

•Use the Left or Right key to select "System settings" and press the Enter key.

•Use the Left or Right key to select "Initialize all settings to default" and press the Enter key.

•PLEXTALK will ask you to confirm that you wish to proceed. Press the Enter key to set all settings to factory defaults or the Cancel key to abort. After a short while, PLEXTALK will restart.

The following is a list of the default settings for the PLEXTALK.

•Select recording mode:  Default

•Select auto section setting:  Off

•Select time to auto stop recording when no sound:  Off

•Select trigger of starting recording:  When record key is pressed.

•Select background noise level:  Normal

•Turn guidance on or off when recording:  Guidance on

•Select external audio input jack:  Microphone mode

•Auto Gain Control:  On

•Shuffle and repeat setting:  Normal playback

•Reading audio file name: Read audio file name

•Multimedia DAISY playback setting: Auto

•Skippable: Play

•Playback setting for Voice Bookmark:  Voice Bookmark on

•Select the waiting sound:  Waiting sound 1

•Select the key press confirmation sound: Key press confirmation sound on

•USB client mode: Internal memory

•Set the text to speech pitch:  Normal

•Recording volume:  10

•Playback speed:  Normal

•Playback tone:  Normal

•Guidance volume:  Normal

•Playback volume:  13

•Bookmarks: deleted

•Events on Calendar: deleted

•Alarms: cleared

Note: "Alarm" folder in the Internal memory will not be deleted.

Note: Network settings are not deleted by initializing all settings to default. Refer to the section 21.3.7 "Initializing network settings to default"

# Appendix A: Main Specifications

Note:  Shinano Kenshi reserves the right to modify the products specification at any time without prior notice.

## General specifications

SD card slot:

SD/SDHC card

Playable contents:

Books conforming to DAISY 2.0, 2.02 and 3.0 standards.  DAISY PDTB and PDTB2.

Audio files in MP3, unprotected WMA, AMR-WB+, Ogg Vorbis, Wave (PCM) formats, Audible Format 4(.aa) and LKF.

Music CDs (CD-DA) (External USB CD/DVD drive)

Text file: plain text, html, Microsoft Word .doc, .docx, EPUB2 and PDF

Web radio:

Supported playlist file type: PLS, M3U, ASX

Playable Audio format: MP3, unprotected WMA

Streaming format (protocol): HTTP, mms

Recording methods:

DAISY 2.02 and 3.0

Recording audio formats for DAISY:

PCM 44.1kHz Stereo

PCM 22.05kHz Mono

MP3 256kbps Stereo

MP3 128kbps Stereo

MP3 64kbps Mono

MP3 32kbps Mono

Recording audio formats for Voice Memo:

MP3 32kbps Mono

Audio input and output:

Headphone terminal (Stereo)

External microphone/ Line input terminal (Stereo)

Built in microphone (Monaural)

Built in speaker (Monaural)

Externally connected terminal:

USB 2.0

USB OTG (USB bus power)

Internal Memory:

7.2 GB

File system:

FAT-32/16

Clock precision:

Approximately 60 seconds per month

Electric Power Source:

AC adaptor, AC 100 V to 240 V, 50 or 60 Hz

Dimensions and weight:

Length 112 mm

Width 55 mm

Height 16 mm

Weight Approx. 110g

Power consumption (maximum):

20W

Working temperature and humidity:

Fahrenheit from 40 to 104

Celsius from 5 to 40

Humidity from 20% to 80%, no moisture.

## Audio features

Headphones (line output):

Compatible impedance 32 ohm unbalanced

Jack 3.5mm diameter stereo mini jack

External Microphone (Plug-in power):

Input impedance 6.8k ohm unbalanced

Jack 3.5mm diameter stereo mini jack

Line Input:

Input impedance 10k ohm unbalanced

Jack 3.5mm diameter stereo mini jack

Built In Speaker:

Output impedance 8 ohm

Output 150mW

Build-in Microphone:

Mode Electric condenser microphone

## Main functions

Guide voice:

Gives spoken information about the current operation.

Provides key descriptions in Key Describer mode.

Speaks menu items.

Gives help information (press the Number 5 key).

Search function:

Heading, Group, Page, Phrase, Bookmark, Album, Track, Level, Screen, Paragraph, Line, Sentence, Spell, Word, Character, Percent, Keyword

Volume adjustment:

0 to 25 (in increments of 1)

Guide volume adjustment:

-5 to +5 (in increments of 1)

Play speed adjustment:

12 settings from -3, through 0, to +8 (in increments of 1).

Normal speed is at 0.

-3 is half speed, +4 is double speed, +8 triple speed.

Tone adjustment:

13 settings from -6, through 0, to +6 (in increments of 1)

Normal tone is at 0

Recording time (when using 2 gigabytes SD card):

(Note:  All times are approximate)

DAISY PCM 44.1kHz stereo 3 hours

DAISY PCM 22.05kHz mono 12 hours

DAISY MP3 256kbps stereo 15 hours

DAISY MP3 128kbps stereo 30 hours

DAISY MP3 64kbps mono 60 hours

DAISY MP3 32kbps mono 120 hours

Recording level adjustment:

Select input between External Microphone and Line input

0 to 20 (in increments of 1)

Available Auto Gain Control

Memory function:

Bookmarks and voice Bookmarks, maximum is 10000

Voice Bookmarks, Total 30 minutes recording

Voice Memo, Total 30 minutes recording, 200 items

## Battery specifications

Type:  Lithium Ion

Dimensions:

Length 53 mm

Width 38 mm

Height 9 mm

Capacity:  1430 milli-ampere hour

Charging time:

4 hours approximately via an AC adaptor

4.5 hours approximately via a USB cable

Note: The charging via a USB cable works during power off.

Operating time:

DAISY format, MP3 recording, 8 hours

DAISY format, MP3 playing, 10 hours

Plain text, playing, 7 hours

Note:  All times are approximate.

Note:  The charging time and operating time vary depending on the ambient temperature and working conditions.

## Memory Card Drive of a personal computer via USB cable

Type:  SD/SDHC card or Internal memory

Operating systems:  Windows 2000, XP, Vista, Windows 7 or Windows 8

## Wireless communication function

Wireless LAN: IEEE802.11b/g

Available security mode: WEP, WPA-PSK(TKIP), WPA-PSK(AES), WPA2-PSK(TKIP), WPA2-PSK(AES)

# Appendix B: Menu Hierarchy

This appendix lists all menu items in the Main Menu along with their corresponding sub menu hierarchy.

ONLINE SERVICE

EDIT (DAISY):

Set heading

Cancel heading

Remove section

Exit DAISY editing mode

EDIT (text file):

Delete the current line

Delete all text in the file

Save the edited text

Save and exit text editing

Exit text editing without saving

Rename the file

RECORDING SETTINGS:

Select recording mode

Default

Conference

Audio

Analog cassettes

Radio

Custom

Then select sound quality from the following

PCM 44.1kHz stereo

PCM 22.05kHz mono

MP3 256kbps 44.1kHz stereo

MP3 128kbps 44.1kHz stereo

MP3 64kbps 44.1kHz mono

MP3 32kbps 22.05kHz mono.

Turn guidance on or off when recording

Guidance on

Guidance off

Select external audio input jack

Microphone mode

Line in mode

Auto gain control

Auto gain control on

Auto gain control off

PLAYBACK SETTINGS:

Shuffle and repeat settings

Normal playback

Repeat title (DAISY, Recordings, Text, Audio book)

Repeat track (Music)

Repeat album (Music)

Repeat entire albums (Music)

Shuffle repeat (Music)

Reading audio file name

Read audio file name

Read audio track number

Multimedia DAISY playback setting

Auto

Audio

Text

Skippable

("Play" or "Skip" for each skippable item.)

Select page number

Select footnote

Select note reference

Select annotation

Select line number

Select sidebar

Select producer note

BOOKMARK:

Playback setting for Voice Bookmark

Voice Bookmark on

Voice Bookmark off

Clear all bookmarks in current title

VOICE MEMO:

Delete the selected voice memo

Delete all voice memos

PODCAST:

Update the episode list

Download all episodes in the feed

CALENDAR:

Delete all events

Delete the events of more than three months ago

ALARM:

Select On or Off

Change the setting

Clear alarm setting

MEDIA MANAGEMENT:

Media and title information

Delete all files on the SD card

Delete all files in the USB device

Delete all files in Internal Memory

Create a new title

Create a new text file

Delete the selected title,  (Delete the selected album, Delete all Voice Memos)

Delete a file

Prepare for editing

Build book

Network folder

SYSTEM SETTINGS:

System properties

System version

Serial number

Installed key (if obtained.)

Select the text to speech voice

Voice 1

Voice 2

Select the waiting sound

Waiting sound 1

Waiting sound 2

No waiting sound

Select the key press confirmation sound

Key press confirmation sound on

Key press confirmation sound off

Set the system date and time

USB client mode

Internal memory

SD card

Set the text to speech pitch

Network settings

Confirm network information

Load network settings

Save network settings

Enable or disable wireless network device

Wireless network settings

Available wireless network

Add wireless network setting

Change or delete wireless network setting

Change the priority and automatic connection settings

Change wireless network setting

Delete wireless network setting

Network folder setting

Initialize all network settings to default

Online Service setting

Load the setting

Confirm and change the setting

Change the setting

Delete the setting

Setting for label playback

Set priority to Audio Label

Text Label only

Network update

Delete authorization keys

Initialize all settings to default

BOOKSHELF and ANNOUNCEMENT:

Update Bookshelf

Select online service and update Bookshelf

Update Announcements

# Appendix C: Troubleshooting

## General operating problems

Symptom:  You turn on the Power Switch, but the power does not come on.

Possible cause 1:  Loose power cord or AC adaptor.

Solution:  Firmly reinsert the mains power cable into PLEXTALK and ensure that the AC adaptor is firmly inserted.

Possible cause 2:  Key Lock is On.

Solution:  Turn off the key lock.

Symptom:  The power is on, but PLEXTALK does not operate.

Possible cause:  Key Lock is On.

Solution:  Turn off the key lock.

Symptom:  PLEXTALK will not shut down and no keys can be operated.

Possible cause 1:  Key Lock is On.

Solution:  Turn off the key lock.

Possible cause 2:  Unknown error.

Solution:  Turn the Key Lock on and then press and hold down the Power key for more than 10 seconds.   If this does not work, remove the AC Adaptor and Battery from the unit and then reinsert them.

Symptom:  Cannot record to an SD card.

Possible cause 1:  Fault with the input.

Solution:  Check the input source and then correct the input setting.

Possible cause 2:  There is no available space on the SD card.

Solution:  Either delete some unnecessary contents from the current SD card or insert a different SD card with sufficient space.

Possible cause 3:  The SD card is locked.

Solution:  Remove the SD card in the correct way, switch the lock to the "Unlocked" position and reinsert it into PLEXTALK.

Symptom:  The recording level cannot be adjusted.

Possible cause:  You have the Auto Gain Control turned on.

Solution:  Turn the Auto Gain Control off in the "Recording settings" menu.

Symptom:  There is no voice guidance.

Possible cause 1:  The volume has been set to '0'.

Solution:  Increase the volume so that the guide speech can be heard.

Possible cause 2:  The Turn guidance on or off when recording" option is set to "Guidance off".

Solution:  Change the setting to "Guidance on".

## Spoken errors during DAISY playback

Guide voice:  "Unsupported SD card" or "Unsupported USB device."

Possible cause:  The SD card or USB media has been formatted with an unsupported file system.

Solution:  Insert an SD card or USB media which is formatted as FAT-16 or FAT-32.

Guide voice:  "Read error."

Possible cause:  Title cannot be played back.

Solution:  Press the Title key followed by the Left or Right key to try to select a different title.

## Errors using the Voice Memo

Guide voice:  "Not enough space in the voice memo area."

Possible cause:  Not enough internal memory for recording a new Voice Memo.

Solution:  Free up some memory by deleting unnecessary Voice Memos. You may also wish to backup your memos before carrying out this process.

## Errors when using PLEXTALK as an SD card drive on a computer

Symptom:  Drive does not work.

Possible cause 1:  The USB cable is not connected correctly.

Solution:  Reinsert the USB cable. Check the cable, the connector and whether the pins are dented, bent or damaged.

Possible cause 2:  The power is not on.

Solution:  Check that the power cord and AC adaptor are securely connected and that PLEXTALK itself is turned on.

Symptom:  Cannot write to the SD card.

Possible cause 1:  The SD card is locked.

Solution:  Eject the SD card, move the lock to the unlocked position, reinsert it and try again.

Possible cause 2:  The power is not on.

Solution:  Check that the Power cord and AC adaptor are securely connected, and turn on the Power Switch.

Symptom:  Nothing can be added to the SD card.

Possible cause 1:  There is not enough space on the SD card for added data.

Solution:  Free up some space on the SD card and try again.

Possible cause 2:  The SD card is locked.

Solution:  Eject the SD card, move the lock to the unlocked position, reinsert it and try again.

# Appendix D: Technical data about recording modes and sound qualities

## 1. Recording mode settings

Each recording mode has predetermined settings for the following 4 parameters:

•Auto section setting; PLEXTALK can be set to automatically create a new section, in a DAISY recording, after a specified amount of time with no audio input, for example 2 seconds. Use the setting "off" to create a continuous recording with no new sections.

•Time to auto stop recording when no sound; This is the amount of time that PLEXTALK will continue to record with no audio input. After this time, recording stops automatically.

•Trigger of starting recording; Recording can be set to start either when you press the Record Key or when input audio is detected.

•Background noise level of input sound; If you are recording in a low noise environment, such as a studio, use the "low" setting. If your recording environment is noisy, such as a conference or lecture, use the "High" setting.

## 2. Settings for preset recording modes

The preset parameters for the 5 recording modes are as follows:

Default mode:

•Select auto section setting:  Off

•Select time to auto stop recording when no sound:  Off

•Select trigger of starting recording:  When record key is pressed.

•Select background noise level of input sound:  Normal (-28dB)

Conference mode:

•Select auto section setting:  Off

•Select time to auto stop recording when no sound:  Off

•Select trigger of starting recording:  When record key is pressed.

•Select background noise level of input sound:  High (-20 dB)

Audio mode:

•Select auto section setting:  2 seconds

•Select time to auto stop recording when no sound:  1 minute

•Select trigger of starting recording:  When record key is pressed.

•Select background noise level of input sound:  Low (-40 dB)

Analogue Cassette mode:

•Select auto section setting:  4 seconds

•Select time to auto stop recording when no sound:  1 minute

•Select trigger of starting recording:  When sound is detected.

•Select background noise level of input sound:  Normal (-28 dB)

Radio mode:

•Select auto section setting:  Off

•Select time to auto stop recording when no sound:  1 minute

•Select trigger of starting recording:  When sound is detected.

•Select background noise level of input sound:  Normal (-28 dB)

## 3. Available settings for Custom mode

In Custom mode you can set each parameter to the settings listed below:

•"Select auto section setting" can be set to:  "Off", "1 second", "2 seconds", "3 seconds", "4 seconds" or "5 seconds".

•"Select time to auto stop recording when no sound" can be set to:  "Off", "30 seconds", "1 minute", "2 minutes", "3 minutes", "4 minutes" or "5 minutes".

•"Select trigger of starting recording" can be set to either "When record key is pressed" or "When sound is detected".

•"Select background noise level of input sound" can be set to:  "Normal (-28 dB)", "Low (-40 dB)" or "High (-20 dB)".

Note:  The settings for the Default, Conference, Audio, Analogue cassette and Radio modes cannot be changed. The settings for the Custom mode can be changed to suit your requirements.

Note:  When recording with the built in microphone, the "Background noise" setting will automatically be set to "High".

## 4. Available recording time for the specified sound qualities

The available recording time will depend on the sound quality setting you select. The higher the sound quality, the less recording time. Below are listed the approximate recording times you can expect to get with 4 media types.

PCM 44.1kHz stereo:

•SD card, 1GB:  90 minutes

•SD card, 2GB:  3 hours

•SD card, 4GB:  6 hours

•SD card, 8GB:  12 hours

PCM 22.05kHz mono:

•SD card, 1GB:  6 hours

•SD card, 2GB:  12 hours

•SD card, 4GB:  24 hours

•SD card, 8GB:  48 hours

MP3 256 kbps stereo:

•SD card, 1GB:  7.5 hours

•SD card, 2GB:  15 hours

•SD card, 4GB:  30 hours

•SD card, 8GB:  60 hours

MP3 128 kbps stereo:

•SD card, 1GB:  15 hours

•SD card, 2GB:  30 hours

•SD card, 4GB:  60 hours

•SD card, 8GB:  120 hours

MP3 64 kbps mono:

•SD card, 1GB:  30 hours

•SD card, 2GB:  60 hours

•SD card, 4GB:  120 hours

•SD card, 8GB:  240 hours

MP3 32 kbps mono:

•SD card, 1GB:  60 hours

•SD card, 2GB:  120 hours

•SD card, 4GB:  240 hours

•SD card, 8GB:  480 hours

Note:  The above recording times are approximations and may vary depending on the number of headings and other structure in the recording.

# Appendix E: Questions and Answers on Network setting

| Trouble / Question | Solution / Answer |
| --- | --- |
| I want to know the IP address of PLEXTALK | You can confirm the IP address from "Confirm network information" option of the network setting menu. |
| PLEXTALK does not select your desired access point. | You can confirm list of available access point by wireless network setting option in the network setting. When PLEXTALK cannot find access point, please confirm status of network power has been set ON. If the access point is listed, please confirm the priority of the access point. You can set the highest priority from "Change the priority and automatic connection settings" option of the network setting menu. |
| It failed to find access point. I am using access point that support IEEE802.11a | PLEXTALK only support IEEE802.11b/g. If your access point cannot use IEEE802.11a and IEEE802.11b/g in same time, please select IEEE802.11b/g. |
| It failed to connect access point. I am using non broadcasted SSID. | Non broadcasted SSID require a long time to establish a connection between PLEXTALK. Please allow the PLEXTALK up to 5 minutes to see if it finally succeeds or not. If possible, please try to configure your access point setting to enable broadcasting the SSID name. When it succeeds in connecting after broadcast setting, that means your network configuration is fine, but non-broadcast setting is problem. If it failed even with broadcasted SSID, please confirm wireless security setting. |
| It failed to connect access point. I am using DHCP. | Even if PLEXTALK could establish the wireless connection between access point, but when it failed to retrieve IP address, connection failed message will show up. In this case, please try fixed IP address. If it succeeds in connecting, DHCP server is problem. Please confirm your DHCP server setting. If it still fails to connect, please confirm wireless security setting. |
| It failed to connect access point. I am using MAC address filtering. | When MAC address filtering is enabled in access point security setting, it refuses a connection of un-configured network device. You need to configure your access point to add PLEXTALK MAC address. You can confirm MAC address of PLEXTALK by "Confirm network information" option of the network setting menu. |
| PLEXTALK always tries connection to not available SSID. I have configured Stealth SSID settings also. | When you have configured a network setting with Stealth SSID enabled, PLEXTALK tries connection to stealth (= non broadcasted SSID) always. Please set Stealth SSID access point to low priority in the list of wireless settings. |
| Plextalk cannot connect to the network service. I am using WEP security with Fixed IP address. | When you are using WEP security with fixed IP address setting, PLEXTALK will succeed in connecting to the access point even it has a wrong password. Please confirm WEP password again. |
| Does PLEXTALK support WPS (WIFI Protected Setup?) | WPS is not supported. You need to configure the network setting by yourself. |
| It failed to connect to the access point. | Please try and restart the access point and PLEXTALK. |

# Appendix F: List of entering characters

ABC mode

| Key name | Assignment |
| --- | --- |
| #1 | Symbols “.”, “@”, “/”, “:”, “1” |
| #2 | a, b, c, 2, A, B, C, |
| #3 | d, e, f, 3, D, E, F, |
| #4 | g, h, i, 4, G, H, I, |
| #5 | j, k, l, 5, J, K, L, |
| #6 | m, n, o, 6, M, N, O, |
| #7 | p, q, r, s, 7, P, Q, R, S, |
| #8 | t, u, v, 8, T, U, V, |
| #9 | w, x, y, z, 9, W, X, Y, Z, |
| #0 | “?”, “_”, “=”, “0” |
| * | Backspace |
| * + # | New line |
| # | Space |
| Play/stop | Play start from cursor position / pause |
| Upper arrow | Push: Previous line Hold: Top of document |
| Down arrow | Push: Next line Hold: End of document |
| Left arrow | Push: Previous character Hold: Home |
| Right arrow | Push: Next character Hold: End |
| 0 + upper arrow | Previous paragraph |
| 0 + down arrow | Next paragraph |
| 0 + left arrow | Previous word |
| 0 + right arrow | Next word |
| * + 1 | Save and exit editing mode |
| * + 2 | Tab |
| * + 3 | Backspace |
| * + 4 | Insert / Overwrite |
| * + 5 | Home |
| * + 6 | screen up (40 lines up) |
| * + 7 | Delete |
| * + 8 | End |
| * + 9 | screen down (40 lines down) |
| Bookmark | Select input mode from ABC mode / Number mode / Symbol table |

Symbol table

| Key name | Assignment |
| --- | --- |
| #1 | Not used |
| #2 | Up Go to the uppermost part of the table |
| #3 | Not used |
| #4 | Left Hold: Go to the leftmost part of the table |
| #5 | Check currently selected symbol |
| #6 | Right Hold: Go to the rightmost part of the table |
| #7 | Not used |
| #8 | Down Hold: Go to the lowermost part of the table |
| #9 | Not used |
| #0 | Not used |
| # | Enter the Symbol |
| (Symbols) | Table has 8 Column * 4 row . , ‘ “ : ; ? ¦ + - = / \ _ [ ] ! @ # $ % ` { } ^ & * ( ) ~ < > |

* Other keys are same as ABC mode.

06/2013 UG-ENG0300-0

127-4381901