PLEXTALK  PTN2 (ver3)

User's Guide

rev 4.0

©2009 Shinano Kenshi Co., Ltd.

# IMPORTANT NOTICES.

## FCC NOTICE.

Declaration of Conformity.

Model Number: PTN2

Trade Name:  PLEXTALK

Responsible party: Shinano Kenshi Co., Ltd.

Address: 6-15-26 Chuo, Ueda-shi, Nagano-ken 386-0012 JAPAN

This device complies with Part 15 of the FCC Rules; Operation is subject to the following two conditions:

(1) This device may not cause harmful interference, and

(2) This device must accept any interference received, including interference that may cause undesired operation.

This equipment has been tested and found to comply with the limits for a Class B digital device, pursuant to part 15 of the FCC Rules.  These limits are designed to provide reasonable protection against harmful interference in a residential installation.  This equipment generates, uses and can radiate radio frequency energy and, if not installed and used in accordance with the instructions, may cause harmful interference to radio communications.  However, there is no guarantee that interference will not occur in a particular installation.  If this equipment does cause harmful interference to radio or television reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by one or more of the following measures:

- Reorient or relocate the receiving antenna.

- Increase the separation between the equipment and receiver.

- Connect the equipment into an outlet on a circuit different from that to which the receiver is connected.

- Consult the dealer or an experienced radio / TV technician for help.

CAUTION:

Any changes or modification not expressly approved by the party responsible for compliance could void the user's authority to operate the equipment.

## Canadian Department of Communications Statement.

This digital apparatus does not exceed the Class B limits for radio noise emissions from digital apparatus set out in the Radio Interference Regulations of the Canadian Department of Communications.

This class B digital apparatus complies with Canadian ICES-003.

Cet appareil numerique de la classe B est conforme a la norme NMB-003 du Canada.

CAUTION:

Use of controls or adjustments or performance of procedures other than those specified herein may result in hazardous radiation exposure.

# TABLE OF CONTENTS.

IMPORTANT NOTICES.	2

FCC NOTICE.	2

Canadian Department of Communications Statement.	3

TABLE OF CONTENTS.	4

1.  Legal and copyright information.	7

1.1.  Copyright information about this guide.	7

1.2.  Licenses and trademarks.	7

1.3.  Restrictions on copying.	7

2.  Getting started.	9

2.1.  Terminology and conventions used.	9

2.2.  What you get in the box.	9

2.3.  Description of the unit.	10

2.4.  Charging PLEXTALK.	13

2.5.  Maintenance and battery care.	14

2.6.  Turning PLEXTALK on and off.	16

2.7.  Key Describer function.	16

2.8.  Key Lock function.	16

2.9.  Inserting and ejecting SD card, USB memory and CD.	17

3. Introduction to the menu system.	19

3.1.  Main menu.	19

3.2.  Accessing and navigating the menu.	19

4.  Setting up your PLEXTALK.	21

4.1.  Selecting your preferred synthetic voice.	21

4.2.  Selecting the pitch of the synthetic voice.	21

4.3.  Setting the date and time.	22

4.4.  Selecting a preferred waiting sound.	22

5.  Power management features.	24

5.1.  Battery low warning.	24

5.2.  Automatic power off function.	24

5.3.  Compulsory power off function.	24

5.4.  Sleep timer function.	25

6.  Playing features common to all formats.	26

6.1.  Start and stop playback.	26

6.2.  Fast forward or fast rewind.	26

6.3.  Changing the playback volume.	27

6.4.  Changing the guide volume.	27

6.5.  Changing the playback speed.	28

6.6.  Changing the audio tone.	28

6.7.  Accessing the Navigation menu.	29

6.8.  Selecting the media and title.	29

6.9.  Folder, category and title navigation.	30

6.10.  Getting information about media and title.	30

6.11.  Getting information on the current status.	31

6.12.  Selecting a playback mode.	31

6.13.  Using bookmarks.	32

7.  Playing DAISY titles.	35

7.1.  Heading navigation.	35

7.2.  Moving to the beginning or end of a DAISY title.	36

7.3.  Page navigation.	36

7.4.  Group navigation.	37

7.5.  Phrase navigation.	37

7.6.  Navigating by screen, paragraph, line, sentence, word or character (DAISY text).	38

7.7.  Playback mode.	38

8.  Playing Audio Files or Music CDs.	40

8.1.  Track navigation.	40

8.2.  Moving to the beginning of an album.	41

8.3.  Album navigation.	41

8.4.  Playback mode.	42

8.5.  Announcing albums and tracks by name or number.	43

9.  Playing Text Files.	44

9.1.  Navigating by screen, paragraph, line, sentence, word or character.	44

9.2.  Navigating by percentage.	45

9.3.  Playback mode.	45

10.  Playing Audio Book.	46

11.  Backup.	47

11.1.  General backup information.	47

11.2.  Backup a title or album from a CD to other media.	47

11.3.  Backup a title or album between SD card and USB media.	48

11.4.  Backup time for music CD.	48

12.  Media Management.	50

12.1.  Deleting a title.	50

12.2.  Erasing an SD card.	50

13.  Other useful features.	52

13.1.  The built in help file.	52

13.2.  Checking system properties.	52

13.3.  "Skippable" settings.	52

13.4.  Reset PLEXTALK to factory defaults.	53

Appendix A - Main specifications.	55

Appendix B - Menu hierarchy.	59

Appendix C - Troubleshooting.	61

# 1.  Legal and copyright information.

This chapter contains important information regarding copyright, licences and trademarks.  It also outlines the restrictions, which apply when using the PLEXTALK for copying or recording digital sound.  You are strongly advised to read this chapter thoroughly before continuing.

## 1.1.  Copyright information about this guide.

Shinano Kenshi reserves the right to modify the products described in this guide at any time without prior notice.

Shinano Kenshi makes no representation or warranties with respect to the contents hereof and specifically disclaims any implied warranties of merchantability or fitness for any particular purpose.  Further, Shinano Kenshi Co., Ltd.  reserves the right to revise this guide and to make changes in its content, without obligation to notify any person or organization of such revision or change.

This guide is copyright.  All right reserved.  It may not be copied, photocopied, translated, or reduced to any electronic medium or machine-readable form without prior permission from Shinano Kenshi.

Manual copyright 2009 Shinano Kenshi Co., Ltd.

## 1.2.  Licenses and trademarks.

All licenses and trademarks are the property of their respective owners.

## 1.3.  Restrictions on copying.

Before using your PLEXTALK, you should review the following important information to ensure that you understand the legal restrictions that apply to its use for certain operations.

### 1.3.1.  General information.

This PLEXTALK device is designed to help you record and reproduce sound works, to which you own the copyright, or where you have gained permission from the copyright owner or the rightful licenser.  Unless you own the copyright or have gained the appropriate permission from the copyright owner or the rightful licenser, your unauthorized recording, reproduction or distribution thereof, may result in severe civil and criminal penalties under copyright laws and international copyright treaties.  If you are uncertain about your rights, contact your legal advisor.

Under no circumstances will Shinano Kenshi be responsible for the consequences of any illegal copying performed using this PLEXTALK recorder.

### 1.3.2.  Music CDs.

Copies of Music CDs are for individual enjoyment and, under the Copyright Act, cannot be used otherwise, without the authorization of the copyright holder.

# 2.  Getting started.

## 2.1.  Terminology and conventions used.

This section details the terminology and conventions used throughout this guide.  The details are as follows:

- WARNING:  A warning notice denotes a hazard that could result in personal injury.

- CAUTION:  A caution notice denotes a hazard that could damage or destroy the PLEXTALK product, destroy the recording media, destroy or affect your data or computer configuration.

- Note:  A note is used to give additional important information about a feature that has not already been mentioned.  Generally these are listed after the instructions for reference.

- SD Card:  PLEXTALK is able to use both "SD" and "SDHC" cards.  In this User's Guide, both cards are called "SD cards".

## 2.2.  What you get in the box.

Below is a list of all the items you should find in the box with your PLEXTALK.  Please make sure that all items are included.  If there are any missing items, please contact your local agent.

- 1 PLEXTALK PTN2

- 1 Mask

- 1 AC adaptor

- 1 User's Guide CD

- 1 Quick Reference guide

- 1 Safety Instruction guide

Note:  It is strongly recommended that you retain all packaging, in case you need to return your PLEXTALK for any reason in the future.

## 2.3.  Description of the unit.

This section describes the physical look of the PLEXTALK and the layout of the keys and other controls to enable you to locate them when referred to in later chapters.  The names and main purposes are also detailed here.

Holding the unit carefully, take a few moments to locate the top and bottom surfaces.  The top has 27 rubber keys on it whereas the bottom has only 5 rubber feet.

Locate the perforated surface of the speaker grill of the PLEXTALK.  Keep the feet downwards and rotate it until the speaker grill is at the upper right from you.  The PLEXTALK should now be set up in its normal operating position.  You can confirm this by feeling the longest side is closest to you.  There should be a narrow, but noticeable, straight dip in this edge.  It is the CD slot.

### 2.3.1.  Top surface.

The PLEXTALK has 27 rubber keys on the top surface.  These are grouped in 5 blocks - a column of 4 keys on the left side, a rectangular block of 12 keys in the centre, a row of 3 keys below the rectangular block, a row of 3 keys above the rectangular block and a column of 5 keys on the right side.

In the column of four keys on the left side, the top key is an ellipse shaped key and the bottom key is a triangle shaped key.  The remaining two keys in the centre are diamond shaped keys.

From top to bottom, the four keys are:

- Menu key - used to access the Main Menu.

- Bookshelf key - used to switch media.

- GoTo key - used for page and heading navigation.

- Bookmark key - used for bookmark navigation, insertion and deletion.

The rectangular block of 12 keys is generally called the Number key and is located at the centre of the top surface.  It is laid out exactly like a push button telephone keypad with 4 rows of 3 keys. The number 5 key has a small raised dot on it to help you locate it.  The Hash key, which is located at the bottom right of the Number Pad, is used for confirming, so it will be called the Enter key throughout this guide.  Similarly, the Star key, which is located at the bottom left of the Number key, is used to cancel, and will be called the Cancel key.  Some of the keys have secondary functions and these are listed below:

Number 2 = Arrow Up or Cancel

Number 4 = Arrow Left or Move Back

Number 6 = Arrow Right or Move Forward

Number 8 = Arrow Down or Enter

Number 9 = Backup files

Star = Undo

Hash = Redo

Underneath the Number key is the row of 3 keys.  The two outside keys are semi circular in shape.  The centre key is rectangular in shape and has a distinctive recessed top surface for easy location.  From left to right, the names and the uses of these three keys are as follows:

- Rewind key - Back or Fast Rewind (if held)

- Play/Stop key - Play/Stop

- Fast Forward key - forward or Fast Forward (if held)

Above the Number key is the row of 3 keys.  The two outside keys are vertically long ellipse in shape and the centre key is larger ellipse in shape.  They are divided into the top and the bottom at the centre.  From left to right, the names of these keys are as follows:

- Tone key - the upper side is called "Tone up key", the lower side is called "Tone down key."

- Volume key - the upper side is called "Volume up key", the lower side is called "Volume down key."

- Speed key - the upper side is called "Speed up key", the lower side is called "Speed down key."

The last group of keys on the top surface is the 5 keys on the right hand side.  The key, which is located in immediate right of the Number 3 or 6 key, is the Information key.  It is ellipse in shape.  The key, which is located immediately to the right of the Fast Forward key, is the Title key.  It is divided into the right and the left at the centre.  The right side is called "Next Title key", the left side is called "Previous Title key."  The use of these keys are as follows:

- Information key - used to get the information of the DAISY book, audio files and text files.  Press and hold the key to hear the current date and time.

- Title key - used to switch media by pressing the right side and left side together or pressing and holding the right side or left side.  To switch titles, press the right side called "Next Title key" or the left side called "Previous Title key."

The other three keys are arranged linearly on the right side of the unit.  The top key is a small round shaped and the bottom key is a small square shaped.  The centre of the two keys is a larger round shaped key, which is slightly sunken.  From top to bottom, the three keys are:

- Sleep timer key - used to set sleep timer

- Power button - used for power on/off

- Eject key - used for ejecting CDs.

### 2.3.2.  Front face.

The CD Insertion and Ejection Slot is located at the front of the unit. Insert your DAISY book CD, music CD or MP3CD into the slot. Once the CD has been inserted, the CD will be taken into the unit automatically.

### 2.3.3.  Left side.

On the left side of the unit there are three items.  From front to back, there are the SD card slot, the headphone jack and the USB slot.  The SD card slot is for both the SD card and SDHC card. The USB slot can be used for USB flash memory.

### 2.3.4.  Back face.

There is one item on the rear of the unit.   On the centre is a small round hole in a square shaped housing.  This is the DC input terminal, which is the socket to take the AC power supply.

### 2.3.5.  Bottom surface.

There is a battery cover on the bottom surface.  The battery cover is fixed with a screw.  To replace the battery, remove the screw and open the battery cover then pull out the battery.

WARNING:  You should only use the recommended battery with this unit.  Failure to do so may cause an explosion or fire.

## 2.4.  Charging PLEXTALK.

In order to charge the internal battery, you need to connect the PLEXTALK to the mains supply.  Do this as follows:

- To charge the battery using mains power, plug the supplied AC adapter into the power input terminal on the centre of the back face.

- Then plug the power adaptor into the mains socket.  Charging will start.

## 2.5.  Maintenance and battery care.

### 2.5.1.  Unit care.

Carefully wipe any dust and dirt from PLEXTALK with a soft cloth that has been slightly dampened with a diluted, neutral detergent, then wipe dry with a cloth.

WARNING:  Always disconnect PLEXTALK from the mains supply before cleaning.

### 2.5.2.  Battery care.

In order that you get the most out of your rechargeable battery, you should take careful note of the following points:

- The battery will normally take 4 hours to fully charge. However, when using your PLEXTALK for the first time, or after one month of storage, you should connect to a mains power supply for at least 24 hours.

- The battery life depends on how frequently it is used and recharged.  Its capacity to store charge reduces over time.  When the usable time following a charge becomes too short for practical use, replace the battery with a new one.  These can be purchased from your local agent.  Your agent should also be able to give you advice about recycling and disposal of used batteries.

- The usable time may be affected by the ambient temperature and working environment.

In order to maintain battery performance, the following procedure should be carried out every six months:

- Fully charge the battery

- Use on battery power until dead

- Recharge

### 2.5.3.  Crossed-out dustbin mark.

Symbol mark called "crossed-out dustbin mark" is printed on the surface of the battery.

Note:  This symbol mark is valid only for EU countries.

This symbol mark is according to the directive 2006/66/EC.  This symbol means that batteries and accumulators, at their end-of-life, should be disposed of separately from regular municipal waste.  If a chemical symbol is printed beneath the symbol shown above, this chemical symbol means that the battery or accumulator contains a heavy metal at a certain concentration listed below.

Hg: mercury (0.0005%)

Cd: cadmium (0.002%)

Pb: lead (0.004%)

These substances may cause serious hazards to human and the global environment if not cared in an appropriate manner.  In the European Union there are separate collection systems for used batteries and accumulators.  Please dispose of batteries and accumulators properly at your local community waste collection or recycling center.  Our product is designed and manufactured with battery and accumulator components which can be recycled or reused.

## 2.6.  Turning PLEXTALK on and off.

To turn the unit on or off, do the following:

- Press and hold down the Power button to turn the unit on.  When the power is on, the Power button will turn green.

- Press and hold the Power button to turn off the unit.

Note:  The booting up process should take approximately 30 seconds when the unit is first used or after the battery has been replaced.

## 2.7.  Key Describer function.

The Key Describer feature is turned on and off using the Menu key.

As its name implies, the Key Describer function allows the user to get a description of any key on the PLEXTALK.

- Turn on the Key Describer by pressing and holding down the Menu key.  Now press any key on the unit and a description of its function will be spoken.

- Turn off the Key Describer function by pressing and holding down the Menu key.

## 2.8.  Key Lock function.

Key Lock can be used to lock the keys and controls so that they can't be accidentally operated.

- To lock the keys, press and hold the Fast Forward key and Rewind key at the same time.

- To unlock the keys, press and hold the Fast Forward key and Rewind key at the same time once more.

## 2.9.  Inserting and ejecting SD card, USB memory and CD.

To insert the SD card, do the following:

- Turn off the power.

- Hold the SD card so that the cut off corner may be located at the front right corner.  In other words, the cut off corner should be nearest to the front side of the unit.

- Insert the SD card into the SD card slot on the left side of the unit.

- Press the card all the way into the slot ensuring that it engages properly, with the outer edge flush with the body of the PLEXTALK.  Do not use excessive force when doing so.

To eject the SD card, do the following:

- Turn off the power.

- Push the SD card inwards and then release it.  The card should pop out leaving the outside edge of the card protruding.

- Carefully grip the card and pull it out of the slot.

CAUTION:  PLEXTALK's SD card slot is only compatible with SD or SDHC cards.  Inserting other memory card such as a Memory Stick or MultiMedia Card, will cause a malfunction and should not be used.

Note:  SD cards have a locking switch that enables you to protect your data if you want.  This is located on the longer side of the card away from the cut off corner.  If the switch is towards the leading edge of the card, you will be able to back up and delete titles.  If the switch is away from the leading edge, your data will be protected.

To insert the USB memory, do the following:

- Turn off the power.

- Insert the USB memory into the USB slot.

To eject the USB memory, do the following:

- Turn off the power.

- Pull out the USB memory straight from the unit.

Note:  External USB CD/DVD drive can not be used with PLEXTALK.

To insert a CD into the PLEXTALK, do the following:

- Ensure that the PLEXTALK is turned on.

- Turn the CD so that the label side is upwards.

- Gently push the CD into the CD slot on the front of the PLEXTALK.

- Continue to push the CD into the slot until the PLEXTALK takes over and feeds the CD the rest of the way.

Note:  If you accidentally insert a CD upside down, the CD will be ejected.

To eject a CD, do the following:

- Press and release the Eject key.

- The CD will be ejected. You can now pull the CD the rest of the way out of the slot.

# 3. Introduction to the menu system.

A number of the features of PLEXTALK are accessed through the built in menu system.  This chapter aims to give you an understanding of how the menu is structured and how you access it.

## 3.1.  Main menu.

At the highest level there is the Main Menu which has 4 items.  These are:

- Playback setting - allows you to listen to files repeatedly.

- Skippable - allows you to select whether you skip some kind of items or not.

- Media management - allows you to get media information, delete a title in a media except CD.

- System settings - allows you to get system information and to control the way certain features operate or play.

## 3.2.  Accessing and navigating the menu.

To navigate around the menu, use the following methods:

- Access the menu by pressing the Menu key once.

- To select a different option on the same level of the menu, press either the Number 4 key to move to the left or the Number 6 key to move to the right.  As you move to a new menu option, it will be spoken.

- On all menus, if you continue in the same direction, you will eventually return to the starting point as the menus are designed to loop round.

- To select a menu option, press the Number 8 key or the Enter key.  Assuming that there is a sub menu, and it is accessible at that time.  This will take you down one level in the menu structure and leave you on the relevant sub menu.

- To move up one level in the menu, press the Number 2 key or the Cancel key.

- To exit immediately from the menu system, from any level in the structure, press the Menu key.

# 4.  Setting up your PLEXTALK.

## 4.1.  Selecting your preferred synthetic voice.

You are given the choice of two synthetic voices for the text to speech voice.  To select your preferred text to speech voice, do the following:

- Press the Menu key once to access the Main Menu.

- Use the Number 4 or 6 key to go to the "System settings" option and select it by pressing the Enter key or Number 8 key.

- Use the Number 4 or 6 key to go to the "Select the synthetic voice" option and select it by pressing the Enter key or Number 8 key.

- Use the Number 4 or 6 key to select your preferred voice and press the Enter key or Number 8 key to confirm.

Note:  If you change the synthetic voice, PLEXTALK will shut down and restart.

## 4.2.  Selecting the pitch of the synthetic voice.

To adjust the pitch of the built in text to speech voice, do the following:

- Press the Menu key once to access the Main Menu.

- Use the Number 4 or 6 key to go to the "System settings" option and select it by pressing the Enter key or Number 8 key.

- Use the Number 4 or 6 key to go to the "Select the pitch of the synthetic voice" option and select it by pressing the Enter key or Number 8 key.

- Use the Number 4 or 6 key to choose your preferred pitch setting, in the range from -5 to +5, with 0 being the normal setting, and select it by pressing the Enter key or Number 8 key.

## 4.3.  Setting the date and time.

To set the date and time for the internal clock, do the following:

- Press the Menu key and select "System Settings" by using the Number 4 or 6 key and then press the Enter key or Number 8 key to confirm.

- Select the "Set the date and time" option from the menu and press the Enter key or Number 8 key.

- Use the Number Pad to enter the current year with 4 digits and press Enter, or just press Enter to accept the current setting.

- Use the Number Pad to enter the current month with 2 digits and then press Enter, or just press Enter to accept the current setting.

- Use the Number Pad to enter the current day with 2 digits and then press Enter, or just press the Enter key to accept the current setting.

- Use the Number Pad to enter the current time with 4 digits and press Enter.  Use leading zeros where necessary.

- If you wish to select the AM or PM setting, before pressing the Enter key, use the Rewind key or Fast Forward key to make your selection.

- By way of confirmation, the date and time that you have just entered will be spoken back to you.

Note:  When entering incorrect number, press the Cancel key to clear the input number.

## 4.4.  Selecting a preferred waiting sound.

When the PLEXTALK is busy, such as when it is accessing a media, it plays a sound to indicate that it is carrying out that action.

To select your preferred waiting sound, do the following:

- Press the Menu key once to access the Main Menu.

- Use the Number 4 or 6 key to go to the "System settings" option and select it by pressing the Enter key or Number 8 key.

- Use the Number 4 or 6 key to go to the "Select the waiting sound" option and select it by pressing the Enter key or Number 8 key.

- Use the Number 4 or 6 key to listen to the available options and press the Enter key or Number 8 key when you are on the waiting sound you prefer.

# 5.  Power management features.

## 5.1.  Battery low warning.

When the PLEXTALK is operating on battery power and the power level drops to a level where it needs to be recharged, the PLEXTALK will say  "Low battery."  Player will shut down in one minute.  If you wish to continue to use the PLEXTALK, you need to plug in the AC adaptor.

## 5.2.  Automatic power off function.

In order to save battery power, the PLEXTALK will automatically shut down after 15 minutes of inactivity even if the battery level is still OK.

Note:  PLEXTALK will not automatically shut down if it is running on mains power.

## 5.3.  Compulsory power off function.

If for any reason, PLEXTALK will not shut down or you are not able to navigate with any keys, you can try using the compulsory power off function, as an alternative to remove the battery.

To use the compulsory power off feature, press and hold down the Sleep timer key and Power button together for more than 10 seconds. PLEXTALK should shut down.

Note:  If this method does not work, and your PLEXTALK is still powered on, you should plug off the AC adaptor, remove the battery and insert the battery again.

## 5.4.  Sleep timer function.

The Sleep Timer feature enables you to specify an amount of time, after which, PLEXTALK will shut down.

### 5.4.1.  Setting the sleep timer.

To set the sleep timer, do the following:

- Press the Sleep Timer key once.  PLEXTALK will say, "Sleep timer.  15 minutes.  Set."

- Press the Sleep Timer key again to increase the time for shutting down.  The Sleep Timer setting will increase by 15 minutes for each press, up to a maximum of 1 hour.  Repeat pressing the Sleep Timer key until you reach the desired setting.

Note:  When you press the Sleep Timer key again, the PLEXTALK may announce the remaining time for shutting down. In this case, press the Sleep Timer key again while the PLEXTALK is announcing, and the Sleep Timer setting will increase.

### 5.4.2.  Cancelling the sleep timer.

To cancel the sleep timer, do the following:

- Press the Sleep Timer key.  PLEXTALK will tell you that the sleep timer is currently on and the number of minutes remaining before shut down.

- Keep pressing the Sleep Timer key repeatedly until PLEXTALK says "Off".

# 6.  Playing features common to all formats.

PLEXTALK is able to play DAISY titles, Audio Files, Text Files and Audio book.

- PLEXTALK is able to recognise and play audio files in the following file formats:  MP3, Wave, WMA, AMR-WB+ and Ogg Vorbis.

- Text files are files that are either in plain text format, in Microsoft Word .doc format or in HTML format.

- Audio books are files that came from "Audible.com".

Note:  If a CD has multiple kinds of content, for example, DAISY files and Audio files, PLEXTALK will recognize one kind of content only.

## 6.1.  Start and stop playback.

To start or stop playback, do the following:

- To start playback, press the Play/Stop key.  PLEXTALK will start playback from the position where it was stopped previously.

- To stop playback, press the Play/Stop key.

Note: To start or stop playback, press and release the Play/Stop key.  If you press and hold the key, the information on the current status will be announced.

## 6.2.  Fast forward or fast rewind.

The fast rewind and fast forward features use the Rewind key and Fast Forward key either side of the Play/Stop key.

- To fast rewind, press and hold down the Rewind key.  Playback will start when you release the key.

- To fast forward, press and hold down the Fast Forward key.  Playback will start when you release the key.

When you play a DAISY book with audio or Audio file, as you hold down the Rewind or Fast Forward key, the fast rewind or fast forward will skip 1 minute, 5 minutes, 10 minutes, 20 minutes and so on.  If the key is released before the "10 minutes" option is spoken, playback will start at the place the key is released.  If the key is released after this point, the fast rewind or fast forward will continue until the Play/Stop key is pressed to stop the process.  Playback will then start automatically.

## 6.3.  Changing the playback volume.

The volume can be set on a scale from 0 to 25, in steps of 1, with 0 being silent and 25 being the highest possible setting.

- To increase the playback volume, either press the Volume up key repeatedly, increasing the level by 1 setting each time, or press and hold down the Volume up key until the volume reaches the desired level.

- To decrease the playback volume, either press the Volume down key repeatedly, decreasing the level by 1 setting each time, or press and hold down the Volume down key until the volume reaches the desired level.

- Pressing the Volume up key and down key together will lead to middle volume position, volume 13.

## 6.4.  Changing the guide volume.

The volume of the guide speech can either be adjusted in conjunction with the playback volume or it can be adjusted independently.

To set the guide volume independently, do the following:

- Press the Menu key once to access the Main Menu.

- Use the Number 4 or 6 key to go to the "System settings" option and select it by pressing the Enter key or the Number 8 key.

- Use the Number 4 or 6 key to go to the "Select guide volume" option and select it by pressing the Enter key or the Number 8 key.

- Use the Number 4 or 6 key to select your preferred guide volume setting and press the Enter key or the Number 8 key to confirm the setting.

## 6.5.  Changing the playback speed.

There are 11 speed settings ranging from -2 to +8, which can be adjusted 1 level at a time.  The normal speed is at the 0 position.  -2 is half of the normal speed, +4 is double speed and +8 is triple speed.

- To increase the playback speed, press the Speed up key.

- To decrease the playback speed, press the Speed down key.

- Pressing the Speed up key and down key together will lead to normal speed position.

Note:  Pressing and holding the Speed up key or down key will cause the speed setting to change rapidly in the selected direction.

## 6.6.  Changing the audio tone.

There are 13 tone settings ranging from -6 to +6, which can be adjusted 1 level at a time.

- To increase the tone, press the Tone up key.

- To decrease the tone, press Tone down key.

- Pressing the Tone up key and down key together will lead to normal tone position.

Note:  Pressing and holding the Tone up key or down key will cause the tone setting to change rapidly in the relevant direction.

## 6.7.  Accessing the Navigation menu.

One of the most useful features of PLEXTALK is the ability to choose how you want to navigate.

To use the Navigation Menu, do the following:

- Use the Number 2 or 8 key to move upwards or downwards.  The items you hear spoken are the options available to you from the Navigation Menu at that time.

- Having selected your preferred navigation method, you can now use your Number 4 or 6 key to move backwards or forwards at your chosen level of navigation.

- For example, if you go to the "Level 1" option with your Number 2 or 8 key, and then use your Number 4 or 6 key, you will be able to move backwards or forwards to the next Level 1 heading.

## 6.8.  Selecting the media and title.

At times you may wish to switch from using an SD card to another media or vice versa.  You can do this as follows:

- Press the Next Title key and Previous Title key together, or press and hold the Next Title key or Previous Title key, or press the bookshelf key repeatedly until your desired media is announced.  PLEXTALK will move sequentially through the 3 available options, "SD card", "USB" and "CD".

- After your desired media is announced, use the Next Title key or Previous Title key to move through the available contents on that media.

- When you are on the item you wish to play, press the Play/Stop key and playback will start at the last playing position in that item.

## 6.9.  Folder, category and title navigation.

If there are multiple items on the SD card or USB media, you can switch from the item you are currently reading to different one.

To navigate by folders, do the following:

- Press the bookshelf key repeatedly until you hear the media you want to go to.

- Press the Number 2 key once to select folder navigation.

- Press the Number 4 or 6 key to move sequentially through the available folders.

- If the folder contains the title you wish to play, press the Next Title key or Previous Title key to select title.

Note:  All Audio Files will be grouped into a single logical folder called "Audio".

To select a specific title by the Number Pad, do the following:

- Press the bookshelf key repeatedly until you hear the media you want to go to.

- Press the Number 2 key twice to select category navigation.

- Press the Number 4 or 6 key to move sequentially through the available categories, which are "DAISY", "Audio", "Text" and "Audio book".

- Press the GoTo key.

- Using the Number Pad, type in the number of the title you wish to go to.

- Press the Enter key to play or the Cancel key to clear your entry.

## 6.10.  Getting information about media and title.

To get a variety of information about the current media and title, do the following:

- Press the Menu key once to access the Main Menu.

- Use the Number 4 or 6 key to go to the "Media Management" option and select it by pressing the Enter key or the Number 8 key.

- Use the Number 4 or 6 key to go to the "Media and title Information" option and select it by pressing the Enter key or the Number 8 key.

- PLEXTALK will announce the information on the current media and title.

- When you have noted the relevant information, press the Enter key to exit the menu system.

## 6.11.  Getting information on the current status.

You can get a range of information on the current status, about heading, page, bookmark and so on.  Whether PLEXTALK is playing or not, do the following:

- Press the Information key once.  Or press and hold the Play/Stop key.

- A range of information will be spoken.

- If you have heard what you want to hear, you can stop the spoken information by pressing the Enter key or the Play/Stop key.

- Press and hold down the Information key to hear the current date and time.

Note:  If you do not wish to listen to all the information, and wish to step through individual items more quickly, you can do this by repeatedly pressing the Information key.

## 6.12.  Selecting a playback mode.

To select the shuffle and repeat settings from playback modes, do the following:

- Press the Menu key once to access the Main Menu.

- Use the Number 4 or 6 key to go to the "Playback Setting" option and select it by pressing the Enter key or Number 8 key.

- Use the Number 4 or 6 key to go to the "Shuffle and repeat settings" option and select it by pressing the Enter key or Number 8 key.

- Use the Number 4 or 6 key to go to your preferred playing mode and select it by pressing the Enter key or Number 8 key.

## 6.13.  Using bookmarks.

### 6.13.1.  Types of bookmark.

There are 2 types of bookmark you can use in PLEXTALK as follows:

- Numbered Bookmarks - You can add up to 10000 Numbered Bookmarks.  Each bookmark is assigned a unique number in the range 1 to 65000.

- Resume Bookmarks - For every title, album or text file, the current playing location will be stored, so playback can be resumed from that location the next time the title, album or text file is used.  Resume Bookmarks are inserted automatically by PLEXTALK when you move to a different title, album, or file or when you shut down the player.  No action is required from you.

Note:  PLEXTALK will remember the Resume Bookmark for up to 1000 titles, albums or text files.

### 6.13.2.  Add bookmarks.

To insert a Numbered Bookmark, whether PLEXTALK is playing or not, do the following:

- Press the Bookmark key twice.

- Using the Number Pad, enter the number you wish to assign to this bookmark.

- If you allow PLEXTALK to assign a number, do not enter the number or enter the number "0".

- Press the Enter key to confirm or the Cancel key to clear your entry.

Note:  When you allow PLEXTALK to assign the number, PLEXTALK selects the smallest number from the unused numbers.

### 6.13.3.  Navigating to bookmarks.

To jump to a Numbered Bookmark, whether PLEXTALK is playing or not, do the following:

- Press the Bookmark key once.

- Using the Number Pad, enter the number of the bookmark you wish to go to.

- Press the Enter key to confirm or the Cancel key to clear your entry.

- When a valid bookmark number has been entered, PLEXTALK will go to the location of that bookmark and playback will start automatically.

Once you have entered bookmarks into a title, album or file, a "Bookmark" option will become available on the Navigation Menu.  In order to navigate to the next bookmark with a lower or higher number, do the following:

- Use the Number 2 or 8 key to go to the "Bookmark" option on the Navigation Menu.

- Press the Number 4 key to move to the next bookmark with a lower number.  Playback will start automatically.

- Press the Number 6 key to move to the next bookmark with a higher number.   Playback will start automatically.

### 6.13.4.  Clearing bookmarks.

To clear a Numbered Bookmark, the procedure is as follows:

- Press the Bookmark key 3 times.

- Using the Number Pad, enter the number of the bookmark you wish to remove.

- Press the Enter key to confirm or the Cancel key to clear your entry.

- Press the Number 0 key followed by the Enter key to remove all the bookmarks on the current DAISY title or Text file. On music files, this procedure will delete all the bookmarks on all albums in current media.

# 7.  Playing DAISY titles.

This chapter covers additional features that can be used when you are playing a DAISY title.

## 7.1.  Heading navigation.

There are 2 ways of navigating to headings within a DAISY title.  You can either go to headings direct or you can move through the available headings one at a time.

To go direct to a specific heading, do the following:

- Press the GoTo key twice.

- Using the Number Pad, type in the number of the heading you wish to go to.

- Press the Enter key to confirm or the Cancel key to clear your entry.

- When you have entered a valid heading number and pressed the Enter key, PLEXTALK will switch to the heading you selected and playback will start automatically.

To navigate headings sequentially, using heading levels, do the following:

- Use the Number 2 or 8 key to go to your desired level of navigation, for example Level 1, Level 2 and so on.

- Press the Number 4 key to move backwards to the previous heading on the selected level.  PLEXTALK will move to the heading and playback will start automatically.

- Press the Number 6 key to move forwards to the next heading on the selected level.  PLEXTALK will move to the heading and playback will start automatically.

Note:  When navigating in this way, PLEXTALK will stop at all headings that have a level the same as or higher than the level you have selected.  For example, if you chose to navigate on Level 3, PLEXTALK will find all Level 1, Level 2 and Level 3 headings as you move backwards or forwards.

## 7.2.  Moving to the beginning or end of a DAISY title.

To move to the beginning of a DAISY title, do the following:

- Press the GoTo key twice.

- Press the Enter key and the playing position will move to the beginning of the current item.

To move to the end of a DAISY title, do the following:

- Press the GoTo key twice.

- Press the Number 0 key followed by the Enter key and the playing position will move to the end of the current item.

## 7.3.  Page navigation.

Navigating to pages is very similar to heading navigation.

To go direct to a specified page, do the following:

- Press the GoTo key once.

- Using the Number Pad, type in the number of the page you wish to go to.

- Press the Enter key to confirm or the Cancel key to clear your entry.

- When you have entered a valid page number and pressed the Enter key, PLEXTALK will switch to the page you selected and playback will start automatically.

To navigate pages sequentially, do the following:

- Use the Number 2 or 8 key to go to the "Page" option on the Navigation Menu.

- Press the Number 4 key to move backwards to the previous page.  PLEXTALK will move back to the previous page marker and playback will start automatically.

- Press and hold the Number 4 key to move backwards every ten pages.  Playback will start when you release the key.

- Press the Number 6 key to move forwards to the next page.  PLEXTALK will move forwards to the next page marker and playback will start automatically.

- Press and hold the Number 6 key to move forwards every ten pages.  Playback will start when you release the key.

Note:  If there are no page markers in the title, the "Page" option will not be listed on the Navigation Menu.

## 7.4.  Group navigation.

To move backwards or forwards a group at a time, do the following:

- Use the Number 2 or 8 key to go to the "Group" option on the Navigation Menu.

- Use the Number 4 key to move backwards to the previous group item in the title.  PLEXTALK will move backwards to the previous group item and playback will start automatically.

- Use the Number 6 key to move forwards to the next group item in the title.  PLEXTALK will move forwards to the next group item and playback will start automatically.

Note:  If there are no group markers in the title, the "Group" option will not be listed on the Navigation Menu.

## 7.5.  Phrase navigation.

To move backwards or forwards one phrase, do the following:

- To jump back one phrase, press the Number 2 or 8 key to select the "Phrase" option on the Navigation Menu and then press the Number 4 key.  PLEXTALK will move backwards to the start of the previous phrase and playback will start automatically.  Or simply press the Rewind key to move back one phrase.

- To jump forwards one phrase, press the Number 2 or 8 key to select the "Phrase" option on the Navigation Menu and then press the Number 6 key.  PLEXTALK will jump forwards to the start of the next phrase and playback will start automatically.  Or simply press the Fast Forward key to move forwards one phrase.

## 7.6.  Navigating by screen, paragraph, line, sentence, word or character (DAISY text).

DAISY text file will always give you the navigation options "Screen", "Paragraph", "Line", "Sentence", "Word" and "Character".  These options will automatically appear on the Navigation Menu when you access this type of file.

To navigate by any of these options, the process is identical as follows:

- Use the Number 2 or 8 key to select your preferred navigation method that is screen, paragraph, line, sentence, word or character.

- Use the Number 4 key to jump back in the file by the selected amount.

- Use the Number 6 key to jump forward in the file by the selected amount.

Note:  "Screen" means 40 lines.

## 7.7.  Playback mode.

You can select from two playback modes when using a DAISY title.  The available options are as follows:

- Normal playback:  it plays through the title and stops when finished.

- Repeat title: the current title will repeat continuously until playback is stopped.

# 8.  Playing Audio Files or Music CDs.

This chapter covers features that are specific to playing Audio Files or Music CDs.

PLEXTALK is able to play following types of audio file:

- MP3 Files; MPEG1 Layer 3 from 32kbps to 320kbps and MPEG2 Layer 3 from 16kbps to 160kbps.

- WAVE Files.

- WMA Files; PLEXTALK does not support DRM protected WMA files.

- AMR-WB+ Files.

- Ogg Vorbis Files; from 45 kbps to 320 kbps

CAUTION:  PLEXTALK may not be able to play a Music CD that is not compliant with the CD digital audio (CD-DA) standard, such as copy-protected Music CDs.

Note:  The maximum number of files that can be stored in a media is 6000.  Files in excess of 6000 are not recognized.  The maximum number of folders is 1024, including a root folder.  Folders in excess of 1024 are not recognized.

## 8.1.  Track navigation.

You can either go to tracks direct or you can move through the available tracks one at a time.

To go direct to a specified track, do the following:

- Press the GoTo key once.

- Using the Number Pad, type in the number of the track you wish to go to.

- Press the Enter key to confirm or the Cancel key to clear your entry.

- When you have entered a valid track number and pressed the Enter key, PLEXTALK will switch to the track you selected and playback will start automatically.

To navigate tracks sequentially, do the following:

- Use the Number 2 or 8 key to go to the "Track" option on the Navigation Menu.

- Press the Number 4 key to move backwards to the previous track.  PLEXTALK will move back to the previous track and playback will start automatically.

- Press the Number 6 key to move forward to the next track.  PLEXTALK will move forward to the next track and playback will start automatically.

- Or simply press the Rewind key to move backwards to the previous track.  Press the Fast Forward key to move forward to the next track.

Note:  When moving back a track at a time, if you allow the track to play for more than 3 seconds, the next time you move back a track, you will move to the start of the current track.  If you move back a second time within 3 seconds, you will move to the previous track.

## 8.2.  Moving to the beginning of an album.

To move to the beginning of an album, do the following:

- Press the GoTo key once.

- Press the Enter key and the playing position will move to the start of the current item.

## 8.3.  Album navigation.

You can either go to albums direct or you can move through the available albums one at a time.

To go direct to a specified album, do the following:

- Press the GoTo key twice.

- Using the Numeric Pad, type in the number of the album you wish to go to.

- Press the Enter key to confirm or the Cancel key to clear your entry.

- When you have entered a valid album number and pressed the Enter key, PLEXTALK will switch to the album you selected and playback will start automatically.

To navigate albums sequentially, do the following:

- Press the Number 2 or 8 key and select "Album" from the Navigation Menu.

- Press the Number 4 key to move backwards to the previous album.  PLEXTALK will move back to the previous album.

- Press the Number 6 key to move forward to the next album.  PLEXTALK will move forward to the next album.

## 8.4.  Playback mode.

When playing Audio Files, you can select a number of different playback modes.  The available options are as follows:

- Normal playback:  it plays through the current album and stops when finished.

- Repeat track:  the current track will repeat continuously until playback is stopped.

- Repeat album:  the current album will repeat continuously until playback is stopped.

- Repeat all albums:  all albums will be repeated sequentially until playback is stopped.

- Shuffle repeat:  the files in all the album will be continuously played in random order until playback is stopped.

## 8.5.  Announcing albums and tracks by name or number.

When navigating albums or tracks, PLEXTALK announces albums and tracks by name or number.  Either setting to read the name or the number can be selected.

For this setting, do the following:

- Press the Menu key once to access the Main Menu.

- Use the Number 4 or 6 key to go to the "Playback Setting" option and select it by pressing the Enter key.

- Use the Number 4 or 6 key to go to the "Reading audio file name" option and select it by pressing the Enter key.

- Use the Number 4 or 6 key to select between "Read audio file name" and "Read audio track number", and select it by pressing the Enter key.

# 9.  Playing Text Files.

In the context of this guide, Text Files can either be plain text, Microsoft Word .doc files or HTML files.

## 9.1.  Navigating by screen, paragraph, line, sentence, word or character.

A Text File will always give you the navigation options "Screen", "Paragraph", "Line", "Sentence", "Word" and "Character".  These options will automatically appear on the Navigation Menu when you access this type of file.  For Word and HTML files that have been coded with headings, you will be given an additional navigation option on the Navigation Menu.  This will be called "Level".

To navigate by any of these options, the process is identical as follows:

- Use the Number 2 or 8 key to select your preferred navigation method i.e. paragraph, sentence, word or character.

- Use the Number 4 key to jump back in the file by the selected amount.

- Use the Number 6 key to jump forward in the file by the selected amount.

To navigate paragraphs sequentially, do the following:

- To move backwards to the previous paragraph, simply press the Rewind key.  Press the Fast Forward key to move forward to the next paragraph.

Note:  "Screen" means 40 lines.

## 9.2.  Navigating by percentage.

In Text Files, you can navigate by a percentage of the file.  To navigate by percentage, do the following:

- Press the GoTo key once.

- Type in a percentage between 0 percent and 100 percent, and press the Enter key.

- PLEXTALK will move the playback position to that position in the Text File.

## 9.3.  Playback mode.

When playing Text Files, you can select a preferred playback mode.  The available options are as follows:

- Normal playback:  it plays through the title and stops when finished.

- Repeat title:  the current title will repeat continuously until playback is stopped.

# 10.  Playing Audio Book.

PLEXTALK is able to play the "Audio Book" which is supplied from "audible.com".  Audio book navigations are similar to a DAISY book except some points.

- Heading and percentage navigation is available by GoTo key.

- About the previous and next navigation by 4 or 6 key, you can select heading, ten minutes, 30 seconds, and bookmark by 2 or 8 key.

- Two playback modes, "Normal playback" and "Repeat title" options, can be selected from playback setting.

# 11.  Backup.

## 11.1.  General backup information.

Before backing up any data, the following points should be noted:

- When back up data from a media to the other media, the former is called "source media" and the latter is called "target media."

- You can backup a variety of data from CD to other media.  CD drive can only be used as a source for backup purposes.

- If you backup from a Music CD, you can choose the audio compression to be used.  It will be backed up as Audio Files and not in DAISY format.

- In the back up process, PLEXTALK gives a verbal warning if there is insufficient space available on the target media.

## 11.2.  Backup a title or album from a CD to other media.

To backup a title or album from CD to other media, do the following:

- If you backup a title or album to SD card or USB media, insert SD card or USB media with sufficient free space.  Ensure that SD card is unlocked.

- Insert the source CD into the CD slot.

- Navigate, on the CD, to the title or album that you wish to backup.

- Press the Number 9 key once to access the Backup feature.

- When backing up from a Music CD, press the Number 4 or 6 key to go to your preferred audio compression and press the Enter key or Number 8 key to confirm your selection.

- PLEXTALK will ask you to confirm that you wish to continue with the backup.  Press the Enter key or Number 8 key to continue or the Cancel key to abort.

- If you choose to continue, the backup process will start.  PLEXTALK will announce when the backup is complete.

Note:  If you wish to cancel the backup process whilst it is running, press the Cancel key.

## 11.3.  Backup a title or album between SD card and USB media.

To back up a title or album between SD card and USB media, do the following:

- Insert SD card and USB media.  Ensure that SD card is unlocked if it is the target media.

- Navigate, on the source media, to the title or album that you wish to back up.

- Press the Number 9 key once to access the backup feature.

- PLEXTALK will ask you to confirm that you wish to proceed with the backup.  Press the Enter key or Number 8 key to proceed or the Cancel key to abort.

- If you choose to proceed, the backup process will start.  PLEXTALK will announce when the backup is complete.

Note:  If you wish to cancel the backup process whilst it is running, press the Cancel key.

## 11.4.  Backup time for music CD.

For the range of available sample rates or bit rates, the approximate backup times are given below:

Backing up about 50 minutes of audio from a music CD to SD card:

- PCM 44.1kHz Stereo ; 35 minutes approximately.

- MP3 256K Stereo High Quality ; 50 minutes approximately.

- MP3 256K Stereo ; 40 minutes approximately.

- MP3 128K Stereo High Quality ; 55 minutes approximately.

- MP3 128K Stereo ; 40 minutes approximately.

Note:  All times are approximate.  The backup time will depend on the size of the source title and numbers of files.  It will also depend on the writing speed of the SD card.

# 12.  Media Management.

## 12.1.  Deleting a title.

PLEXTALK enables you to delete a title, album or text file.

To delete a title, do the following:

- Go to the title that you wish to delete.

- Press the Menu key once to access the Main Menu.

- Use the Number 4 or 6 key to go to the "Media management" option and select it by pressing the Enter key or Number 8 key.

- Use the Number 4 or 6 key to go to the "Delete selected title" option and select it by pressing the Enter key or Number 8 key.

- PLEXTALK will then ask you to confirm that you wish to proceed.  Press the Enter key to confirm or the Cancel key to abort.

Note:  The titles on the optical disc can not be deleted.

## 12.2.  Erasing an SD card.

PLEXTALK allows you to erase an SD card as many times as you want.  To erase an SD card, do the following:

- Press the Menu key once to access the Main Menu.

- Use the Number 4 or 6 key to go to the "Media Management" option and select it by pressing the Enter key or Number 8 key.

- Use the Number 4 or 6 key to go to the "Delete all files on the SD card" option and select it by pressing the Enter key or Number 8 key.

- PLEXTALK will ask you to confirm that you wish to continue with the formatting process.  Press the Enter key or Number 8 key again to continue or the Cancel key to abort.

- If you choose to continue, the formatting process will start.  PLEXTALK will announce when the process is complete.

Note:  If the SD card is erased, all data on the SD card will be deleted.

# 13.  Other useful features.

## 13.1.  The built in help file.

To access the built in help file, do the following:

- Press the Number 0 key once.

- Navigate the file in the same way that you navigate other DAISY text only titles.

To exit the help file, do the following:

- Press the Number 0 key once.

## 13.2.  Checking system properties.

To find out the system version or the serial number of your PLEXTALK, do the following:

- Press the Menu key once to access the Main Menu.

- Use the Number 4 or 6 key to go to the "System settings" option and select it by pressing the Enter key or Number 8 key.

- Use the Number 4 or 6 key to go to the "System properties" option and select it by pressing the Enter key or Number 8 key.

- The system version and serial number will be played repeatedly.

- Also, the PDTB key information will be played only if you have installed the key.

- When you have noted the information that you require, press the Enter key or Number 8 key to exit the menu system.

## 13.3.  "Skippable" settings.

Some DAISY books have items that you are able to skip. PLEXTALK enables you to select whether you skip the item or not.  This feature is called "Skippable."  Skippable items are as follows:

- Page number

- Footnote

- Note reference

- Annotation

- Line number

- Sidebar

- Producer note

To set the "Skippable" settings, do the following:

- Press the Menu key once to access the Main Menu.

- Use the Number 4 or 6 key to go to the "Skippable" option and select it by pressing the Enter key.

- Use the Number 2 or 8 key to select Skippable item and use the Number 4 or 6 key to select "Skip" or "Play."

- After deciding all the Skippable items to be skipped or not, press the Enter key.

## 13.4.  Reset PLEXTALK to factory defaults.

To reset all settings back to the factory defaults, do the following:

- Press the Menu key once to access the Main Menu.

- Use the Number 4 or 6 key to go to the "System settings" option and select it by pressing the Enter key.

- Use the Number 4 or 6 key to go to the "Initialize all settings to default" option and select it by pressing the Enter key.

- PLEXTALK will ask you to confirm that you wish to proceed.  Press the Enter key to set all settings to factory defaults or the Cancel key to abort.  After a short while, PLEXTALK will restart.

The following is a list of all the default settings for the PLEXTALK.

- Playback volume (Main volume):  13

- Playback speed:  Normal

- Playback tone:  Normal

- Playback setting:  Normal playback

- Skippable: Play

- Guide volume:  Normal

- Synthetic voice: depending on language

- Select waiting sound:  Waiting sound 1

- The pitch of the synthetic voice:  Normal

# Appendix A - Main specifications.

Note:  Shinano Kenshi reserves the right to modify the products specification at any time without prior notice.

This section details the main specifications for the PLEXTALK PTN2.

### General specifications.

CD Loading system:

Slot insertion

Compatible CDs:

12cm diameter CD, CD-R and CD-RW discs.  PLEXTALK is only compatible with round 12cm CDs.  8cm CD and irregularly shaped CDs, such as the business card type, will cause a malfunction and should not be used.

SD card slot:

SD/SDHC card

Playable contents:

Books conforming to DAISY 2.0, 2.02 and 3.0 standards

Audio files in MP3, WMA, AMR-WB+, Ogg Vorbis, Wave (PCM) formats and Audible Format 4(.aa).

Music CDs (CD-DA)

Audio input and output:

Headphone terminal (Stereo)

Built in speaker (Monaural)

Externally connected terminal:

USB 2.0

Clock precision:

Approximately 60 seconds per month

Electric Power Source:

AC adaptor, AC 100 V to 240 V, 50 or 60 Hz

Dimensions and weight:

Length 219 mm

Width 170 mm

Height 56 mm

Weight Approx.  1300 grams

Power consumption (maximum):

25W

Working temperature and humidity:

Fahrenheit from 40 to 104

Celsius from 5 to 40

Humidity from 20% to 80%, no moisture.

### Audio features.

Headphones (line output):

Compatible impedance 32 ohm unbalanced

Jack 3.5mm diameter stereo mini jack

Built In Speaker:

Output impedance 4 ohm

Output 2.0W

### Main functions.

Guide voice:

Gives spoken information about the current operation

Provides key descriptions in Key Describer mode

Speaks menu items

Gives help information (press the Information key)

Search function:

Heading, Group, Page, Phrase, Bookmark, Album, Track, Screen, Paragraph, Line, Sentence, Word, Character

Volume adjustment:

0 to 25 (in increments of 1)

Guide volume adjustment:

-5 to +5 (in increments of 1)

Play speed adjustment:

11 settings from -2, through 0, to +8 (in increments of 1).

Normal speed is at 0.

-2 is 0.5 times speed, +4 is double speed, +8 triple speed.

Tone adjustment:

13 settings from -6, through 0, to +6 (in increments of 1)

Normal tone is at 0

Memory function:

Bookmarks, maximum is 10000

### Battery specifications.

Type:  Nickel-metal hydride

Dimensions:

Length 53.64 mm

Width 56.68 mm

Height 29.00 mm

Capacity:  1800 milli-ampere hour

Charging time:  4 hours approximately

Operating time:

DAISY format, CD playing, 5 hours

Music CD (CD-DA) format, CD playing, 2 hours

DAISY format, SD card playing, 8 hours

Note:  All times are approximate.

Note:  The charging time and operating time vary depending on the ambient temperature and working conditions.

### Copyright information about Ogg Vorbis.

This software is provided by the copyright holders and contributors "as is" and any express or implied warranties, including, but not limited to, the implied warranties of merchantability and fitness for a particular purpose are disclaimed. In no event shall the foundation or contributors be liable for any direct, indirect, incidental, special, exemplary, or consequential damages (including, but not limited to, procurement of substitute goods or services; loss of use, data, or profits; or business interruption) however caused and on any theory of liability, whether in contract, strict liability, or tort (including negligence or otherwise) arising in any way out of the use of this software, even if advised of the possibility of such damage.

# Appendix B - Menu hierarchy.

This appendix lists all menu items in the Main Menu along with their corresponding sub menu hierarchy.

### PLAYBACK SETTINGS:

+Shuffle and repeat settings

Normal playback

Repeat title (DAISY, Text, Audio book)

Repeat track (Audio Files)

Repeat album (Audio Files)

Repeat all albums (Audio Files)

Shuffle repeat (Audio Files)

+Reading audio file name

Read audio file name

Read audio track number

### SKIPPABLE:

+Select page number

Play

Skip

+Select footnote

Play

Skip

+Select note reference

Play

Skip

+Select annotation

Play

Skip

+Select line number

Play

Skip

+Select sidebar

Play

Skip

+Select producer note

Play

Skip

### MEDIA MANAGEMENT:

+Media and title information

+Delete all files on the SD card

+Delete selected title (album)

### SYSTEM SETTINGS:

+System properties

System version

Serial number

Installed key (if obtained.)

+Select guide volume (On scale -5 to +5)

+Select the synthetic voice

English 1

English 2

+Select the waiting sound

Waiting sound 1

Waiting sound 2

No waiting sound

+Set the date and time

+Select the pitch of the synthetic voice (On the scale -5 to +5)

+Initialize all settings to default

# Appendix C - Troubleshooting.

### General operating problems.

Symptom:  You turn on the Power button, but the power does not come on.

Possible cause:  Loose power cord or AC adaptor.

Solution:  Firmly reinsert the mains power cable into PLEXTALK and ensure that the AC adaptor is firmly inserted.

Symptom:  The power is on, but PLEXTALK does not operate.

Possible cause:  Key Lock is On.

Solution:  Turn off the key lock by pressing the Rewind key and Fast Forward key at the same time.

Symptom:  PLEXTALK will not shut down and no keys can be operated.

Possible cause 1:  Key Lock is On.

Solution:  Turn off the key lock by pressing the Rewind key and Fast Forward key at the same time.

Possible cause 2:  Unknown error.

Solution:  Press and hold down the Sleep timer key and Power button together for more than 10 seconds, then PLEXTALK will shut down.  If this does not work, remove the AC Adaptor and Battery from the unit and then reinsert them.

Symptom:  There is no voice guidance.

Possible cause 1:  The volume has been set to '0'.

Solution:  Increase the volume so that the guide speech can be heard.

Possible cause 2:  The power is off.

Solution:  Turn the power on.

### Spoken errors during DAISY playback.

Guide voice:  "Unsupported SD card" or "Unsupported USB device."

Possible cause:  The SD card or USB media has been formatted with an unsupported file system.

Solution:  Insert an SD card which is formatted as FAT-16 or FAT-32.

Guide voice:  Read error.

Possible cause:  Title cannot be played back.

Solution:  Press the Previous Title key or Next Title key to try to select a different title, or switch the media including a different title.

### Errors during backup.

Guide voice:  "There is no target media".

Possible cause:  You have not inserted a suitable backup media.

Solution:  Insert either an SD card or a USB media with sufficient storage space to perform the backup and then try again.

Guide voice:  "The SD card is locked".

Possible cause:  You have locked the SD card using the lock switch on the side of the card.

Solution:  Eject the card, move the locking switch to the unlocked position, reinsert it into PLEXTALK and try again.