[image: image5.png]



PLEXTALK  PTP1

Version 5
User's Guide  for U.K. English
[image: image2.wmf]
[image: image1.wmf]
[image: image3.wmf]


     
©2008 Shinano Kenshi Co., Ltd.
IMPORTANT NOTICES

FCC NOTICE

Declaration of Conformity

Model Number: PTP1

Trade Name:  PLEXTALK

Responsible party: Shinano Kenshi Co., Ltd.

Address: 1078, Kami-maruko, Ueda-shi, Nagano-ken 386-0498 JAPAN

This device complies with Part 15 of the FCC Rules; Operation is subject to the following two conditions:

(1) This device may not cause harmful interference, and

(2) This device must accept any interference received, including interference that may cause undesired operation.

This equipment has been tested and found to comply with the limits for a Class B digital device, pursuant to part 15 of the FCC Rules. These limits are designed to provide reasonable protection against harmful interference in a residential installation. This equipment generates, uses and can radiate radio frequency energy and, if not installed and used in accordance with the instructions, may cause harmful interference to radio communications. However, there is no guarantee that interference will not occur in a particular installation. If this equipment does cause harmful interference to radio or television reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by one or more of the following measures:

· Reorient or relocate the receiving antenna.

· increase the separation between the equipment and receiver.

· Connect the equipment into an outlet on a circuit different from that to which the receiver is connected.

· Consult the dealer or an experienced radio / TV technician for help.

CAUTION: Any changes or modification not expressly approved by the party responsible for compliance could void the user's authority to operate the equipment.

Canadian Department of Communications Statement

This digital apparatus does not exceed the Class B limits for radio noise emissions from digital apparatus set out in the Radio Interference Regulations of the Canadian Department of Communications.

This class B digital apparatus complies with Canadian ICES-003.  

Cet appareil numerique de la classe B est conforme a la norme NMB-003 du Canada.  

CAUTION:  Use of controls or adjustments or performance of procedures other than those specified herein may result in hazardous radiation exposure.

Table of Contents

2IMPORTANT NOTICES


2FCC NOTICE


2Canadian Department of Communications Statement


4Table of Contents


101. Legal and copyright information.


101.1. Copyright information about this guide.


101.2. Licenses and trademarks.


101.3. Restrictions on copying or recording.


111.3.1. General information.


111.3.2. Music CDs.


122. Getting started.


122.1. Terminology and conventions used.


122.2. What you get in the box.


132.3. Description of the unit.


132.3.1. Front face.


142.3.2. Top edge.


142.3.3. Left edge.


152.3.4. Right edge.


152.3.5. Bottom edge.


152.3.6. Back face.


152.4. Inserting the battery.


162.5. Charging PLEXTALK.


172.6. Maintenance and battery care.


172.6.1. Unit care.


172.6.2. Battery care.


182.6.3. Crossed-out dustbin mark.


182.7. Turning PLEXTALK on and off.


192.8. Key Describer function.


192.9. Key Lock function.


192.10. Inserting and ejecting SD card


213.  Introduction to the menu system.


213.1.Main menu.


213.2.  Accessing and navigating the menu.


223.3.  Shortcut for the menu selection.


234.  Setting up your PLEXTALK.


234.1. Selecting your preferred synthetic voice.


234.2. Setting the text to speech pitch.


234.3. Setting the date and time.


244.4. Selecting a preferred waiting sound.


244.5. Selecting the Key Press Confirmation Sound.


265.  Power management features.


265.1.  Battery low warning.


265.2.  Automatic power off function.


265.3.  Compulsory power off function.


265.4.  Sleep timer function.


265.4.1.  Setting the sleep timer.


275.4.2.  Cancelling the sleep timer.


286.  Playing features common to all formats.


286.1.  Start and stop playback.


286.2.  Fast forward or fast rewind.


296.3.  Changing the playback volume.


296.4.  Changing the playback speed.


296.5.  Changing the audio tone.


306.6.  Changing the guide volume.


306.7.  Changing the guide speed.


306.8.  Accessing the Navigation menu.


316.9.  Selecting the media and title.


316.10.  Title navigation.


326.11. Go to a title directly


326.12. Switching sorting order of titles


336.13.  Getting information about media and title.


336.14.  Getting information on the current status.


336.15.  Selecting a playback mode.


346.16.  Using bookmarks.


346.16.1.  Types of bookmark.


346.16.2.  Inserting bookmarks.


356.16.3.  Navigating to bookmarks.


356.16.4.  Clearing bookmarks.


366.16.5.  Turning on and off Voice Bookmark speech.


366.17.  Understanding how PLEXTALK groups files and folders.


387.  Playing DAISY titles.


387.1.  Heading navigation.


387.2.  Moving to the beginning or end of the title.


397.3.  Page navigation.


397.4.  Group navigation.


397.5.  Phrase navigation.


407.6.  10 minutes skip, 30 seconds skip


407.7.  Navigating by screen, paragraph, line, sentence, spell, word or character in case of DAISY text mode.


407.8.  Navigating by percentage.


407.9.  Playback mode.


417.10. Multimedia DAISY playback setting


428. Playing "Recordings"


439.  Playing Audio Files or Music CDs.


439.1.  Track navigation.


449.2.  Album navigation.


449.3.  10 minutes skip, 30 seconds skip


449.4.  Playback mode.


449.5.  Announcing albums and tracks by name or number.


459.6.  About playing sequences.


459.7.  Restrictions on Audio Files.


4610.  Playing Text Files.


4610.1.  Navigating by screen, paragraph, line, sentence, spell, word or character.


4610.2.  Navigating by percentage.


4710.3.  Playback mode.


4811.  Playing Audio Book.


4912. Podcast


4912.1.  Adding Podcast to PLEXTALK


4912.2.  Listening to Podcast for the first time


5012.3. Playing Podcast


5012.4. Navigating by Episode, 10 minutes or 30 seconds.


5112.5.  Getting the latest episode list


5112.6. Download all Episodes of the feed


5112.7. Notice during the download


5213. Web radio


5213.1. Adding web radio station to the playlist


5213.2.  Listening to web radio


5314.  Recording a DAISY title.


5314.1.  Basic information about recording with PLEXTALK.


5314.2.  Recording hardware.


5414.3.  Recording media.


5414.4.  Recording states.


5414.5.  Basic recording technique.


5514.6.  Muting the guide speech when recording.


5514.7.  Connecting and selecting an input source.


5614.8.  Selecting a recording mode and sound quality setting.


5714.8.1.  About recording modes and sound quality.


5714.8.2.  Selecting a preset recording mode.


5814.8.3.  Selecting the Custom recording mode.


5914.9.  Selecting the title to record into.


6014.10.  Setting the recording level.


6114.11.  Using the level monitor.


6114.12.  Turning the recording monitor on and off.


6214.13.  Getting recording information.


6315.  Editing a DAISY title.


6315.1.  Editing heading markers during recording.


6315.2.  Setting a heading after recording.


6415.3.  Cancelling a heading setting after recording.


6415.4.  Removing a section or multiple sections.


6616.  Media management features.


6616.1. Erasing an SD card.


6616.2. Creating a new title.


6616.3. Deleting a title.


6716.4. Deleting a file


6716.5. Preparing to edit a DAISY title.


6816.6. Building a DAISY title.


6816.7. Network folder function


7017. Using PLEXTALK's Voice Memo functions.


7017.1. Recording a new Voice Memo.


7017.2. Reviewing existing Voice Memos.


7117.3. Back up Voice Memos to SD card.


7117.4. Deleting Voice Memos.


7218.  Backup


7218.1.  General backup information.


7218.2.  Back up Voice Memos.


7318.3.  Backup a DAISY title, album or text from an SD card to an external USB media.


7318.4.  Backup a DAISY title, album or text from an external USB media to an SD card.


7418.5.  Backup time for music CD.


7519.  Using PLEXTALK as an SD card drive.


7519.1.  What can PLEXTALK be used for when connected to a personal computer?


7519.2.  Connecting PLEXTALK to a computer as an external drive.


7619.3.  Disconnecting PLEXTALK from a computer.


7720. Network settings


7720.1. Connecting to the wireless network.


7720.1.1. Enables network device


7720.1.2. Confirm available wireless network and connect.


7920.2. How PLEXTALK connect to network


7920.3. Advanced Network settings


7920.3.1. Confirming network information


8020.3.2. Loading network setting


8020.3.3. Saving the network setting


8120.3.4. Adding wireless network setting


8120.3.5. Change or delete wireless network setting


8220.3.6. Network folder setting


8320.3.7. Initializing network settings to default


8521. Editing text files


8521.1. Creating a new text file


8521.2. Editing an existing text file on the SD card


8521.3. Saving the edited text


8621.4. Exiting text editing


8621.4.1. Saving and exiting text editing


8621.4.2. Exiting text editing without saving the text


8721.5. Renaming the file


8721.6. Deleting current line


8721.7. Deleting all text


8822. Entering text


8822.1. Entering alphabet by ABC mode


8922.2. Entering numbers by number mode


8922.3. Entering symbols by Symbol table


9022.4. Moving entering position


9122.5. Enter space, linefeed and erase characters


9223. Other useful features.


9223.1.  Accessing the built in help file.


9223.2.  Checking system properties.


9223.3.  Deleting authorization keys


9323.4.  Reset PLEXTALK to factory defaults.


95Appendix A - Main Specifications.


95General specifications.


96Audio features.


97Main functions.


98Battery specifications.


98Memory Card Drive Specifications.


99Wireless communication function


99Copyright information about Ogg Vorbis.


100Appendix B - Menu Hierarchy.


103Appendix C - Troubleshooting.


103General operating problems.


104Spoken errors during DAISY playback.


104Errors using the Voice Memo.


104Errors when using PLEXTALK as an SD card drive on a computer.


106Appendix D -  Technical data about recording modes and sound qualities.


1061.  Recording mode settings.


1062.  Settings for preset recording modes.


1073.  Available settings for Custom mode.


1074.  Available recording time for the specified sound qualities.


109Appendix E - Questions and Answers on Network setting


111Appendix F - List of text input




1. Legal and copyright information.

This chapter contains important information regarding copyright, licences and trademarks.  It also outlines the restrictions, which apply when using the PLEXTALK for copying or recording digital sound.  You are strongly advised to read this chapter thoroughly before continuing.

1.1. Copyright information about this guide.

Shinano Kenshi reserves the right to modify the products described in this guide at any time without prior notice.

Shinano Kenshi makes no representation or warranties with respect to the contents hereof and specifically disclaims any implied warranties of merchantability or fitness for any particular purpose. Further, Shinano Kenshi Co., Ltd.  reserves the right to revise this guide and to make changes in its content, without obligation to notify any person or organization of such revision or change.

This guide is copyright.  All right reserved.  It may not be copied, photocopied, translated, or reduced to any electronic medium or machine-readable form without prior permission from Shinano Kenshi.

Manual copyright 2008 Shinano Kenshi Co., Ltd.

1.2. Licenses and trademarks.

All licenses and trademarks are the property of their respective owners.

Windows 2000, Windows XP, Windows Vista, Windows 7 are the trademarks of Microsoft Corporation.

iTunes is the trademark of  Apple Inc.

1.3. Restrictions on copying or recording.

Before using your PLEXTALK, you should review the following important information to ensure that you understand the legal restrictions that apply to its use for certain operations.

1.3.1. General information.

This PLEXTALK device is designed to help you record and reproduce sound works, to which you own the copyright, or where you have gained permission from the copyright owner or the rightful licenser.  Unless you own the copyright or have gained the appropriate permission from the copyright owner or the rightful licenser, your unauthorized recording, reproduction or distribution thereof, may result in severe civil and criminal penalties under copyright laws and international copyright treaties. If you are uncertain about your rights, contact your legal advisor.

Under no circumstances will Shinano Kenshi be responsible for the consequences of any illegal copying performed using this PLEXTALK recorder.

1.3.2. Music CDs.

Copies of Music CDs are for individual enjoyment and, under the Copyright Act, cannot be used otherwise, without the authorization of the copyright holder.

2. Getting started.

2.1. Terminology and conventions used.

This section details the terminology and conventions used throughout this guide.  

The details are as follows:

· WARNING:  A warning notice denotes a hazard that could result in personal injury.

· CAUTION:  A caution notice denotes a hazard that could damage or destroy the PLEXTALK product, destroy the recording media, destroy or affect your data or computer configuration.

· Note:  A note is used to give additional important information about a feature that has not already been mentioned.  

· SD Card:  The PLEXTALK is able to use both "SD" and "SDHC" cards.  In this User's Guide, both cards are called "SD cards".

2.2. What you get in the box.

Below is a list of all the items you should find in the box with your PLEXTALK.  

Please make sure that all items are included. If there are any missing items, please contact your local agent.

· 1 PLEXTALK Pocket

· 1 SD Card 2GB (Note:  The SD card will come already inserted in the unit.)

· 1 AC adaptor

· 1 Rechargeable battery

· 1 Set of stereo headphones

· 1 USB cable (long) for connecting to a computer

· 1 USB cable (short) for connecting to a USB peripheral

· 1 Pouch

· 1 User's Guide CD

· 1 Quick Reference guide

· 1 Safety Instruction guide

Note:  It is strongly recommended that you retain all packaging, in case you need to return your PLEXTALK for any reason in the future.

2.3. Description of the unit.

This section describes the physical look of the PLEXTALK and the layout of the keys and other controls to enable you to locate them when referred to in later chapters. The names and main purposes are also detailed here.

Whilst holding the unit carefully, take a few moments to locate the front of the unit. The front has the microphone and speaker grill at one end and 23 keys arranged across the rest of the surface.  Turn the unit so that you are holding it with the keys uppermost and with the grill away from you.  The PLEXTALK should now be set up in its normal operating position. Keep the unit in this position as the physical layout is described.

2.3.1. Front face.

The grill that you used to orientate the unit is where the internal speaker and the built in microphone are located. The speaker is on the right and the microphone is in the centre.

Just below the grill, in the centre, there are 2 LEDs. The one on the left is the Power LED.  

It should be off when the unit is turned off. It becomes orange while booting up.  It shows green when the unit is on. The one on the right is the Battery LED and indicates the charging status. It is off when the PLEXTALK is operating on battery power. During charging Phase 1, it will show a red light. During charging Phase 2, it will alternate between red and green, and when the battery is fully charged, it turns solid green.  

Below the grill, to the left and right of the LEDs, there are two oval buttons. The key on the left is coloured red and is the Record key, used for DAISY recording, Voice memo recording and for recording Voice Bookmarks. The key on the right is coloured green and is the Power key, used for power on/off and for setting the sleep timer. In the centre of this key there are two prominent tactile dots.

Below the Record key and the Power key, there are 4 function keys - 2 on the left and 2 on the right of the unit. The upper left key is the GoTo key, used for page and heading navigation. The lower left key is the Bookmark key, used for bookmark navigation, insertion and deletion. The upper right key is the Menu key, used to access the main menu. Press and hold this key to turn on or off the Key Describer mode. The lower right key is the Title key, used for changing the title or media. 

In between the 4 function keys, at the centre of the unit, is the circular Play/Stop key surrounded by four arrow keys - Up, Down, Left and Right.  The Play/Stop key is slightly indented and is coloured blue. It is used to start/stop playback and acts as the Enter key for some operations. The Up key is used for moving up in the list of navigation options. It is also used for increasing the speed and tone settings or to cancel an operation. The Down key is used for moving down in the list of navigation options. It is also used for decreasing the speed and tone settings or to act as the Enter key for some operations.

The Left key is used for moving left in menus or to move back in a title. Press and hold the key for fast rewind. The Right key is used for moving right in menus or to move forward in a title.  Press and hold the key for fast forward.

Below the function and arrow keys are a further 12 small circular keys, arranged in 4 rows of 3 keys. These are generally called the Number Pad. The Number Pad is laid out exactly like a push button telephone keypad. The first row has numbers 1, 2 and 3. The second row has numbers 4, 5 and 6. The third row has numbers 7, 8 and 9 and the fourth row has the Star key, Number 0 key and the Pound/Hash key. The number 5 key has a small raised dot on it to help you locate it.

Some of the keys have secondary functions and these are listed below:

· Number 1 = Change speed or tone in combination with the Up or Down key

· Number 5 = Information key.  Press once for a list of information.  Press repeatedly to step quickly through the list of information.  Press and hold to get the current date and time.

· Number 9 = Backup

· Number 0 = Help.  Press once to enter or exit the built in help file.

· Star = Undo/Cancel

· Pound/Hash = Redo/Enter

Throughout this guide, the term "Enter key" refers to the Play/Stop key, the Down key or the Pound/Hash key. The term "Cancel key" refers to the Up key or the Star key.

2.3.2. Top edge.

On the top edge of the PLEXTALK, there are 2 sockets - one towards the left side and one in the centre. The socket on the left is for an external microphone and line in and the centre socket is for headphones. All sockets take 3.5mm diameter stereo mini jack plugs.

In the top left corner, there is a small hole which is designed for fitting a carrying strap if required.

2.3.3. Left edge.

On the left edge of the unit, roughly level with the Record key, there is a slot for the SD card.

2.3.4. Right edge.

On the right edge of the unit, there are 2 switches. The upper switch is for increasing and decreasing the volume. Pushing the switch towards the top of the unit, increases the volume. Pulling the switch towards the bottom of the unit, decreases the volume.

The lower switch is the Key Lock key. This is to prevent accidental activation of any other key, including the Power key. Pushing the switch toward the top of the unit locks all keys.  

Pulling the switch toward the bottom of the unit releases the lock. 

2.3.5. Bottom edge.

There are 2 items on the bottom edge of the unit. On the left is a small round hole in a square-shaped housing. This is the DC input terminal. On the right is a slightly smaller rectangular slot. This is the USB socket.

2.3.6. Back face.

Carefully turn over your PLEXTALK and hold it face down in one hand, keeping the end with the grill away from you.

In the centre of the Back face, there are 2 raised ridges. These are at the top end of the battery cover. Towards the bottom edge of the unit, but still on the Back face, there is a rectangular button which is flush with the surface. This button is at the bottom edge of the battery cover and is used to release the cover.

To remove the battery cover, press down firmly on the rectangular button with the tip of a finger. At the same time, use the thumb of your other hand, on the 2 ridges, to slide the battery cover towards the bottom edge of the unit. It should release fairly easily but you may need to practice a couple of times to master the technique.

2.4. Inserting the battery.

In order to insert or replace the battery, do the following:

· Turn off the power and remove the power adapter.

· Remove the battery cover, as described in the previous section, and lay the PLEXTALK and battery cover to one side.

· Locate the new battery and orientate it so that the one edge, which is completely smooth, is facing away from you and the edge, which has 2 obvious projections on it, is facing to your right. These two projections are the battery terminals.

· Assuming that you have done this correctly, there should be 2 further projections, one either side of the bottom left corner. Note these as these are useful when trying to remove the battery later.

· Pick up the PLEXTALK, ensuring that the front face is downwards and the battery housing is towards you. Carefully insert the battery, keeping the two battery terminals on the leading edge. This should mean that you are introducing the battery from left to right across the battery housing.

· Push the battery terminals gently into place and then press down gently on the left side of the battery and it should slip easily into the housing. Do not use force to insert the battery as this should not be necessary.

· Place the battery cover over the battery housing, around 5 millimeters from its normal position. While pressing the battery cover towards the battery housing, gently slide the battery cover back up towards the top of the unit until the rectangular button pops up into the locked position.

WARNING:  You should only use the recommended battery with this unit. Failure to do so may cause an explosion or fire.

Note:  To remove an old battery, gently lift the bottom left corner of the battery using a fingernail under one of the 2 projections that are on the bottom left corner and then lift it carefully out of the housing. This takes a bit of practice but is simple once you understand the technique.

2.5. Charging PLEXTALK.

In order to charge the internal battery, you need to connect the PLEXTALK to the mains supply or to a computer using the supplied USB cable.  Do this as follows:

· Place the PLEXTALK face up in its normal operating position.

· To charge the battery using mains power, plug the supplied AC adapter into the power input terminal on the bottom left edge of the unit. Then plug the power adaptor into the mains socket. Charging will start.

· To charge the battery using the USB connection, plug the smaller end of the long USB cable into the USB socket on the bottom right edge of the PLEXTALK. Then plug the wider end of the cable into a spare USB socket on a computer. Ensure that the computer is turned on and charging will start.

CAUTION:  Never use force when connecting the USB cable as you may damage the cable in the process.

Note: The charging via a USB cable works during power off.

2.6. Maintenance and battery care.

2.6.1. Unit care.

Carefully wipe any dust and dirt from the PLEXTALK with a soft cloth that has been slightly dampened with a diluted, neutral detergent, then wipe dry with a cloth.

WARNING:  Always turn the power off and disconnect PLEXTALK from the mains supply before cleaning.

2.6.2. Battery care.

In order that you get the most out of your rechargeable battery, you should take careful note of the following points:

· The battery will normally take 4 hours to fully charge. However, when using your PLEXTALK for the first time, or after one month of storage, you should connect to a mains power supply for at least 24 hours. 

· If the PLEXTALK is not to be used for over a month, remove the battery and store it in a cool place with low humidity.

· The battery life depends on how frequently it is used and recharged. Its capacity to store charge reduces over time. When the usable time following a charge becomes too short for practical use, replace the battery with a new one. These can be purchased from your local agent. Your agent should also be able to give you advice about recycling and disposal of used batteries.

· The usable time may be affected by the ambient temperature and working environment.

In order to maintain battery performance, the following procedure should be carried out every six months:

· Fully charge the battery.

· Use on battery power until dead.

· Recharge.

2.6.3. Crossed-out dustbin mark.

Symbol mark called "crossed-out dustbin mark" is printed on the surface of the battery and on the inside of the battery housing.

[image: image4.png]



Note:  This symbol mark is valid only for EU countries. 

This symbol mark is according to the directive 2006/66/EC.  This symbol means that batteries and accumulators, at their end-of-life, should be disposed of separately from regular municipal waste. If a chemical symbol is printed beneath the symbol shown above, this chemical symbol means that the battery or accumulator contains a heavy metal at a certain concentration listed below.

Hg: mercury (0.0005%)

Cd: cadmium (0.002%)

Pb: lead (0.004%)

These substances may cause serious hazards to human and the global environment if not cared in an appropriate manner.  In the European Union there are separate collection systems for used batteries and accumulators. Please dispose of batteries and accumulators properly at your local community waste collection or recycling center. Our product is designed and manufactured with battery and accumulator components which can be recycled or reused.

2.7. Turning PLEXTALK on and off.

To turn the unit on or off, do the following:

· Press and hold down the Power key to turn the unit on. When the power is on, the left LED will turn orange and then green.  

· Press and hold the Power key to turn into power saving mode. Keep pressing and holding another 3 seconds to turn the power off.

Note: When PLEXTALK is in the power saving mode, PLEXTALK resumes by pressing any key. Then time will be announced.

Note: When PLEXTALK is in the power saving mode, it drains the battery gradually.

Note: The booting up process should take approximately 40 seconds after turning the power off or after the battery has been replaced.

Note:  When you turn off the power, if you do not hold the key down long enough, the Sleep Timer will be activated.

Note:  If Key Lock switch is on, the Power key will not function.

2.8. Key Describer function.

As its name implies, the Key Describer function allows the user to get a description of any key on the PLEXTALK. 

· Turn on the Key Describer by pressing and holding down the Menu key for around 5 seconds. Now press any key on the unit and a description of its function will be spoken.  

· Turn off the Key Describer function by pressing and holding down the Menu key for around 5 seconds. 

2.9. Key Lock function.

Key Lock can be used to lock the keys and controls so that they can't be accidentally operated. 

· To apply the lock, push the Key Lock switch towards the top of the unit. 

· To unlock the PLEXTALK, pull the Key Lock switch towards the bottom of the unit. 

2.10. Inserting and ejecting SD card

To insert the SD card, do the following:

· Turn off the power.

· Hold the SD card so that the cut off corner may be located at the front right corner. In other words, the cut off corner should be nearest to the bottom of the unit.

· Insert the SD card into the card slot on the left edge of the unit.  

· Press the card all the way into the slot ensuring that it engages properly, with the outer edge flush with the body of the PLEXTALK. Do not use excessive force when doing so.

To eject the SD card, do the following:

· Turn off the power.

· Push the SD card inwards and then release it. The card should pop out leaving the outside edge of the card protruding.

· Carefully grip the card and pull it out of the slot.

CAUTION:  The PLEXTALK's SD card slot is only compatible with SD or SDHC cards.  

Inserting other memory card such as a Memory Stick or MultiMedia Card, will cause a malfunction and should not be used.

Note:  SD cards have a locking switch which enables you to protect your data if you want.  

This is located on the left side of the card away from the cut off corner.  If the switch is towards the leading edge of the card, you will be able to record and delete titles. If the switch is away from the leading edge, your data will be protected.

3.  Introduction to the menu system.

A number of the features of the PLEXTALK are accessed through the built in menu system. This chapter aims to give you an understanding of how the menu is structured and how you access it.  

3.1. Main menu.

At the highest level there is the Main Menu which has 7 items.  These are:

· Edit - allows simple editing of a DAISY recording.

· Recording Settings - allows you to select from a variety of recording settings or to set your own customized settings.

· Playback setting – allows you to repeat setting and to change the method of reading name of album and track.

· Bookmark - allows you to specify how Voice Bookmarks play and allows you to delete all bookmarks.

· Voice Memo - keep short Voice Memos.

· Podcast – allows you to update the episode list and download episodes.

· Media Management - allows you to get media information, build recorded DAISY titles, erase SD card data and so on.

· System settings - allows you to get system information and to control the way certain features operate or play.

3.2. Accessing and navigating the menu.

To navigate around the menu, use the following methods:

· Access the menu by pressing the Menu key once.

· To select a different option on the same level of the menu, press either the Left key to move to the left or the Right key to move to the right. As you move to a new menu option, it will be spoken.

· On all menus, if you continue in the same direction, you will eventually return to the starting point as the menus are designed to loop round. 

· To select a menu option, press the Down key or the Enter key. Assuming that there is a sub menu, and it is accessible at that time.   This will take you down one level in the menu structure and leave you on the relevant sub menu.

· To move up one level in the menu, press the Up key.  

· To exit immediately from the menu system, from any level in the structure, press the Menu key. 

3.3. Shortcut for the menu selection.

You can register and select a specific menu item with Number Pad. To register the specific menu item, do the following:

· Press the Menu key and go to the menu item you want to register.

· While pressing and holding the Menu key, press and hold your favorite Number Pad until confirmation sound. Then the PLEXTALK will register the specific menu item with this number.

To select the specific menu item directly, do the following:

· While pressing and holding the Menu key, press and release the registered Number Pad. Then you can select the specific menu item.

4.  Setting up your PLEXTALK.

4.1. Selecting your preferred synthetic voice.

You are given the choice of two synthetic voices for the text to speech voice.  To select your preferred text to speech voice, do the following:

· Press the Menu key once to access the Main Menu.

· Use the Left or Right key to go to the "System settings" option and select it by pressing the Enter key.

· Use the Left or Right key to go to the "Select the text to speech voice" option and select it by pressing the Enter key.

· Use the Left or Right key to select your preferred voice and confirm it by pressing the Enter key. PLEXTALK will make your selection the new default setting and will exit the menu system.

Note:  If you change languages, the PLEXTALK will restart.

4.2. Setting the text to speech pitch.

To adjust the pitch of the built in text to speech voice, do the following:

· Press the Menu key once to access the Main Menu.

· Use the Left or Right key to go to the "System settings" option and select it by pressing the Enter key.

· Use the Left or Right key to go to the "Set the text to speech pitch" option and select it by pressing the Enter key.

· Use the Left or Right key to choose your preferred pitch setting, in the range from -5 to +5, with 0 being the normal setting, and select it by pressing the Enter key.

4.3. Setting the date and time.

To set the date and time for the internal clock, do the following:

· Press the Menu key once to access the Main Menu.

· Use the Left or Right key to go to the "System settings" option and select it by pressing the Enter key.

· Select the "Set the system date and time" option from the menu and press Enter.

· Use the Number Pad to enter the current year with four digits and press Enter, or just press Enter to accept the current setting.

· Use the Number Pad to enter the current month with one or two digits and then press Enter, or just press Enter to accept the current setting. 

· Use the Number Pad to enter the current day with one or two digits and then press Enter, or just press the Enter key to accept the current setting. 

· Use the Number Pad to enter the current time with four digits and press Enter.  Use leading zeros where necessary.  

· If you wish to select the AM or PM setting, before pressing the Enter key, use the Left or Right keys to make your selection.

· By way of confirmation, the date and time that you have just entered will be spoken back to you.

Note:  When entering incorrect number, press the Cancel key to clear the entered number.

Note:  To check the current date and time, press and hold down the Number 5 key.

4.4. Selecting a preferred waiting sound.

When the PLEXTALK is busy, such as when it is accessing a media, it plays a sound to indicate that it is carrying out that action. To select your preferred waiting sound, do the following:

· Press the Menu key once to access the Main Menu.

· Use the Left or Right key to go to the "System settings" option and select it by pressing the Enter key.

· Use the Left or Right key to go to the "Select the waiting sound" option and select it by pressing the Enter key.  

· Use the Left or Right key to listen to the available options and press the Enter key when you are on the waiting sound you prefer.

4.5. Selecting the Key Press Confirmation Sound.

The PLEXTALK gives you two choices of Key Press Confirmation Sound on or off. To select the Key Press Confirmation Sound, do the following:

· Press the Menu key once to access the Main Menu.

· Use the Left or Right key to go to the "System settings" option and select it by pressing the Enter key.

· Use the Left or Right key to go to the "Select the Key Press Confirmation Sound" option and select it by pressing the Enter key. 

· Use the Left or Right key to select either "Key Press Confirmation Sound On" or "Key Press Confirmation Sound Off", and select it by pressing the Enter key. The PLEXTALK will make your selection the new default setting and will restart.
5.  Power management features.

5.1. Battery low warning.

When the PLEXTALK is operating on battery power and the power level drops to a level where it needs to be recharged, the PLEXTALK will say "Low battery."  Then player will shut down in one minute. If you wish to continue to use the PLEXTALK, you need to plug in the AC adaptor. 

5.2. Automatic power off function.

In order to save battery power, the PLEXTALK will automatically turn into power saving mode after 2 minutes of inactivity even if the battery level is still OK.

When the PLEXTALK is connecting to the mains supply or USB device, the PLEXTALK automatically turns into power saving mode after 15 minutes of inactivity.

5.3. Compulsory power off function.

If for any reason, the PLEXTALK will not shut down or you are not able to navigate with any keys, you can try using the compulsory power off function, as an alternative to removing the battery.

To use the compulsory power off feature, press and hold down the Power key for more than 10 seconds, then release the Power key. The PLEXTALK should shut down.

Note:  If this method does not work, and your PLEXTALK is still powered on, you should plug off the AC adaptor, remove the battery and insert the battery again.

5.4. Sleep timer function.

The Sleep Timer feature enables you to specify an amount of time, after which, the PLEXTALK will shut down. 

5.4.1. Setting the sleep timer.

To set the sleep timer, do the following:

· Quickly press and release the Power key. The PLEXTALK will say, "Sleep timer.  15 minutes.  Set."

· Press and release the Power key again to increase the time for shutting down. The Sleep Timer setting will increase as 30 minutes, 45 minutes, 1 hour, 1 hour 30 minutes, and 2 hours for each press.

5.4.2. Cancelling the sleep timer.

To cancel the sleep timer, do the following:

· Quickly press and release the Power key. The PLEXTALK will tell you that the sleep timer is currently on and the number of minutes remaining before shut down.

· Keep pressing the Power key repeatedly until the PLEXTALK says "Off".

6.  Playing features common to all formats.

The PLEXTALK is able to play following formats.

· DAISY book

· Music:  MP3, Wave, WMA, AMR-WB+ and Ogg Vorbis.  

· Text files: plain text format, Microsoft Word .doc, .docx format or HTML format.

· Audio books: Audible, MP3, Wave, WMA, AMR-WB+ and Ogg Vorbis.

· Podcast

· Web radio

Note:  If a CD has multiple kinds of content, for example, DAISY files and Audio files, the PLEXTALK will recognize one kind of content only.

Note:  Word files and WMA files on external USB CD/DVD media are unable to play.

6.1.  Start and stop playback.

To start or stop playback, do the following:

· To start playback, press the Play/Stop key. The PLEXTALK will start playback from the position where it was stopped previously.

· To stop playback, press the Play/Stop key.

6.2.  Fast forward or fast rewind.

The fast rewind and fast forward features use the Left and Right keys either side of the Play/Stop key.

· To fast rewind, press and hold down the Left key.  Playback will start when you release the key.

· To fast forward, press and hold down the Right key.  Playback will start when you release the key.

When you play a DAISY book with audio or Audio file, as you hold down the Left and Right key, the fast rewind or fast forward will skip 5 seconds, 10 seconds, 20 seconds, 30 seconds, 1 minute, 2 minutes, 3 minutes and so on. If the key is released before the "1 minute" option is spoken, playback will start at the place the key is released.  If the key is released after this point, the fast rewind or fast forward will continue until the Play/Stop key is pressed to stop the process.  

Note: There are no functions of fast-forward and fast-rewind for playing web radio.

6.3.  Changing the playback volume.

The volume can be set on a scale from 0 to 25, in steps of 1, with 0 being silent and 25 being the Highest possible setting. The volume of the guide speech is also adjusted in conjunction with the playback volume.

· To increase the playback volume, either push the Volume switch up repeatedly, increasing the level by 1 setting each time, or push and hold the Volume switch up until the volume reaches the desired level.

· To decrease the playback volume, either pull the Volume switch down repeatedly, decreasing the level by 1 setting each time, or pull and hold the Volume switch down until the volume reaches the desired level.

6.4.  Changing the playback speed.

There are 11 speed settings ranging from minus 2 to plus 8, which can be adjusted 1 level at a time. The normal speed is at the 0 position. Minus 2 is half of normal speed, plus 4 is double speed and plus 8 is triple speed.

· To increase the playback speed, press the Number 1 key once and then press the Up key.  

· To decrease the playback speed, press the Number 1 key once and then press the Down key. 

Note:  Pressing and holding the Up or Down key, will cause the speed setting to change rapidly in the selected direction. 

Note: Playback speed of web radio cannot be changed.

6.5.  Changing the audio tone.

There are 13 tone settings ranging from minus 6 to plus 6, which can be adjusted 1 level at a time. 

· To increase the tone, press the Number 1 key twice and then press the Up key.

· To decrease the tone, press the Number 1 key twice and then press the Down key.

Note:  Pressing and holding the Up or Down key, will cause the tone setting to change rapidly in the relevant direction.

6.6.  Changing the guide volume.

The volume of the guide speech can be changed. There are 11 volume settings ranging from minus 5 to plus 5, which can be adjusted 1 level at a time. To set the guide volume, do the following:

· Press the Number 1 key three times.

· Press the Up key to increase the guide volume or press the Down key to decrease the guide volume.

Note: When the playback volume is changed, the volume of the guide speech is also changed proportionally corresponding to the playback volume.

6.7.  Changing the guide speed.

The speed of the guide speech can be changed independently from the playback speed.

There are 11 speed settings ranging from minus 2 to plus 8, which can be adjusted 1 level at a time. To set the guide speed independently, do the following:

· Press the Number 1 key four times.

· Press the Up key to increase the guide speed or press the Down key to decrease the guide speed.

6.8.  Accessing the Navigation menu.

One of the most useful features of the PLEXTALK is the ability to choose how you want to navigate. To use the Navigation Menu, do the following:

· Use the Up or Down keys to move upwards or downwards in the navigation menu. The items you hear spoken are the options available to you from the Navigation Menu at that time.

· Having selected your preferred navigation method, you can now use your Left or Right key to move backwards or forwards at your chosen level of navigation.  

· For example, if you go to the "Level 1" option with your Up or Down key, and then use your Left or Right key, you will be able to move backwards or forwards to the next Level 1 heading.

6.9.  Selecting the media and title.

At times you may wish to switch from using an SD card to the USB media or vice versa.  You can do this as follows:

· Press the Title key repeatedly until your desired media is announced. PLEXTALK will move sequentially through the 3 available options, "SD card", "USB" and "Voice Memo". 

· After your desired media is announced, use the Left or Right key to move through the available contents on that media.

· When you are on the item you wish to play, press the Play/Stop key and playback will start at the last playing position in that item.

6.10. Title navigation.

In situations where there are multiple items on the SD card or USB media, you may wish to switch from the item you are currently reading to different one.  

You can play your favorite title by using "category" "folder" "title" and "file" levels.

In the "category" level, there are "DAISY book", "Music", "Text", "Audio book", "Podcast" and "Web radio". 

To search the title by title navigaton, do the following:

· Press the Title key repeatedly until you hear the media you want to go to.

· Press the Up or Down key to select the level of title navigation from "category" "folder" "title" or "file".

· Press the Left or Right key to move sequentially through the selected level.

· When selecting the "folder" or "category" level, if this folder or category contains the title or the album you wish to play, press the Down key once or twice to switch to the "title" level.  The PLEXTALK will say "Select title". Use the Left or Right key to search the title you wish to play.

· When selecting the "title" or "file" level, you are able to select a favorite title or file.

· If you wish to play the selected title, press the Play/Stop key.  Playback will start from the position where it was stopped previously.

How to seek for your favorite music at music category

You can select playback range from  “Tracks”, “Albums”, “Folders”, or  “All music” under music category. To play music in track-wise, select your favorite track at  “File” level and then press enter key. To play music in album-wise, select your favorite album at “Title” level and then press enter key. Also, to play music in folder-wise, select your favorite folder at “Folder” level and then press enter key. After selecting a folder at “Folder” level and pressing the enter key, all albums in the folder become available to play. To play all music, there are two ways. Use the Up or Down key to select "Category" level, then use the Left or Right key to select "Music" and press the Enter key. Or, use the Up or Down key to select "Title" or  "Folder" level, then use the Left or Right key to select "All music" and press the Enter key.

You can use “Repeat” or “Shuffle” playback in the range range which you pressed the Enter key.

Note: Navigation by file is available in "Music", "Audio book" and "Podcast", but not available in other categories.

Note: Audio book from Audible cannot be available file navigation.

6.11. Go to a title directly

To go to a title directly, do the following:

· Press the Title key repeatedly until you hear the media you want to go to.

· Press the Up or Down key to select the “category” level.

· Use the Left or Right key to select favourite category. 

· Type in the number of the title you wish to go to, and press the Enter key.

· Press the Enter key again to play the title.

6.12. Switching sorting order of titles

When selecting a title, you can change the sorting order of the titles. There are three types of the sorting order that are "All titles list", "Recent titles list" and "New titles list ". 

· All titles list: All files and folders are sorted by their names.

· Recent titles list: The titles are sorted by date played.

· New titles list: The titles are sorted by time stamp of the file.

To play the recently played title, do the following:

· Press the Title key once.

· Press the GoTo key repeatedly to select the "Recent titles list".

· Use the Up or Down key to select either “Category” or “Title” and use the Left or Right key to select the title you prefer. And then press the Enter key to play.

· Or using the Number Pad, enter the number of the title you prefer and press the Enter key to select. Then press the Enter key once more to start playback. 

Note: PLEXTALK memorize up to 10 titles in each Category.

6.13. Getting information about media and title.

To get a variety of information about the current media and title, do the following:

· Press the Menu key once to access the Main Menu.

· Use the Left or Right key to go to the "Media Management" option and select it by pressing the Enter key.

· Use the Left or Right key to go to the "Media title Information" option and select it by pressing the Enter key.

· PLEXTALK will announce the information on the current media and title.

· When you have noted the relevant information, press the Enter key to exit the menu system.

6.14. Getting information on the current status.

You can get a range of information on the current status, about heading, page, bookmark and so on. Whether PLEXTALK is playing or not, do the following:

· Press the Number 5 key once. 

· A range of information will be spoken. 

· If you do not wish to listen to all the information, and wish to step through individual items more quickly, you can do this by repeatedly pressing the Number 5 key.

· If you have heard what you want to hear, you can stop the spoken information by pressing the Enter key. 

· Press and hold down the Number 5 key to hear the current date and time.

6.15. Selecting a playback mode.

To select the shuffle and repeat settings from playback modes, do the following:

· Press the Menu key once to access the Main Menu.

· Use the Left or Right key to go to the "Playback Settings" option and select it by pressing the Enter key. 

· Use the Left or Right key to go to the "Shuffle and repeat settings" option and select it by pressing the Enter key.

· Use the Left or Right key to go to your preferred playing mode and select it by pressing the Enter key.

6.16. Using bookmarks.

6.16.1. Types of bookmark.

There are 3 types of bookmark you can use in PLEXTALK as follows:

· Numbered Bookmarks; You can insert up to 10000 Numbered Bookmarks. Each bookmark is assigned a unique number in the range 1 to 65000. 

· Voice Bookmarks; These are Numbered Bookmarks that have accompanying voice annotations. Voice Bookmarks use memory. The total memory available gives about 30 minutes of recording time. If you try to exceed this allocation, the oldest bookmark that has not been used recently is automatically deleted before the new one is created.

· Resume Bookmarks; For every title, album or text file, the current playing location will be stored so playback can be resumed from that location the next time the title, album or text file is used. Resume Bookmarks are inserted automatically by PLEXTALK when you move to a different title, album or file, or when you shut down the player. No action is required from you.

Note:  PLEXTALK will remember the Resume Bookmark for up to 1000 titles, albums or text files.

6.16.2. Inserting bookmarks.

To insert a Numbered Bookmark, whether PLEXTALK is playing or not, do the following:

· Press the Bookmark key twice. 

· Using the Number Pad, enter the number you wish to assign to this bookmark. If you allow PLEXTALK to assign a number, do not enter the number or enter the number “0”.

· Press the Enter key to confirm or the Cancel key to clear your entry.

Note:  When you allow PLEXTALK to assign the number, PLEXTALK selects the smallest number from the unused numbers.

To insert a Voice Bookmark, whether PLEXTALK is playing or not, do the following:

· Press the Bookmark key twice. 

· Using the Number Pad, enter the number you wish to assign to this Voice Bookmark. If you allow PLEXTALK to assign a number, do not enter the number or enter the number “0”.

· Press the Record key once to confirm or the Cancel key to clear your entry.

· Press and hold down the Record key and speak your voice annotation.

· Release the Record key when you have finished. PLEXTALK will confirm that the Voice Bookmark has been set.

6.16.3. Navigating to bookmarks.

To jump to a Numbered Bookmark or a Voice Bookmark, whether PLEXTALK is playing or not, do the following:

· Press the Bookmark key once. 

· Using the Number Pad, enter the number of the bookmark you wish to go to.

· Press the Enter key to confirm or the Cancel key to clear your entry.

· When a valid bookmark number has been entered, the PLEXTALK will go to the location of the bookmark and playback will start automatically.

Once you have entered bookmarks into a title, album or file, a "Bookmark" option will become available on the Navigation Menu. In order to navigate to the next bookmark with a lower or higher number, do the following:

· Use the Up or Down key to go to the "Bookmark" option on the Navigation Menu.

· Press the Left key to move to the next bookmark with a lower number. Playback will start automatically. 

· Press the Right key to move to the next bookmark with a higher number. Playback will start automatically. 

6.16.4. Clearing bookmarks.

To clear a Numbered Bookmark or a Voice Bookmark, the procedure is identical, as follows:

· Press the Bookmark key 3 times.

· Using the Number Pad, enter the number of the bookmark you wish to remove.

· Press the Enter key to confirm or the Cancel key to clear your entry.

· When you confirm the action, PLEXTALK will remove the specified bookmark from the current title or album and will confirm this with a spoken message.

If you wish to clear all bookmarks in the current DAISY title or Text file, or clear all bookmarks in all albums, do the following:

· Press the Menu key to access the Main Menu.

· Use the Left or Right key to go to the "Bookmark" option and select it by pressing the Enter key.

· Use the Left or Right key to go to the "Clear all bookmarks in current title" option and select it by pressing the Enter key.

· PLEXTALK will ask you to confirm this action.  If you wish to proceed, press the Enter key and PLEXTALK will say "Deleted". 

Note:  The procedure will only clear bookmarks in the current DAISY title or Text file and will not affect bookmarks in other titles or files.  

6.16.5. Turning on and off Voice Bookmark speech.

When you are using Voice Bookmarks, you are able to select whether you want to hear them spoken or not during playback. To turn Voice Bookmarks On or Off, do the following:

· Press the Menu key to access the Main Menu.

· Use the Left or Right key to go to the "Bookmark" option and select it by pressing the Enter key.

· Use the Left or Right key to go to the "Playback setting for Voice Bookmark" option and select it by pressing the Enter key.

· Use the Left or Right key to select either "Voice Bookmark on" or "Voice Bookmark off" and press the Enter key to confirm your choice.

6.17. Understanding how PLEXTALK groups files and folders.

When PLEXTALK accesses the SD card or USB media, it automatically assesses the contents and groups items into "DAISY title", “Recordings”, "Music", "Text", "Audio book", "Podcast" and "Web Radio", regardless of where these items are stored on the media. When you subsequently navigate through the titles, they will be already ordered for you regardless of where they are physically located on the media.

For example, if you have 3 Text Files in the root folder and 2 more in a sub folder, PLEXTALK will view these as 5 Text Files stored in 2 folders.

If there are multiple types of contents such as DAISY titles, Audio Files or Text Files, PLEXTALK will automatically sort them by their type and name.

7.  Playing DAISY titles.

This chapter covers additional features that can be used when you are playing a DAISY title. 

7.1.  Heading navigation.

There are two ways of navigating to headings within a DAISY title. You can either go to headings direct or you can move through the available headings one at a time. To go direct to a specific heading, do the following:

· Press the GoTo key twice. 

· Using the Number Pad, type in the number of the heading you wish to go to.

· Press the Enter key to confirm or the Cancel key to clear your entry.

To navigate headings sequentially, using heading levels, do the following:

· Use the Up or Down key to go to your desired level of navigation, for example Level 1, Level 2 etc.

· Press the Left key to move backwards to the previous heading on the selected level.  

· Press the Right key to move forwards to the next heading on the selected level.  

Note:  When navigating in this way, PLEXTALK will stop at all headings that have a level the same as or higher than the level you have selected. For example, if you chose to navigate on Level 3, PLEXTALK will find all Level 1, Level 2 and Level 3 headings as you move backwards or forwards. 

7.2.  Moving to the beginning or end of the title.

To move to the beginning of the title, do the following:

· Press the GoTo key twice.

· Press the Enter key and the playing position will move to the start of the current title. 

To move to the end of the title, do the following:

· Press the GoTo key twice. 

· Press the Number 0 key followed by the Enter key and the playing position will move to the end of the current title. 

7.3.  Page navigation.

Navigating to pages is very similar to heading navigation. To go direct to a specified page, do the following:

· Press the GoTo key once. 

· Using the Number Pad, type in the number of the page you wish to go to.

· Press the Enter key to confirm or the Cancel key to clear your entry.

To navigate pages sequentially, do the following:

· Use the Up or Down key to go to the "Page" option on the Navigation Menu.

· Press the Left key to move backwards to the previous page.  

· Press the Right key to move forwards to the next page. 
Note:  If there are no page markers in the title, the "Page" option will not be listed on the Navigation Menu.

7.4.  Group navigation.

To move backwards or forwards a group at a time, do the following:

· Use the Up or Down key to go to the "Group" option on the Navigation Menu.

· Use the Left key to move backwards to the previous group item in the title.  

· Use the Right key to move forwards to the next group item in the title.  

Note:  If there are no group markers in the title, the "Group" option will not be listed on the Navigation Menu.

7.5.  Phrase navigation.

To move backwards or forwards one phrase, do the following:

· To jump back one phrase, press the Up or Down key to select the "Phrase" option on the Navigation Menu and then press the Left key.  

· To jump forwards one phrase, press the Up or Down key to select the "Phrase" option on the Navigation Menu and then press the Right key.  

7.6.  10 minutes skip, 30 seconds skip 

To skip by 10 minutes or 30 seconds at the Audio DAISY formats, do the followings:

· Use the Up or Down key to select 10 minutes or 30 seconds.

· To skip forward 10 minutes or 30 seconds, press the Right key.

· To skip backward 10 minutes or 30 seconds, press the Left key.

7.7.  Navigating by screen, paragraph, line, sentence, spell, word or character in case of DAISY text mode.

In addition to DAISY navigation options, DAISY text file will always give you the navigation options "Screen", "Paragraph", "Line", "Sentence", "Spell", "Word" and "Character".  

These options will automatically appear on the Navigation Menu when you access this type of file. To navigate by any of these options, the process is identical as follows:

· Use the Up or Down keys to select your preferred navigation method that is screen, paragraph, line, sentence, spell, word or character.

· Use the Left key to jump back in the file by the selected amount. 

· Use the Right key to jump forward in the file by the selected amount. 

Note:  "Screen" means 40 lines.

Note:  The spell navigation announces each word then spells the word.

7.8.  Navigating by percentage.

You can navigate by a percentage of the file. To navigate by percentage, do the following:

· Press the GoTo key three times.

· Type in a percentage between 0 percent and 100 percent, and press the Enter key.

· PLEXTALK will move the playback position to that position.

7.9.  Playback mode.

You can select from two playback modes when using a DAISY title. The available options are as follows:

· Standard play:  it plays through the title and stops when finished. 

· Repeat title: the current title will repeat continuously until playback is stopped.

7.10. Multimedia DAISY playback setting

Multimedia DAISY is DAISY format that is contained both audio and text files. Plextalk can be set which format is played if you play the Multimedia DAISY.

· Select Multimedia DAISY title.

· Press the menu key.

· Use the Left or Right key to go to “Playback settings” option and press the Enter key.

· Use the Left or Right key to go to “Multimedia DAISY playback setting” option and press the Enter key.

· Use the Left or Right key to select “Audio” or “Text” and press the Enter key.

8. Playing "Recordings"

DAISY titles in the "PlexRecording" folder are divided into the category called "Recordings".

Recorded title with this PLEXTALK is automatically stored into "PlexRecording" folder.

Playback method of "Recordings" is the same as DAISY title.

Note: If you back up "Recordings" title from SD card to USB device, it will be stored in the "PlexBackup" folder of the USB device. Therefore the backed up title is not categorized into "Recordings" but "DAISY title".

9.  Playing Audio Files or Music CDs.

This chapter covers features that are specific to playing Audio Files or Music CDs.  

PLEXTALK is able to play following types of audio file:

· MP3 Files; MPEG1 Layer 3 from 32kbps to 320kbps and MPEG2 Layer 3 from 16kbps to 160kbps.

· WAVE Files.

· WMA Files; PLEXTALK does not support DRM protected WMA files.

· AMR-WB+ Files.

· Ogg Vorbis Files; from 45 kbps to 320 kbps

CAUTION:  PLEXTALK may not be able to play a Music CD that is not compliant with the CD digital audio (CD-DA) standard, such as copy-protected Music CDs.

Note:  PLEXTALK cannot be used to record or edit Audio Files on either SD cards or USB media.

Note:  WMA file on external USB CD/DVD media are unable to play.

9.1.  Track navigation.

You can either go to tracks direct or you can move through the available tracks one at a time. To go direct to a specified track, do the following:

· Press the GoTo key once.

· Using the Number Pad, type in the number of the track you wish to go to.

· Press the Enter key to confirm or the Cancel key to clear your entry.

To navigate tracks sequentially, do the following:

· Use the Up or Down key to go to the "Track" option on the Navigation Menu.

· Press the Left key to move backwards to the previous track.  

· Press the Right key to move forwards to the next track.
Note:  When moving back a track at a time, if you allow the track to play for more than 3 seconds, the next time you move back a track, you will move to the start of the current track. If you move back a second time within 3 seconds, you will move to the previous track.

9.2.  Album navigation.

You can either go to albums direct or you can move through the available albums one at a time. To go direct to a specified album, do the following:

· Press the GoTo key twice. 

· Using the Number Pad, type in the number of the album you wish to go to.

· Press the Enter key to confirm or the Cancel key to clear your entry.

To navigate albums sequentially, do the following:

· Press the Up or Down key and select "Album" from the Navigation Menu.

· Press the Left key to move backwards to the previous album.  

· Press the Right key to move forwards to the next album.
9.3.  10 minutes skip, 30 seconds skip 

To skip by 10 minutes or 30 seconds, do the followings:

· Use the Up or Down key to select 10 minutes or 30 seconds.

· To skip forward 10 minutes or 30 seconds, press the Right key.

· To skip backward 10 minutes or 30 seconds, press the Left key.

9.4.  Playback mode.

When playing Audio Files, you can select a number of different playback modes. The available options are as follows:

· Standard play - It plays through the current album and stops when finished. 

· Repeat track - The current track will repeat continuously until playback is stopped.

· Repeat album - The current album will repeat continuously until playback is stopped.

· Repeat entire albums – Repeat all of the music in the range selected at title navigation.

· Shuffle repeat – Shuffle and repeat all of the music in the range selected at title navigation.

9.5.  Announcing albums and tracks by name or number.

When navigating albums or tracks, PLEXTALK announces albums and tracks by name or number. Either setting to read the name or the number can be selected. For this setting, do the following:

· Press the Menu key once to access the Main Menu.

· Use the Left or Right key to go to the "Playback Setting" option and select it by pressing the Enter key.  

· Use the Left or Right key to go to the "Reading audio file name" option and select it by pressing the Enter key. 

· Use the Left or Right key to select between "Read audio file name" and "Read audio track number", and select it by pressing the Enter key.

9.6.  About playing sequences.

This section gives you information about the playing sequence used for playing files and folders.

· Playback of files; PLEXTALK will playback the files in the order of their file names. 

· Playback of folders; If more than one folder exists on the SD card or USB media, the audio files will be played in the order of the folder names.

9.7.  Restrictions on Audio Files.

Restrictions on numbers of files and folders:

· The maximum number of files that can be stored in a folder is 1024.  Files in excess of 1024 are not recognized.

· The maximum number of files that can be stored in a media is 6000.  Files in excess of 6000 are not recognized.

· The maximum number of folders is 1024, including a root folder.  Folders in excess of 1024 are not recognized.

· PLEXTALK treats file folders as albums and recognizes up to 8 hierarchies including root.

· If a folder contains a DAISY book, PLEXTALK does not search the other contents under the folder.
Note:  When there are a large number of folders, PLEXTALK may take longer to read the SD card or USB information.

10.  Playing Text Files.

PLEXTALK is able to play the Text Files by using synthetic voices. PLEXTALK supports following types of text files:

· Plain text files

· Microsoft Word .doc, .docx files

· HTML files

Note:  Word file on external USB CD/DVD media are unable to play.

10.1.  Navigating by screen, paragraph, line, sentence, spell, word or character.

A Text File will always give you the navigation options "Screen", "Paragraph", "Line", "Sentence", "Spell", "Word" and "Character". These options will automatically appear on the Navigation Menu when you access this type of file. For Word and HTML files that have been coded with headings, you will be given an additional navigation option on the Navigation Menu. This will be called "Level".  To navigate by any of these options, the process is identical as follows:

· Use the Up or Down key to select your preferred navigation method that is screen, paragraph, line, sentence, spell, word or character.

· Use the Left key to jump back in the file by the selected amount. 

· Use the Right key to jump forward in the file by the selected amount. 

Note:  “Screen” means 40 lines.

Note:  The spell navigation announces each word then spells the word.

10.2.  Navigating by percentage.

In Text Files, you can navigate by a percentage of the file. To navigate by percentage, do the following:

· Press the GoTo key once.

· Type in a percentage between 0 percent and 100 percent, and press the Enter key.

· PLEXTALK will move the playback position to that position in the Text File.

10.3.  Playback mode.

When playing Text Files, you can select a preferred playback mode. The available options are as follows:

· Standard play; It plays through the title and stops when finished. 

· Repeat title; the current title will repeat continuously until playback is stopped.

11.  Playing Audio Book.

PLEXTALK is able to play the "Audio Book".  PLEXTALK supports following types of audio files:

· ".aa" files comes from Audible.

· MP3, WMA, WAVE, AMR, and Ogg files

Create “Audible" folder at the root of SD card and store the “.aa” files in this folder. If there are other types of audio files than “.aa” in the “Audible" folder, PLEXTALK does not recognize these files. PLEXTALK handles an “.aa” file as one title. Therefore “.aa” file can be available "title" navigation, but not available file navigation.

Meanwhile, you should create "PlexAudioBook" folder at the root of SD card and store the Audio Book files other than ".aa" in this folder.

All auidio files stored in "PlexAudioBook" folder are played as “Audio Book”, not as “Music”. 

PLEXTALK handles a folder in “PlexAudioBook” folder as one title.

Note: PLEXTALK recognizes “.aa” files in other folder than “Audible" folder. But it is recommended that you should store the “.aa” files in the “Audible" folder.

Note: PLEXTALK does not recognize “.aa” files in "PlexAudioBook” folder.

Navigations of audio book are exactly similar to Music files. Refer to section 9 "Playing Audio Files or Music CDs" for details.

Note: “.aa” audio file support heading and percent navigation.

Note: Two playback modes, "Standard play" and "Repeat title" options, can be selected from shuffle and repeat settings.

12. Podcast 

Podcast is on demand technology that you can download audio files on the various web sites such as music contents. Once adding the list of your favorite Podcasts to Plextalk, you can download the updated audio files in the series. Audio file delivered from Podcast hereinafter referred to as “Episode”. 

Note:  Refer to Section 20 "Network settings” for  instruction of network settings.

Note:  Only audio file Episodes can be downloaded to PLEXTALK.

12.1. Adding Podcast to PLEXTALK

To listen to Podcast, you have to add the list of your favorite podacst feeds by yourself.  

First, you need to create a file, which extension is “opml” (hereinafter “opml file”).  You can create this file with a computer and add the list of your favorite podcast feeds to this file. Then, create “PlexPodcast" folder at the root of SD card and store the opml file in this folder. There are two ways to create an opml file.

< Use an application available at PLEXTALK website >

You can download an application with the manual from PLEXTALK website and refer to the manual how to create an opml file. 

< Use an application such as iTunes >

You can export your favorite podcast feeds into opml file by using an application such as iTunes. To export an opml file, please refer to manual for the application.

12.2.  Listening to Podcast for the first time

To download and listen to a podcast feed for the first time, do the following:

· Insert the SD card saving an opml file into the Plextalk.

· Press the Title key repeatedly to select “SD card”.

· Use the Up key to select “Category” and then press the Left or Right key to select “Podcast” category.

· Use the Down key to select “Title” and then press the Left or Right key to select your favorite podcast feed. After the choice, press the Enter key.

· For the first download, you will be asked if you would like to update the episode list although there is no episode list yet. Start updating the episode list by pressing Enter key. 

· Select your favorite Episode from updated episode list and then press the Play/Stop key.  
· You will be asked if you would like to download the file. Press the Enter key to download it.

· Download takes longer. Once it’s completed, press the Play/Stop key to play.

Note: Refer to Section 12.7 "Notice during the download" for notice of downloading.

12.3. Playing Podcast

To play Episode that you have already downloaded the Podcast episode list, do the following:

· Press the Title key repeatedly to select SD card.

· Use the Left or Right key to select Podcast and press the Enter key.

· Use the Left or Right key to select your favorite Episode. If you would like to update the latest episode list, use “Update the episode list” option in the "Podcast" menu. Refer to section 12.5. "Getting the latest episode list."

· Press the Play/Stop key to play your favorite Episode. If the Episode has been already downloaded, the playback will start. If it’s still in the middle of downloading, the message will be announced. If it has not been downloaded yet, you will be asked if you would like to download it. Press the Enter key to proceed or Cancel key to cancel.

12.4. Navigating by Episode, 10 minutes or 30 seconds.

You can select your favorite Episode. Also, you can move to your favorite position of the Episode in every 10 minutes or every 30 seconds scale. 

· Use the Up or Down key to select a navigation method among “Episode”, “10 minutes”, or “30 seconds”.

· If selecting “Episode”, press the Right key to move forwards to the next Episode.  Press the Left key to move backwards to the previous Episode.

· If selecting “10 minutes” or “30 seconds”, press the Right key to move to 10 minutes or 30 seconds forwards. Use the Left key to move to 10 minutes or 30 seconds backwards. However, the destination always stays in an Episode. 

Note:  Fast-forward and fast-rewind are also limited within an Episode.

12.5. Getting the latest episode list

You can access to the website of the selected Podcast and get the latest episode list by using “Update the episode list” option from podcast menu.

· Press the Title key to select SD card and then press the Left or Right key to select a podcast feed that you want to update.

· Press the Menu key.

· Use the Left or Right key to select “Podcast” and press the Enter key.

· Use the Left or Right key to select “Update the episode list”, and press the Enter key.

· You will be asked if you would like to update the episode list.  Press the Enter key to start updating the episode list. After completing the update, you can select  Episode from the latest episode list.

12.6. Download all Episodes of the feed

You can download all episodes which are not downloaded yet.

· Press the Title key to select SD card and then press the Left or Right key to select a podcast feed that you would like to download.

· Press the Menu key.

· Use the Left or Right key to select “Podcast” and press the Enter key.

· Use the Left or Right key to select “Download all Episodes of the feed”, and press the Enter key.

· You will be asked if you would like to download the Episodes. Press the Enter key to start updating the episode list at first. And then all Episodes will be downloaded at a time.

Note: Refer to Section 12.7 "Notice during the download" for notice of downloading.

12.7. Notice during the download

· Updating episode list and download takes time. However, you can play other content while downloading. Select other title by the Title key.

· To check the download progress, press 5 key. You can confirm the number of Episodes left to download.

· When turning off the power during the download, the download will be canceled. To restart the download, you need to download the Episode again.

13. Web radio

You can listen to web radio on various web sites with PLEXTALK. You need to add the URL of your favorite web radio station to the playlist and access the site for streaming.

13.1. Adding web radio station to the playlist

To listen to web radio, you have to register web radio stations in advance. First, you need to use a computer and obtain a play list file including URL of the web radio station, which extension is "pls", "m3u", or "asx". Then, create "PlexWebradio" folder at the root of SD card and store the file in this folder. To add several web radio stations, store each of the playlists in this folder.

Note: You can find play list files at Internet site which introduces web radio stations such as “SHOUTcast”.  http://www.shoutcast.com 

13.2.  Listening to web radio

To listen to web radio, do the following:

· Select “SD card” by pressing Title key.

· Use the Up key to select “Category” and then press the Left or Right key to select “Web radio”.

· Use the Down key to select “Title” and then press the Left or Right key to select your favorite Web radio. After the choice, press the Play/Stop key to start streaming.

Note: There are no functions of fast-forward, fast-rewind, or level navigation for playing web radio station. 

Note: You cannot add bookmarks for web radio. After listening to other station or changing media, you cannot listen to the previous station continuously from where you stopped.

Note: Refer to the playable web radio format at Appendix A “Specification”.

14.  Recording a DAISY title.

PLEXTALK is able to record DAISY title to an SD card. This chapter shows you recording method and recording settings.

Note: PLEXTALK is also able to record Voice Bookmarks and Voice Memos to internal memory. Refer to section 6.16 "Using bookmarks" and section 17 “Using PLEXTALK's Voice Memo functions” for details of Voice Bookmarks and Voice Memos.

14.1.  Basic information about recording with PLEXTALK.

Before you start recording with PLEXTALK, you should be clear about several important points as detailed below.

· PLEXTALK cannot record directly onto any USB media.

· PLEXTALK always records in the DAISY format.

· Recorded DAISY titles are stored in the "PlexRecording" folder of SD card.

14.2.  Recording hardware.

Your PLEXTALK is supplied with the basic accessories that you require to make an initial voice recording. You may, however, wish to consider purchasing additional accessories to enhance the quality of your recordings and to enable you to record from external sources such as tape recorders, CD players, radios and other similar devices. The basic items that you require, or may wish to consider purchasing, are as follows:

· SD card; You can record to and edit information on an SD card.  

· External Microphone (Optional); Dynamic microphones and plug-in power stereo microphones can be used. Select any suitable microphone that uses a 3.5 millimetre diameter mini jack plug.

CAUTION:  When a condenser microphone is used, it must be plug-in power compatible and must be used correctly, otherwise it may be damaged.  Please read the microphone manual carefully before use.

· Audio cable (Optional); Assuming that other devices have a suitable connection, you can use an audio cable with 3.5 millimetre diameter stereo mini jack plugs to record direct from a cassette player, CD player, radio or other similar device.  

· Headphones; A set of stereo headphones with 3.5 millimetre diameter stereo mini jack plug is included.  

14.3.  Recording media.

This section highlights a number of issues that you should be aware of before recording to an SD card.

· With PLEXTALK, you can use an SD card to record and edit DAISY titles and you can erase and rerecord as many times as you want.

· PLEXTALK will work with SD cards with a range of capacities up to a maximum of 32 gigabytes. Before purchasing an additional SD card for use with PLEXTALK, you should ensure that it is compatible.  Contact your PLEXTALK dealer for advice.

CAUTION:  Whenever you erase an SD card using a personal computer, make sure you select "FAT 16" or "FAT 32".

14.4.  Recording states.

In terms of recording, PLEXTALK has three main states.  These are "Recording stand by", "Recording pause" and "Recording".

· In recording stand by, you can adjust the recording level and select a recording mode and sound quality setting. No recording is taking place at this time. The Power LED will turn orange and will flash on and off constantly.

· In recording pause, you can adjust the recording level but cannot alter the recording mode or sound quality settings. Again, no recording is taking place at this time. The Power LED will turn orange and will flash on and off constantly.

· During recording, audio input is being recorded to the SD card. You can adjust the recording level but cannot adjust the recording mode or sound quality setting. The Power LED will be orange and will be on constantly.

14.5.  Basic recording technique.

This section outlines the normal steps used in the recording process. The basic steps for recording with PLEXTALK are as follows:

· Insert the SD card you wish to record onto, ensuring that it is unlocked.

· Connect the appropriate hardware and select the input source you wish to use.

· Press the Record key once to go into recording stand by.

· Select a title you wish to record into.

· Set the recording level either manually or using the Auto Gain Control.

· Press the Record key again to start recording.

· Press the Record key again to pause recording at any time.

· Press the Record key to start recording again.

· Press the Play/Stop key to stop recording.

Note:  You can pause recording as often as you need to.

14.6.  Muting the guide speech when recording.

For best quality recordings, it is recommended that you mute the internal guide speech when you are recording through the built in microphone or through an external microphone. This is to avoid the guide speech being heard in your final recording.  

You can mute the guide speech in the following three ways:

· Turn down the guide volume to zero; This option works well for those that can see the recording status light but is more difficult for blind users who require the guide speech to confirm whether PLEXTALK is recording or paused.

· Plug in headphones; All output from PLEXTALK is heard through the headphones and no guide speech is recorded.  

· Turn off the guide speech during recording; PLEXTALK provides you with the ability to automatically turn off guide speech when you start recording. When you are recording, no guide speech is heard but guide speech returns automatically when you pause or stop recording.

Guide speech during recording can be turned off or on as follows:

· Press the Menu key once to access the Main Menu.

· Use the Left or Right key to go to the "Recording settings" option and select it by pressing the Enter key.

· Use the Left or Right key to go to the "Turn guidance on or off when recording" option and select it by pressing the Enter key.

· Use the Left or Right key to select "Guidance on" or "Guidance off" and press the Enter key to confirm your selection. 

14.7.  Connecting and selecting an input source.

When making live recordings of voices, conferences, lectures, presentations and concerts, you may find that you get better quality by connecting an external microphone. To record from devices such as tape recorders, Music CD players and radios, you should connect to PLEXTALK through the external microphone and line in socket, using a suitable audio cable. To connect either an external microphone or an audio cable, do the following:

· Insert the microphone or audio cable jack plug into the combined external microphone and line in socket.

· PLEXTALK will confirm that it is connected by saying "Microphone mode" or "Line in mode".  

· If PLEXTALK says "Microphone mode", this indicates that the "Select external audio input jack" setting is set for a microphone.

· If PLEXTALK says "Line in mode", this indicates that the setting is set for a line in input.

· If you have connected a microphone and PLEXTALK says "Line in mode", or if you have connected an audio cable and PLEXTALK says "Microphone mode", you need now to change the setting in the menu.  See later in this section for details on how to do this.

Note:  At this stage, PLEXTALK does not know if you have connected a microphone or an audio cable.  The speech that you hear only indicates what the "Select external audio input jack" setting is currently set to.

To change the external input source setting, do the following:

· Press the Menu key to access the Main Menu.

· Use the Left or Right key to go to the "Recording settings" option and select it by pressing the Enter key.

· Use the Left or Right key to go to the "Select external audio input jack" option and select it by pressing the Enter key.

· Use the Left or Right key to select either "Microphone mode" or "Line in mode" and press the Enter key to confirm your selection. 

CAUTION:  When you connect from the line output of a CD player to PLEXTALK's Line Input socket, the recorded sound may sometimes be distorted, even if you adjust the recording level.  If this happens, you should connect from the CD player's headphone socket to PLEXTALK's Line Input socket and then adjust the CD player's headphone output volume for recording.

14.8.  Selecting a recording mode and sound quality setting.

14.8.1.  About recording modes and sound quality.

When recording, PLEXTALK gives you the ability to select from 6 recording modes.  The 6 modes are as follows:

· Default; for making general recordings and voice recordings where there is relatively low background noise.

· Conference; for recording environments, such as conferences, lectures and classrooms, where the background noise is expected to be high.

· Audio; for recording from audio devices such as CD players.

· Analog cassettes; for recording from analogue cassettes.

· Radio; for recording from radio broadcasts.

· Custom; if none of the above options gives you the kind of recording that you want, this recording mode allows you to set all the parameters manually.

In addition PLEXTALK allows you to choose from a number of sound quality settings for each recording mode. By choosing a high quality setting, you reduce the available recording time on the SD card. By choosing a low sound quality, you can maximise the available recording time but the audio quality will not be so good. The available sound quality settings are as follows:

· PCM 44.1kHz stereo

· PCM 22.05kHz mono

· MP3 256kbps 44.1kHz stereo

· MP3 128kbps 44.1kHz stereo

· MP3 64kbps 44.1kHz mono

· MP3 32kbps 22.05kHz mono

Note: Technical data about recording modes and sound qualities are detailed in Appendix D.

14.8.2.  Selecting a preset recording mode.

You can either select a recording mode and sound quality setting before or after going into recording stand by. To choose a recording mode and sound quality setting before going into recording stand by, do the following:

· Press the Menu key once to access the Main Menu.

· Use the Left or Right key to go to the "Recording settings" option and select it by pressing the Enter key.

· Use the Left or Right key to go to the "Select recording mode" option and select it by pressing the Enter key.

· Use the Left or Right key to go to the mode you want and select it by pressing the Enter key.

· PLEXTALK will then offer you a chance to select a sound quality setting. Use the Left or Right key to move through the available options and select the one you want by pressing the Enter key.  PLEXTALK will confirm that your selection has been set.

To choose a recording mode and a sound quality setting after going into recording stand by, do the following:

· Press the Record key once and PLEXTALK will go into recording stand by.

· Press the Menu key once to access the Main Menu.

· Use the Left or Right key to go to the "Select recording mode" option and select it by pressing the Enter key.

· Use the Left or Right key to go to the mode you want and select it by pressing the Enter key.

· PLEXTALK will then offer you a chance to select a sound quality setting. Use the Left or Right key to move through the available options and select the one you want by pressing the Enter key.  
· PLEXTALK will confirm that your selection has been set. You will be returned to recording stand by when you have made your selection.

Note:  In both the situations above, the option you select will remain the default setting until you change it in the future.

14.8.3.  Selecting the Custom recording mode.

When you select the Custom recording mode, you have to specify what settings you want for each of the 5 recording parameters. To set the Custom recording mode, do the following:

· Press the Menu key once to access the Main Menu.

· Use the Left or Right key to go to the "Recording settings" option and select it by pressing the Enter key. As above, this is not necessary if you started this process from recording stand by.

· Use the Left or Right key to go to the "Select recording mode" option and select it by pressing the Enter key.

· Use the Left or Right key to go to the "Custom" option and select it by pressing the Enter key.

· PLEXTALK will then offer you a chance to select a sound quality setting. Use the Left or Right key to move through the available options and select the one you want by pressing the Enter key.

· You will now be given the chance to choose your preferred setting for each of the remaining 4 recording parameters in turn.

· PLEXTALK will speak the name of the parameter. Use the Left or Right key to go to the option you want and select it by pressing the Enter key.

· PLEXTALK will then speak the name of the next parameter. Repeat this process until you have selected a setting for the "Background noise level of input sound" parameter.

· When you have made this last selection, PLEXTALK will confirm that the settings have been set.  It will then exit the menu system and return you to the status you started the process from.

14.9.  Selecting the title to record into.

This section details how you can select to record into an existing title or into a new title. 
To record into an existing DAISY title, do the following:

· Navigate to an existing DAISY title on the SD card, as shown earlier, and stop playback.

· Press the Record key to go into recording standby.  

· Alternatively, you can select which title you wish to record into by using the Left and Right keys whilst in recording standby.  

· Press the Record key again to start recording. A new section will be created at the end of the current title. All new audio recorded will be appended to the end of the current title, regardless of where you were in the recording when you started this process.

To record into a new title, you can either create a new title first and then record into it or you can create a new title during the recording process. This section details how to create a new title during the recording process. Create a new DAISY title during recording as follows:

· Press the Record key to go into recording standby.

· Whilst in recording standby, use the Left and Right keys to select which title you wish to record into. As you move right you will find that PLEXTALK generates a title called "New title".

· Highlight the "New title" item and press the Record key. PLEXTALK will start recording into the new title.

Note: “New title” is created under the "PlexRecording" folder.

14.10.  Setting the recording level.

When making any recording, it is important to ensure that the recording level is set correctly. PLEXTALK gives you two methods of adjusting the recording level. You can either choose to have the level set automatically or you can set it manually. You can switch between Automatic Gain Control and manual recording level setting as follows:

· Press the Menu key to access the Main Menu.

· Use the Left or Right key to go to the "Recording settings" option and select it by pressing the Enter key.

· Use the Left or Right key to go to the "Auto Gain Control" option and select it by pressing the Enter key.

· Use the Left or Right key to select "Auto gain control off" or "Auto gain control on" and press the Enter key to confirm your selection. 

Note:  If you have turned on the Auto Gain Control, in recording standby, you will hear PLEXTALK say "Auto" repeatedly to indicate that the level will be set automatically.

You can manually adjust the recording level as follows:

· Set the Automatic Gain Control to "Off", using the method detailed above. This means that you have enabled manual setting of the recording level. The recording level can be set in the range 0 to 20 in increments of 1.

· Press the Record key once to go into recording stand by.

· Press the Up key.  Each time you press the key, the recording level will increase by one step.

· Press the Down key.  Each time you press the key, the recording level will decrease by one step.

· For larger adjustments of the recording level, you can press and hold down the key. PLEXTALK will continue to increase or decrease the level until you release the key or the maximum or minimum setting is reached.

· As you make the adjustments, PLEXTALK will announce the new recording level and will say either "Low", "Good" or "High" to indicate that the recording level is too low, about right or too high. You should aim to get PLEXTALK to say "Good" more often than "Low" or "High".

Note:  If the level monitor is off, you will not hear the "Low", "Good" or "High" announcements.

Note:  When you record audio material with the Auto Gain Control ON, you may not get the recording quality you want. In this situation, redo your recording with the auto gain control off and adjust the recording level manually.

14.11.  Using the level monitor.

The level monitor is PLEXTALK's way of indicating if the recording level is low, good or high or if it is being set automatically. PLEXTALK's default setting is for the level monitor to be on when you enter recording standby. The level monitor will be automatically muted when you subsequently start recording.

To turn the level monitor on or off at any time during recording standby, recording pause or during recording, do the following:

· Press the Title key once.

· PLEXTALK will either say, "Level monitor on" or "Level monitor off".

· Each press of the Title key will toggle between these two settings.

Note:  If the guidance during recording is turned off, PLEXTALK will not announce your key presses during recording. If you require this facility, you are advised to use headphones and turn on the guidance to avoid the level monitor being picked up in the recording.

14.12.  Turning the recording monitor on and off.

PLEXTALK enables you to monitor any recording as it is being made, so you can gauge how the recording will sound in the end. 

You can turn the recording monitor on or off in recording stand by, recording pause or during recording.  This is done as follows:

· Connect a set of stereo headphones to the headphone socket on the top edge of PLEXTALK.  

· Press the Bookmark key once to turn the recording monitor off.

· Press the Bookmark key again to turn the recording monitor on.

Note: You do not require headphones when recording from a line in source.

Note:  For all recordings, PLEXTALK will automatically turn on the recording monitor by default.

14.13.  Getting recording information.

You can get information about the current status of the recording by pressing the Number 5 key.

Note:  If voice guidance during recording is set to "Guidance off", you will not hear any of the above information during recording.  You will still hear the information in the recording stand by and recording pause states.

15. Editing a DAISY title.

15.1. Editing heading markers during recording.

Heading markers can be set either when recording is paused or during recording.  Both methods are detailed below:

· To set a heading during recording, press the GoTo key once. The new heading will be set immediately.

· To set a heading when recording is paused, press the GoTo key once.  The heading will be inserted when recording recommences.

· Whilst still in recording pause, you can cancel the heading by pressing the GoTo key a second time. 

Note:  PLEXTALK will automatically number headings sequentially from the beginning of the title.

15.2. Setting a heading after recording.

To set a heading after completing the recording, do the following:

· Navigate through your recording to the approximate location where you wish to set the heading.

· During playback or when the playback is stopped, press the Menu key once to access the Main Menu.

· Use the Left or Right key to go to the "Edit" option and select it by pressing the Enter key.

· Use the Left or Right key to go to the "Set heading" option and select it by pressing the Enter key.

· PLEXTALK will then tell you that you are about to set Heading XX and will ask you to set the position.

· You will then hear 3 seconds of audio played repeatedly from the location you are in the recording.

· Adjust the heading position by using your Left or Right key.

· Use these keys to move the heading position backwards or forwards by 0.1 second increments.

· When you are happy that the heading is positioned where you want it, press the Enter key to finish the process. PLEXTALK will confirm that the heading has been set.

Note:  Headings in the recording will be renumbered automatically from the beginning.

15.3. Cancelling a heading setting after recording.

To cancel a heading after completing the recording, do the following:

· Navigate to the location of the heading you wish to cancel.

· During playback or when the playback is stopped, press the Menu key once to access the Main Menu.

· Use the Left or Right key to go to the "Edit" option and select it by pressing the Enter key.

· Use the Left or Right key to go to the "Cancel heading" option and select it by pressing the Enter key.

· PLEXTALK will then ask you to confirm the cancellation of the specified heading. If you wish to cancel this heading, press the Enter key. PLEXTALK will confirm that the heading has been cancelled.

Note:  Remaining headings will be renumbered automatically from the beginning.

15.4. Removing a section or multiple sections.

PLEXTALK enables you to remove one or more sections from a DAISY title.  Do this as follows:

· Navigate to the title from which you want to remove one or more sections.

· Move back or forward in the title to the approximate position of the first section you wish to remove.

· Press the Menu key once to access the Main Menu.

· Use the Left or Right key to go to the "Edit" option and select it by pressing the Enter key.

· Use the Left or Right key to go to the "Remove section" option and select it by pressing the Enter key.

· PLEXTALK will now ask you to select the first section to be removed. Use the Left or Right key to move back or forward in the title until you hear the section and select it by pressing the Enter key.

· PLEXTALK will now ask you to select the last section that you want to remove. Use the Left or Right key to move back or forward in the title until you hear the section and select it by pressing the Enter key.

· Finally, PLEXTALK will ask you to confirm that you wish to proceed.  Press Enter to confirm the action or Cancel to abort.

Note:  If you only wish to remove a single section, simply press Enter on the same section for both the start section and end section and proceed as for removing multiple sections.

Note:  You can only remove multiple sections that are contiguous. To remove multiple sections that are not contiguous, you need to remove each section individually.

Note:  PLEXTALK will automatically reorder the heading numbering for the remaining sections in the title.

16.  Media management features.

16.1. Erasing an SD card.

PLEXTALK allows you to erase an SD card as many times as you want.  To erase an SD card, do the following:

· Press the Menu key once to access the Main Menu.

· Use the Left or Right key to go to the "Media Management" option and select it by pressing the Enter key.

· Use the Left or Right key to go to the "Delete SD card" option and select it by pressing the Enter key.

· PLEXTALK will ask you to confirm that you wish to continue with the formatting process.  Press the Enter key again to continue or the Cancel key to abort.

· If you choose to continue, the formatting process will start. PLEXTALK will announce when the process is complete.

Note:  If the SD card is erased, all data on the SD card will be deleted.

16.2. Creating a new title.

To create a new title on the SD card when not in recording standby, do the following:

· Press the Menu key once to access the Main Menu.

· Use the Left or Right key to go to the "Media management" option and select it by pressing the Enter key.

· Use the Left or Right key to go to the "Create a new title" option and select it by pressing the Enter key.

· PLEXTALK will then ask you to confirm that you wish to proceed. Press the Enter key to confirm or the Cancel key to abort.

Note: New title is created under the "PlexRecording" folder.

16.3. Deleting a title.

PLEXTALK enables you to delete a title on an SD card or USB. To delete a title, do the following:

· Go to the title that you wish to delete.

· Press the Menu key once to access the Main Menu.

· Use the Left or Right key to go to the "Media management" option and select it by pressing the Enter key.

· Use the Left or Right key to go to the "Delete the selected title" option and select it by pressing the Enter key.

· PLEXTALK will then ask you to confirm that you wish to proceed. Press the Enter key to confirm or the Cancel key to abort.

16.4. Deleting a file

You can delete a file of “Music”, “Audio book” or “Podcast”.  To delete a file, do the following:

· Go to the file that you wish to delete.

· Press the Menu key once to access the Main Menu.

· Use the Left or Right key to go to the "Media management" option and select it by pressing the Enter key.

· Use the Left or Right key to go to the "Delete a file" option and select it by pressing the Enter key.

· PLEXTALK will then ask you to confirm that you wish to proceed. Press the Enter key to confirm or the Cancel key to abort.

Note: If you delete the Audio Book from "Audible", use the menu option "Delete selected title".

16.5. Preparing to edit a DAISY title.

If you have already built a recorded title or, if you have received a title from elsewhere and you wish to edit it, you first need to prepare the title for editing. To prepare a title for editing, do the following:

· Go to the title you wish to prepare for editing.

· Press the Menu key once to access the Main Menu.

· Use the Left or Right key to go to the "Media Management" option and select it by pressing the Enter key.

· Use the Left or Right key to go to the "Prepare for editing" option and select it by pressing the Enter key.

· PLEXTALK will speak a message asking you if you wish to proceed. If you wish to continue, press the Enter key or press the Cancel key to abort the process.

· During the process, PLEXTALK will speak progress messages and will announce when it is complete. If you wish to cancel the process whilst it is running, press the Cancel key.

CAUTION: When the battery level is low, the process may be aborted. Therefore, it is recommended that you connect to a mains power source before performing this process.

Note: If you prepare a title for editing, you will be able to record or edit this title.

Note: If you prepare for editing a title in the category of "DAISY book", it remains in this category.

16.6. Building a DAISY title.

The Build book process converts a DAISY title, recorded on the PLEXTALK, into a DAISY book that can be played on other DAISY hardware players and on a personal computer using DAISY playback software.  To perform the Build book process, do the following:

· Go to the title you wish to build.

· Press the Menu key once to access the Main Menu.

· Use the Left or Right key to go to the "Media Management" option and select it by pressing the Enter key.

· Use the Left or Right key to go to the "Build book" option and select it by pressing the Enter key.

· PLEXTALK will speak a message asking you if you wish to proceed. If you wish to continue, press the Enter key or press the Cancel key to abort the process.

· PLEXTALK will speak progress messages and will announce when it is complete. If you wish to cancel the process whilst it's running, press the Cancel key.

CAUTION: When the battery level is low, the process may be aborted. Therefore, it is recommended that you connect to a mains power source before performing this process.

Note: If you build a title, you will be unable to record or edit this title. When you would like to record or edit this title once more, prepare this title for editing.

Note: If you build a title in the category of " Recordings", it remains in this category.

Note:  The Build book process, for a DAISY title, will require between 30MB and 40MB of space on the SD card.

16.7. Network folder function

By using the network folder function, your personal computer can access to the SD card in the PLEXTALK via Wireless LAN network as a network shared folder.  To start network folder function, do the following:

· Press the Menu key. 

· Use the Left or Right key to go to the "Media Management" option and press the Enter key.
· Use the Left or Right key to go to the "Network folder" option and press the Enter key. 

· Network connection of network folder is started and PLEXTALK will announce the network status when it connected. 

· Then you can access the SD card in PLEXTALK from your personal computer.

To finish the Network folder function, do the following:

· Press the Cancel key. 

· PLEXTALK will ask you to confirm that you wish to finish. 

· Press the Enter key to finish the function or the Cancel key to continue.

Note: Refer to section 20 "Network settings" for details to use Wireless LAN network.

17. Using PLEXTALK's Voice Memo functions.

In addition to using PLEXTALK as a DAISY player, music player and as a DAISY recorder, you are also able to use it for recording simple Voice Memos. 

17.1. Recording a new Voice Memo.

To record a new Voice Memo, do the following:

· Press and hold down the Record key and speak the memo.

· Release the key when you are finished.

· PLEXTALK will insert a new Voice Memo into the internal memory and will then return you to the title you were in before recording the new Voice Memo.

Note:  You cannot record a continual Voice Memo that lasts for more than 1 minute. When you exceed 1 minute, PLEXTALK automatically stops recording.

Note:  The total amount of available recording time for Voice Memos is 30 minutes. The maximum number of individual memos is 200.

Note:  If the recorded time exceeds 30 minutes, or the number of memos exceeds 200, you will hear a warning message. You should then delete any unnecessary Voice Memos or backup your Voice Memos to an SD card.

Note:  For Voice Memos, the audio compression is fixed at MP3 32kbps monaural.

17.2. Reviewing existing Voice Memos.

You can review your existing Voice Memos either sequentially or by going direct to a specific Voice Memo.  To listen to your existing Voice Memos sequentially, do the following:

· Press the Title key repeatedly until PLEXTALK says "Voice Memo".

· Use the Left or Right key to move sequentially through the existing Voice Memos.

· If you wish to hear the current Voice Memo again, press the Play/Stop key.

To go direct to a specific Voice Memo, do the following:

· Press the GoTo key once.

· Use the Number Pad to enter the number of the required Voice Memo, followed by the Enter key.

Note:  You can go direct to the last Voice Memo by pressing the GoTo key and entering 0 followed by the Enter key.

17.3. Back up Voice Memos to SD card.

Please refer to Section 18.2. for information about Back up Voice Memos.

17.4. Deleting Voice Memos.

If you wish to delete a single Voice Memo, do the following:

· Navigate to the Voice Memo you wish to delete, as detailed previously.

· Press the Menu key once to access the Main Menu.

· Use the Left or Right key to go to the "Voice Memo" option and select it by pressing the Enter key.

· Use the Left or Right key to go to the "Delete the selected Voice Memo" option and press the Enter key.

· PLEXTALK will ask you to confirm that you wish to continue. Press the Enter key again to continue or the Cancel key to abort.

If you wish to delete all Voice Memos, do the following:

· Press the Menu key once to access the Main Menu.

· Use the Left or Right key to go to the "Voice Memo" option and select it by pressing the Enter key.

· Use the Left or Right key to go to the "Delete all Voice Memos" option and press the Enter key.

· PLEXTALK will ask you to confirm that you wish to continue. Press the Enter key again to continue or the Cancel key to abort.

18. Backup

18.1. General backup information.

Before backing up any data, the following points should be noted:

· You can backup a variety of data from SD card to external USB media, or vice versa. However, an external USB CD/DVD drive can only be used as a source for backup purposes.

· In the back up process, PLEXTALK gives a verbal warning if there is insufficient space available on the target media.

· If you backup from a Music CD using an external USB CD/DVD drive, you can choose the audio compression to be used. It will be backed up as Audio Files and not in DAISY format.

· When the battery level is low, the backup process may be aborted. Therefore, it is recommended that you connect to a mains power source before backup.

· If you wish to cancel the backup process whilst it is running, press the Cancel key.

· Podcast episodes and Web radio cannot be backed up.

· Data will be backed up into these folders:

DAISY: "PlexBackup" folder

Recordings: "PlexBackup" folder

Music: "PlexBackup" folder

Text: "PlexBackup" folder

".aa" Audio book: "Backup" folder under the "Audible" folder

Other Audio book: "Backup" folder under the "PlexAudioBook" folder 

18.2. Back up Voice Memos.

This section details how to backup all Voice Memos from the internal memory to SD card.

· Insert an SD card with sufficient free space.

· Press the Title key to Navigate to Voice Memo and press the Enter key.

· Press the Number 9 key once to access the Backup feature.

· PLEXTALK will ask you to confirm that you wish to continue with the backup. Press the Enter key to continue or the Cancel key to abort.

· If you choose to continue, the backup process will start. PLEXTALK will announce when the backup is complete.

Note:  The internal memory for Voice Memos can only be backed up into "PlexBackup" folder of SD card.  

Note:  Voice Memos are saved as a DAISY title with level 1 navigation.  Each memo is saved as a separate section.

Note:  When backing up Voice Memos, all the Voice Memos are backed up.  

You cannot back up a particular Voice Memo.

18.3. Backup a DAISY title, album or text from an SD card to an external USB media.

To backup a DAISY title, album or text from an SD card to an external USB media, do the following:

· Connect an external USB media with sufficient free space.

· Insert the source SD card.

· Navigate, on the SD card, to the title, album or text that you wish to backup, and press the Enter key.

· Press the Number 9 key once to access the Backup feature.

· PLEXTALK will ask you to confirm that you wish to continue with the backup. Press the Enter key to continue or the Cancel key to abort.

· If you choose to continue, the backup process will start. PLEXTALK will announce when the backup is complete.

Note:  You cannot back up to an external USB CD/DVD drive.

18.4. Backup a DAISY title, album or text from an external USB media to an SD card.

To backup a DAISY title, album or text from an external USB media to an SD card, do the following:

· Insert an SD card with sufficient free space and ensure that it is unlocked.

· Connect the external USB media.

· Navigate, on the external USB media, to the title, album or text that you wish to backup, and press the Enter key.

· Press the Number 9 key once to access the Backup feature.

· When backing up from a Music CD, press the Left or Right key to go to your preferred audio compression and press the Enter key to confirm your selection.

· When backing up from a Music CD, PLEXTALK will ask you to which category you are going to backup, "Music" or "AudioBook". Use the Left or Right key to select your preferred category and press the Enter key.

· PLEXTALK will ask you to confirm that you wish to continue with the backup. Press the Enter key to continue or the Cancel key to abort.

· If you choose to continue, the backup process will start. PLEXTALK will announce when the backup is complete.

18.5. Backup time for music CD.

For the range of available sample rates or bit rates, the approximate backup times are given below:

Backing up about 50 minutes of audio from a music CD to SD card:

· PCM 44.1kHz Stereo ; 35 minutes approximately.

· MP3 256K Stereo High Quality ; 50 minutes approximately.

· MP3 256K Stereo ; 40 minutes approximately.

· MP3 128K Stereo High Quality ; 55 minutes approximately.

· MP3 128K Stereo ; 40 minutes approximately.

Note:  All times are approximate. The backup time will depend on the size of the source title and numbers of files. It will also depend on the writing speed of the SD card.

19. Using PLEXTALK as an SD card drive.

You can connect PLEXTALK to a personal computer, using the USB (Universal Serial Bus) 2.0 interface, to use it as an external card drive. This chapter describes how to connect and disconnect PLEXTALK to computers that have a range of operating systems.

Note:  To use PLEXTALK as an external memory card drive, your personal computer's operating system must be Windows 2000, XP, Vista or Windows 7.

19.1. What can PLEXTALK be used for when connected to a personal computer?

With PLEXTALK connected to a computer as a card drive, you can perform standard file management functions such as:

· Transfer DAISY titles, Audio Files and Text Files between the SD card and the computer.

· Create, delete and rename folders on the SD card.

· Delete and rename files on the SD card.

· Transfer audio files from a computer to an SD card, and listen to them using the computer, in other words, use PLEXTALK as a file store for music files.

19.2. Connecting PLEXTALK to a computer as an external drive.

To connect PLEXTALK to a computer, use the following procedure:

· Turn on your computer system.

· Connect mains power to the PLEXTALK and turn it on.

· Connect the USB cable(long) from PLEXTALK to the computer directly.

· The computer recognizes PLEXTALK as an external memory card drive automatically.

CAUTION:  When PLEXTALK is connected to a computer, as an external memory card drive, you should always avoid Switching off PLEXTALK, Unplugging the USB cable or removing the SD card. Failure to do so may result in damage to or loss of data.

Note:  As soon as PLEXTALK recognizes the connection to the computer, it becomes an external memory card drive for the computer and loses all its player and recorder functionality.

19.3. Disconnecting PLEXTALK from a computer.

When disconnecting PLEXTALK from a computer, use the procedure below:

· Left click the "USB" or "Safely remove hardware" icon located in the System Tray at the lower right of the computer screen.

· Choose this drive at the Safely Remove Hardware option in the System Tray.

Note:  If you have difficulty finding or clicking icon in the Task Tray, shut down the computer and PLEXTALK, and then disconnect the cable.

20. Network settings

PLEXTALK has a network capability by using wireless LAN.

WARNING: Refrain from using the wireless LAN function in an aircraft or a hospital according to the instructions of an airline company or a hospital.

20.1. Connecting to the wireless network.

20.1.1. Enables network device

To enable network device, do the following:

· Press the Menu key.

· Use the Left or Right key to go to "System settings" option and press the Enter key.

· Use the Left or Right key to go to "Network settings" option and press the Enter key.

· Use the Left or Right key to go to "Enable or disable network device" option and press the Enter key.

· Use the Left or Right key to select "Enable" and press the Enter key.

20.1.2. Confirm available wireless network and connect.

Confirm currently available wireless network from "Available wireless network" list.

· Press the Menu key.

· Use the Left or Right key to go to "System settings" option and press the Enter key.

· Use the Left or Right key to go to "Network settings" option and press the Enter key.

· Use the Left or Right key to go to "Wireless network settings" option and press the Enter key.

· Use the Left or Right key to go to "Available wireless network" option and press the Enter key.

· PLEXTALK announces a name, link status and signal strength of one of the currently available networks. Press the Left or Right key to confirm information on another network

· Select the wireless network you want to connect to and press the Enter key.

If you select a wireless network that has been configured, PLEXTALK tries to connect to the network. If you select a wireless network that is not configured, do the following:

· Select "security mode". Use the Left or Right key to select a security mode and press the Enter key, then enter a password. Refer to the section 22 "Entering text" for details of Entering text. After you have finished, press the Down key to confirm. If "No security" is selected, entering the password is not required.

· Select "stealth setting". If the wireless network uses stealth function, select "ON". If it does not, select "OFF". Then press the Enter key. 

· Select "DHCP setting". Use the Left or Right key to select "Enable" for automatic setting or "Disable" manual setting and press the Enter key. If "Enable" is selected, setup menu proceeds for proxy setting.

· Enter IP address. One IP address consists of 4 areas. PLEXTALK reads numbers of the first area. If you want to change the numbers, enter new numbers by numerical keys then press the Right key to go to the next area. If you do not want to change the numbers, simply press the Right key.
· Enter numbers from the second area to the fourth area. Press the Right key to go to the next area. To go back to the previous area, press the Left key. Once entering is completed, press the Down key.

· Enter subnet mask. Do the same method as entering of IP address.

· Enter default gateway. Do the same method as entering of IP address.

· Enter DNS setting. Primary and secondary DNS settings are available. Do the same method as entering of IP address.

· Select "proxy server setting". To use a proxy server, press the Left or Right key to select "Enable" and then press the Enter key. If you do not use a proxy server, select "Disable" and press the Enter key. 

· If "Disable" is selected and the Enter key is pressed, menu is closed.

· If "Enable" is selected, enter the address of the proxy server and press the Down key when the entering is finished. 

· Then enter a port number of the proxy server and press the Down key. PLEXTALK tries to connect to the wireless network.

· The menu automatically closes when connection is succeeded. If the connection is failed, PLEXTALK automatically returns to the menu to select available wireless network.

Note: Network settings such as a Network name (SSID) and a security mode depend on each network. Setup the network settings by following your network environment.

Note: If WEP is selected as security mode, a password is 5, 10, 13, or 26 characters. "For security mode besides WEP, a password is between 8 and 64 characters.
Note: If you select a network that is currently connected, PLEXTALK disconnects the network and tries to connect to a network in order of priority set in PLEXTALK. 

PLEXTALK asks whether to disconnect the network, press the enter key to disconnect or press the cancel key to cancel. Refer to "20.3.5 Change or delete wireless network setting" for more detail about priority.

20.2. How PLEXTALK connect to network

PLEXTALK automatically tries to connect to the network as long as "Enable or disable network device" in the menu is "Enable".

Note: Refer to "20.1.1. Enables network device" to enable or disable the network device.

PLEXTALK first tries to connect to a network where it was connected at the last time. If it has failed to connect, PLEXTALK tries to connect to another network in order of priority set in PLEXTALK. Refer to "20.3.5 Change or delete wireless network setting" for more detail about priority.

After the network function is not used for more than five minutes without AC mains power, it may take a few seconds to a few minutes at most for PLEXTALK to resume the network function. This is because PLEXTLK turned the network function into power saving mode and tries to reconnect to the network.

20.3. Advanced Network settings

20.3.1. Confirming network information

Information about current network connection can be confirmed by this menu. The following information is available:

· Access point link status

· Signal strength

· DHCP

· IP address

· Subnet mask

· Default gateway

· Primary DNS

· Secondary DNS

· MAC address of wireless network

· Date of obtained DHCP lease

· Expiry date of DHCP lease

· DHCP server address

If PLEXTALK does not linked to network, the following information is available.

· Link status

· MAC address of wireless network

To confirm the above information, do the following:

· Press the Menu key

· Use the Left or Right key to go to "System settings" option and press the Enter key.

· Use the Left or Right key to go to "Network settings" option and press the Enter key.

· Use the Left or Right key to go to "Confirm network information" option and press the Enter key.

· PLEXTALK announces information. Use the Left or Right key to select the information you want to listen to.

20.3.2. Loading network setting

PLEXTALK can configure multiple network settings at once by loading network settings, which is prepared by personal computer. Application software writes settings on a file called "NetworkSetting.ini". Place this file on to the root of SD card or USB device, then load the settings by "Load network setting" menu.

· Press the Menu key.

· Use the Left or Right key to go to "System settings" option and press the Enter key.

· Use the Left or Right key to go to "Network setting" option and press the Enter key.

· Use the Left or Right key to go to "Load network setting" option and press the Enter key.

· PLEXTALK will ask you whether to proceed for loading file. Press the Enter key to proceed or the Cancel key to cancel.

Note: All old settings are deleted when new network settings are loaded.

Note: If SD card and USB device are connected to the PLEXTALK and both contain "NetworkSetting.ini" respectively, the file in the SD card is loaded.

Note: You can generate the network settings file by using the network setting file generation software. The software and the manual are available at PLEXTALK web site.  

http://www.plextalk.com/in/

20.3.3. Saving the network setting

Network settings in the PLEXTALK can be saved to SD card or USB device for backup purpose. To save settings, do the following:

· Press the Menu key.

· Use the Left or Right key to go to "System setting" option and press the Enter key.

· Use the Left or Right key to go to "Network setting" option and press the Enter key.

· Use the Left or Right key to go to "Save network setting". 
· PLEXTALK will ask you whether to proceed for saving the setting file or not. Press the Enter key to proceed. The file is saved to the root of SD card or USB device. To cancel, press the Cancel key.

Note: If both of SD card and USB　device are connected, the setting file is saved to the root of SD card.

20.3.4. Adding wireless network setting

To add wireless network setting, do the following:

· Press the Menu key.

· Use the Left or Right key to go to "System settings" option and press the Enter key.

· Use the Left or Right key to go to "Network setting" option and press the Enter key.

· Use the Left or Right key to go to "Wireless network settings" option and press the Enter key.

· Use the Left or Right key to go to "Add wireless network setting" option and press the Enter key.

· Select "Security mode". Steps for setting are the same as "Available wireless network". Refer to the section. "20.1.2. Confirm available wireless network and connect".

· After all settings are completed, the added wireless network is registered as the highest priority network. Then the menu is closed and PLEXTALK will try to connect to the added network.

20.3.5. Change or delete wireless network setting

Three options, "Give highest priority and connect", "Change wireless network setting" and "Delete wireless network setting" are available in the "Change or delete wireless network setting" menu. To change or delete wireless network setting, do the followings:

· Press the Menu key.

· Use the Left or Right key to go to "System settings" option and press the Enter key.

· Use the Left or Right key to go to "Network setting" option and press the Enter key.

· Use the Left or Right key to go to "Wireless network settings" option and press the Enter key.

· Use the Left or Right key to go to "Change or delete wireless network setting" option and press the Enter key,

· Use the Left or Right key to select the wireless network whose setting you want to change and press the Enter key. Wireless network is listed in order of priority of the connection.

· Use the Left or Right key to select "Give highest priority and connect", "Change wireless network setting" or "Delete wireless network setting" and press the Enter key.

If "Give highest priority and connect" is selected:

· PLEXTALK will ask you to give highest priority to the selected wireless network. Press the Enter key to give. Then PLEXTALK will try to connect to the network.

If "Change wireless network setting" is selected:

· Select "Security mode". Steps for setting are the same as "Available wireless network". Refer to the section. "20.1.2. Confirm available wireless network and connect".

· After all settings are completed, the updated settings are registered, then menu is closed. Even the setting is updated but priority is kept as same as before.

Note: To change SSID of the network setting, delete the network setting first, then add a new network setting again.

If "Delete wireless network setting" is selected:

· PLEXTALK will ask you whether to proceed for delete, then press the Enter key to delete. After deleting is completed, PLEXTALK will reconnect to the highest priority network.

20.3.6. Network folder setting

You can configure the Network device name, User ID and password. Default values are:

· Network device name: PTP1-(Last 4 digits of serial number), for example "PTP1-1234" 

· User ID: No user ID

· Password: No password

To change the settings of network folder, do the following:

· Press the Menu key. 

· Use the Left or Right key to go to the "System settings" option and press the Enter key.

· Use the Left or Right key to go to the "Network settings" option and press the Enter key.

· Use the Left or Right key to go to the "Network folder setting" option and press the Enter key.

· PLEXTALK announces the network devise name, user ID, and password. PLEXTALK will ask you whether you would like to change them. Press the Down key if you would like to change any one of them.

· PLEXTALK will announce the network device name. If you accept the current name, just press the Down key. If you give it a new name, use the Number pad to enter the new name. Enter the new name within 15 characters using 0-9, A-Z, a-z, and "-" (dash). At least one alphabet should be contained. After you have finished, press the Down key to confirm.

· To confirm the name, PLEXTALK will announce the name by word and then spells the name. For example, if you entered "cat12", PLEXTALK will announce "cat twelve, c, a, t, one, two". For another example, if you entered "abc", PLEXTALK will announce "abc, a, b, c".

· PLEXTALK will announce user ID. If you accept the current user ID, just press the Down key. If you give it a new user ID, enter the new user ID using the Number pad. Enter the user ID within 32 characters using 0-9, A-Z, a-z,  "-" (dash) and "_" (line). After you have finished, press the Down key to confirm. If you delete the present user ID and press the Down key, menu will be closed after setting the default user ID and password.

· PLEXTALK will announce password. If you accept the current password, just press the Down key. If you give it a new password, enter the new password up to 32 characters using the Number pad. After you have finished, press the Down key to confirm. The setting will be completed and PLEXTALK will exit the menu system.

Note: Refer to the section 22 "Entering text" for details of entering text.

20.3.7. Initializing network settings to default

Network settings in  "Enable or disable network device",  "Wireless network settings" and "Network folder setting" can be reset to factory default.  To reset the network settings to default, do the following:

· Press the Menu key.

· Use the Left or Right key to go to "System setting" option and press the Enter key.

· Use the Left or Right key to go to "Network settings" option and press the Enter key.

· Use the Left or Right key to go to "Initialize all network settings to default" option and press the Enter key.

· PLEXTALK will ask you whether to proceed for reset the network setting. Press the Enter key to reset or the Cancel key to cancel.

Note: After the reset, network device will turn to disabled.

21. Editing text files

PLEXTALK provides you to write documents and save it as a text file. This chapter gives the information on creating and saving a text file. Text input modes are detailed in the Chapter 22. 

21.1. Creating a new text file

To create a new text file, do the followings;
· Press the Menu key to access the Main Menu.

· Use the Left or Right key to select "Media management" option and press the Enter key.

· Use the Left or Right key to select "Create a new text file" option and press the Enter key.

· PLEXTALK will ask you to confirm that you wish to continue. Press the Enter key to continue. A new text file will be created and now you are in the Text editing mode. 

21.2. Editing an existing text file on the SD card

To start editing an existing text file, do the followings;
· Press the title key to select SD card and then press the Left or Right key to select the text file you want to edit. 

· Press the Menu key to access the Main Menu.

· Use the Left or Right key to select "Edit" option and press the Enter key.

· PLEXTALK will ask you to confirm that you wish to continue. Press the Enter key to continue. Now you are able to edit the text file.

Note: You can not edit the text files on the USB device.

21.3. Saving the edited text

To save the edited text, do the followings;
· Press the Menu key.

· Use the Left or Right key to select "Save the edited text" option and press the Enter key.

· PLEXTALK will ask you to confirm that you wish to continue. Press the Enter key to continue or Cancel key to abort.

· When the Enter key is pressed, PLEXTALK saves the edited text then returns to text editing mode.

Note: File name will be the first line of the text. If the text has nothing at the time of saving file, current date and time is applied as the file name.

21.4. Exiting text editing

21.4.1. Saving and exiting text editing

To save and exit text editing, do the followings;
· Press the title key.

· PLEXTALK will ask you to confirm that you wish to save and exit text editing. Press the Enter key to exit or Cancel key to continue text editing.

· When the Enter key is pressed, PLEXTALK saves the text and exits text editing.

Alternatively, do the followings;
· Press the Menu key.

· Use the Left or Right key to select "Save and exit text editing" option and press the Enter key.

· PLEXTALK will ask you to confirm that you wish to proceed. Press the Enter key to proceed or Cancel key to abort.

· When the Enter key is pressed, PLEXTALK saves the text and exits text editing.

21.4.2. Exiting text editing without saving the text

To exit the text editing without saving the text, do the followings;
· Press the Menu key.

· Use the Left or Right key to select "Exit text editing without saving" option and press the Enter key.

· PLEXTALK will ask you to confirm that you wish to continue. Press the Enter key to continue or Cancel key to abort.

· If the Enter key is pressed, PLEXTALK discards the edited information then the text is restituted.

21.5. Renaming the file

To rename the file, do the followings;
· Press the Menu key.

· Use the Left or Right key to select "Rename the file" option and press the Enter key.

· Edit the file name.

· Press the Down key and PLEXTALK will ask you to confirm that you wish to continue. 

· Press the Enter key to continue or Cancel key to abort.

Note: Please refer to Chapter 22 for more detail about entering characters.

Note: Press the Play/Stop key to listen to the new filename.

21.6. Deleting current line

To delete the current line, do the followings;
· Press the Menu key.

· Use the Left or Right key to select "Delete the current line" option and press the Enter key.

· PLEXTALK will ask you to confirm that you wish to continue. Press the Enter key to continue or Cancel key to abort.

21.7. Deleting all text

To delete all text, do the followings;
· Press the Menu key.

· Use the Left or Right key to select "Delete all text in the file" option and press the Enter key.

· PLEXTALK will ask you to confirm that you wish to continue. Press the Enter key to continue or Cancel key to abort.

22. Entering text

This chapter describes how to enter characters.

22.1. Entering alphabet by ABC mode

Multiple characters are mapped to each numeric key and one of the character is selected each press.

< Order of character mapping >

Characters mapped to a numeric key appear in order as “lower cases, number, upper cases”. For example, pressing 5 key to switch the characters in order of “j, k, l, 5, J, K, L”.

< Character is confirmed after two seconds automatically >

After selecting a character, the character is automatically confirmed after two seconds inactivity and PLEXTALK announces the character.

< Entering multiple characters mapped to a key continuously >

To enter multiple characters mapped to a key, press the Right key to enter next character. For example, to enter “ab”, enter “a” by pressing the number 2 key once, and press the Right key, then enter “b” by pressing 2 key twice.

< Example of operation >

Entering "http"

h : Press the 4 key twice.

t : Press the 8 key once.

t : Press the 8 key once.

p : Press the 7 key once.

< Characters and numbers available for input >

The characters mapped to each numeric key are as follows:

1 contains ".", "@", "/", ":", "1"

2 contains a, b, c, 2, A, B, and C.

3 contains d, e, f, 3, D, E, and F

4 contains g, h, i, 4, G, H, and I

5 contains j, k, l, 5, J, K, and L

6 contains m, n, o, 6, M, N, and O

7 contains p, q, r, s, 7, P, Q, R, and S.

8 contains t, u, v, 8, T, U, and V.

9 contains w, x, y, z, 9, W, X, Y, and Z.

0 contains ?, _, =, and 0.

< Confirming entered characters >

Press the Play/Stop key to listen to the entered text.

< Changing character type >

To change type of characters, press the bookmark key to switch between “ABC mode”, “Number mode” and “Symbol table”.

Note: If the input allows number only, the mode is not switched even if the bookmark key is pressed.

22.2. Entering numbers by number mode

Number mode is the mode to enter numbers by pressing numerical keys. Press the bookmark key several times to switch to the Number mode, then enter numbers.

Enter numbers by pressing numeric keys. The 1 to 0 keys correspond to the numbers.

22.3. Entering symbols by Symbol table

Symbol table is the mode to enter symbols by using the number 2, 4, 6 and 8 keys to scroll characters on the symbol table below and pressing the Pound/Hash(#) key to enter the character. Press the bookmark key several times to switch to the symbol table, then enter symbols.

To scroll the characters on the symbol table, press the number 2 key to scroll up, press the number 8 key to scroll down, press the number 4 key to scroll left and press the number 6 key to scroll right. Symbol table is described below. To enter a symbol several times, press the Pound/Hash(#) key repeatedly.

	.
	,
	'
	"
	:
	;
	?
	|

	+
	-
	=
	/
	\
	_
	[
	]

	!
	@
	#
	$
	%
	`
	{
	}

	^
	&
	*
	(
	)
	~
	<
	>


22.4. Moving entering position

Move back a character

Press the Left key to move back a character.

Move ahead a character

Press the Right key to move ahead a character.

Move back a word

Press the Left key and number 0 key together to move back a word. 

Move ahead a word

Press the Right key and number 0 key together to move ahead a word.

Move back to the beginning of the line

Press and hold the Left key to move back to the beginning of the line. 

Alternatively press the Star key and 5 key together.

Move ahead to the end of the line

Press and hold the Right key to move ahead to the end of the line.

Alternatively press the Star key and 8 key together.  

Move back a line

Press the Up key to move back a line.

Move ahead a line

Press the Down key to move ahead a line.

Move back a screen (40 lines up)

Press the Star key and 6 key together to move back 40 lines.

Move ahead a screen (40 lines down)

Press the Star key and 9 key together to move ahead 40 lines.

Move back a paragraph

Press the Up key and number 0 key together key to move back a paragraph. 

Move ahead a paragraph

Press the Down key and number 0 key together to move ahead a paragraph.

Move back to the beginning of the text file

Press and hold the Up key to move back to the beginning of the text file. 

Move ahead to the end of the text file

Press and hold the Down key to move ahead to the end of the text file.

22.5. Enter space, linefeed and erase characters

Space (Blank character)

Press the Hash key to enter Space (Blank character).

Backspace (Delete the previous character)

Press the Star key to delete the previous character.

Deleting a character at current position

Press the Star key and 7 key together to delete the current character.

Deleting all characters in the Setting menu

Press and hold the Star key

Deleting all characters during editing text file

Refer to the section “21.7. Deleting all text”.

Deleting current line

Refer to the section “21.6. Deleting current line”.

New line

Press the Star key and Hash key together to begin a new line.

Tab character

Press the Star key and 2 key together to enter a tab character.

Switching "insert" mode and "overwrite" mode

Press the Star key and 4 key together to switch the "insert" mode and "overwrite" mode.

23. Other useful features.

23.1. Accessing the built in help file.

 To access the built in help file, do the following:

· Press the Number 0 key once.

· Navigate the file in the same way that you navigate other DAISY text titles.
To exit the help file, do the following:

· Press the Number 0 key once.

23.2. Checking system properties.

To find out the system version or the serial number of your PLEXTALK and PDTB key information, do the following:

· Press the Menu key once to access the Main Menu.

· Use the Left or Right key to go to the "System settings" option and select it by pressing the Enter key.

· Use the Left or Right key to go to the "System properties" option and select it by pressing the Enter key.

· The system version, serial number will be played repeatedly. Also, the PDTB key information will be played only if you have installed the key. 

· When you have noted the information that you require, press the Enter key to exit the menu system.

23.3. Deleting authorization keys

To delete all authorization keys that have been registered on Plextalk, do the followings:

· Press the Menu key.

· Use the Left or Right key to go to the "System settings" option and press the Enter key.

· Use the Left or Right key to go to the "Delete authorization keys" option and press the Enter key.

· PLEXTALK will ask you to confirm that you wish to continue with the process. Press [1234] by number keys and press the Enter key to delete the keys.

Note: You need to press 1, 2, 3, 4 with exact order.

Note: “Authorization keys” are keys to play the DAISY books that are protected by PDTB format. PDTB is a digital encryption technology that is formulated by DAISY consortium.

23.4. Reset PLEXTALK to factory defaults.

To reset all settings back to the factory defaults, do the following:

· Press the Menu key once to access the Main Menu.

· Use the Left or Right key to go to the "System settings" option and select it by pressing the Enter key.

· Use the Left or Right key to go to the "Initialize all settings to default" option and select it by pressing the Enter key.

· PLEXTALK will ask you to confirm that you wish to proceed. Press the Enter key to set all settings to factory defaults or the Cancel key to abort. After a short while, PLEXTALK will restart.

The following is a list of the default settings for the PLEXTALK. 

· Select waiting sound:  Waiting sound 1

· Text to speech setting, pitch:  0

· Audio source:  External microphone

· Recording mode:  Default

· Auto section setting:  Off

· Background noise level:  Normal

· Time to auto stop recording:  Off

· Trigger of starting recording:  When record key is pressed.

· Auto Gain Control:  On

· Guidance while recording:  Guidance on

· Voice bookmark:  Play

· Recording volume:  10

· Playback setting:  Standard play

· Playback speed:  0

· Playback tone:  0

· Guidance volume:  0

· Playback volume:  13

· Reading audio file name ：Read audio file name

· Multimedia Daisy playback setting： Audio

Note: Bookmarks are deleted by initializing all settings to default.

Note: Network settings are not deleted by initializing all settings to default. Refer to the section 20.3.7 "Initializing network settings to default"

Appendix A - Main Specifications.

Note:  Shinano Kenshi reserves the right to modify the products specification at any time without prior notice.

This section details the main specifications for the PLEXTALK Pocket PTP1.

General specifications.

SD card slot:

SD/SDHC card

Playable contents:

Books conforming to DAISY 2.0, 2.02 and 3.0 standards

Audio files in MP3, WMA, AMR-WB+, Ogg Vorbis, Wave (PCM) formats and Audible Format 4(.aa).

Music CDs (CD-DA)(External USB CD/DVD drive)

Text file : plain text, html, Microsoft .doc, .docx

Web radio

Supported playlist file type: PLS, M3U, ASX

Playable Audio format: MP3, WMA

Streeming format(protocol): HTTP, mms

Recording methods:

DAISY 2.02

Recording audio formats for DAISY 2.02:

PCM 44.1kHz Stereo

PCM 22.05kHz Mono

MP3 256kbps Stereo

MP3 128kbps Stereo

MP3 64kbps Mono

MP3 32kbps Mono

Recording audio formats for Voice Memo:

MP3 32kbps Mono

Audio input and output:

Headphone terminal (Stereo)

External microphone/ Line input terminal (Stereo)

Built in microphone (Monaural)

Built in speaker (Monaural)

Externally connected terminal:

USB 2.0

USB OTG (USB bus power)

Clock precision:

Approximately 60 seconds per month

Electric Power Source:

AC adaptor, AC 100 V to 240 V, 50 or 60 Hz

Dimensions and weight:

Length 112 mm

Width 55 mm

Height 16 mm

Weight Approx.  110g

Power consumption (maximum):

20W

Working temperature and humidity:

Fahrenheit from 40 to 104

Celsius from 5 to 40

Humidity from 20% to 80%, no moisture.

Audio features.

Headphones (line output):

Compatible impedance 32 ohm unbalanced

Jack 3.5mm diameter stereo mini jack

External Microphone (Plug-in power):

Input impedance 2.2k ohm unbalanced

Jack 3.5mm diameter stereo mini jack

Line Input:

Input impedance 20k ohm unbalanced

Jack 3.5mm diameter stereo mini jack

Built In Speaker:

Output impedance 8 ohm

Output 150mW

Build-in Microphone:

Mode Electric condenser microphone

Main functions.

Guide voice:

Gives spoken information about the current operation

Provides key descriptions in Key Describer mode

Speaks menu items

Gives help information (press the Number 5 key)

Search function:

Heading, Group, Page, Phrase, Bookmark, Album, Track, Level, Screen, Paragraph, Line, Sentence, Spell, Word, Character

Volume adjustment:

0 to 25 (in increments of 1)

Guide volume adjustment:

 -5 to +5 (in increments of 1)

Play speed adjustment:

11 settings from -2, through 0, to +8 (in increments of 1).

Normal speed is at 0.

 -2 is half speed, +4 is double speed, +8 triple speed.

Tone adjustment:

13 settings from -6, through 0, to +6 (in increments of 1)

Normal tone is at 0

Recording time (for SD card, 2 gigabytes):

(Note:  All times are approximate)

DAISY PCM 44.1kHz stereo 3 hours

DAISY PCM 22.05kHz mono 12 hours

DAISY MP3 256kbps stereo 15 hours

DAISY MP3 128kbps stereo 30 hours

DAISY MP3 64kbps mono 60 hours

DAISY MP3 32kbps mono 120 hours

Recording level adjustment:

Select input between External Microphone and Line input

0 to 20 (in increments of 1)

Available Auto Gain Control 

Memory function:

Bookmarks and voice Bookmarks, maximum is 10000

Voice Bookmarks, Total 30 minutes recording

Voice Memo, Total 30 minutes recording, 200 items

Battery specifications.

Type:  Lithium polymer

Dimensions:

Length 40 mm

Width 75 mm

Height 20 mm

Capacity:  1500 milli-ampere hour

Charging time: 

4 hours approximately via an AC adapter

4.5 hours approximately via a USB cable

Note: The charging via a USB cable works during power off.

Operating time:

DAISY format, MP3 recording, 8 hours

DAISY format, MP3 playing, 10 hours

Note:  All times are approximate.

Note:  The charging time and operating time vary depending on the ambient temperature and working conditions.

Memory Card Drive Specifications.

Card type:  SD/SDHC card

File system:  FAT-32/16

Operating systems:  Windows 2000, XP, Vista or Windows 7

Wireless communication function

Wireless LAN: IEEE802.11b/g

Copyright information about Ogg Vorbis.

This software is provided by the copyright holders and contributors "as is" and any express or implied warranties, including, but not limited to, the implied warranties of merchantability and fitness for a particular purpose are disclaimed. In no event shall the foundation or contributors be liable for any direct, indirect, incidental, special, exemplary, or consequential damages (including, but not limited to, procurement of substitute goods or services; loss of use, data, or profits; or business interruption) however caused and on any theory of liability, whether in contract, strict liability, or tort (including negligence or otherwise) arising in any way out of the use of this software, even if advised of the possibility of such damage. 

Appendix B - Menu Hierarchy.

This appendix lists all menu items in the Main Menu along with their corresponding sub menu hierarchy.

EDIT:

Set heading

Cancel heading

Remove section

EDIT (text file): 

Delete the current line

Delete all text in the file

Save the edited text

Save and exit text editing

Exit text editing without saving

Rename the file

RECORDING SETTINGS:

Select recording mode

Default

Conference

Audio

Analog cassettes

Radio

Custom

Then select sound quality from the following

PCM 44.1kHz stereo

PCM 22.05kHz mono

MP3 256kbps 44.1kHz stereo

MP3 128kbps 44.1kHz stereo

MP3 64kbps 44.1kHz mono

MP3 32kbps 22.05kHz mono.

Turn guidance on or off when recording

Guidance on

Guidance off

Select external audio input jack

Microphone mode

Line in mode

Auto gain control

Auto gain control on

Auto gain control off

PLAYBACK SETTINGS:

Shuffle and repeat settings

Standard play

Repeat title (DAISY, Text, Audio book) 

Repeat track (Music)

Repeat album (Music)

Repeat entire albums (Music)

Shuffle repeat (Music)

Reading audio file name

Read audio file name

Read audio track number

Multimedia DAISY playback setting

Audio

Text

BOOKMARK:

Playback setting for Voice Bookmark

Voice Bookmark on

Voice Bookmark off

Clear all bookmarks in current title

VOICE MEMO:

Delete the selected voice memo

Delete all voice memos

PODCAST:

Update the episode list

Download all episodes of the feed

MEDIA MANAGEMENT:

Media title information

Delete SD card

Create a new title

Create a new text file

Delete the selected title (Delete the selected album, Delete all Voice Memos)

Delete a file

Prepare for editing

Build book

Network folder

SYSTEM SETTINGS:

System properties

System version

Serial number

Installed key (if obtained.)

Select the text to speech voice

Voice 1

Voice 2

Select the waiting sound

Waiting sound 1

Waiting sound 2

No waiting sound

Select the key press confirmation sound

Key press confirmation sound on

Key press confirmation sound off

Set the system date and time

Set the text to speech pitch

Network settings

Confirm network information

Load network settings

Save network settings

Enable or disable network device

Wireless network settings

Available wireless network

Add wireless network setting

Change or delete wireless network setting

Give highest priority and connect

Change wireless network setting

Delete wireless network setting

Network folder setting

Initialize all network settings to default

Delete authorization keys

Initialize all settings to default

Appendix C - Troubleshooting.

General operating problems.

Symptom:  You turn on the Power Switch, but the power does not come on.

Possible cause 1:  Loose power cord or AC adaptor.

Solution:  Firmly reinsert the mains power cable into PLEXTALK and ensure that the AC adaptor is firmly inserted.

Possible cause 2:  Key Lock is On.

Solution:  Turn off the key lock.

Symptom:  The power is on, but PLEXTALK does not operate.

Possible cause:  Key Lock is On.

Solution:  Turn off the key lock.

Symptom:  PLEXTALK will not shut down and no keys can be operated.

Possible cause 1:  Key Lock is On.

Solution:  Turn off the key lock.

Possible cause 2:  Unknown error.

Solution:  Press and hold down the Power key for more than 10 seconds.  

If this does not work, remove the AC Adaptor and Battery from the unit and then reinsert them.

Symptom:  Cannot record to an SD card.

Possible cause 1:  Fault with the input.

Solution:  Check the input source and then correct the input setting.

Possible cause 2:  There is no available space on the SD card.

Solution:  Either delete some unnecessary contents from the current SD card or insert a different SD card with sufficient space.

Possible cause 3:  The SD card is locked.

Solution:  Remove the SD card in the correct way, switch the lock to the "Unlocked" position and reinsert it into PLEXTALK.

Symptom:  The recording level cannot be adjusted.

Possible cause:  You have the Auto Gain Control turned on.

Solution:  Turn the Auto Gain Control off in the "Recording settings" menu.

Symptom:  There is no voice guidance.

Possible cause 1:  The volume has been set to '0'.

Solution:  Increase the volume so that the guide speech can be heard.

Possible cause 2:  The Turn guidance on or off when recording" option is set to "Guidance off".

Solution:  Change the setting to "Guidance on".

Spoken errors during DAISY playback.

Guide voice:  "Unsupported SD device"

Possible cause:  The SD card has been formatted with an unsupported file system.

Solution:  Insert an SD card which is formatted as FAT-16 or FAT-32.

Guide voice:  "Read error."

Possible cause:  Title cannot be played back.

Solution:  Press the Title key followed by the Left or Right key to try to select a different title.

Errors using the Voice Memo.

Guide voice:  "Not enough space in the voice memo area."

Possible cause:  Not enough internal memory for recording a new Voice Memo.

Solution:  Free up some memory by deleting unnecessary Voice Memos.  You may also wish to backup your memos before carrying out this process.

Errors when using PLEXTALK as an SD card drive on a computer.

Symptom:  Drive does not work.

Possible cause 1:  The USB cable is not connected correctly.

Solution:  Reinsert the USB cable.  Check the cable, the connector and whether the pins are dented, bent or damaged.

Possible cause 2:  The power is not on.

Solution:  Check that the power cord and AC adaptor are securely connected and that PLEXTALK itself is turned on.

Symptom:  Cannot write to the SD card.

Possible cause 1:  The SD card is locked.

Solution:  Eject the SD card, move the lock to the unlocked position, reinsert it and try again.

Possible cause 2:  The power is not on.

Solution:  Check that the Power cord and AC adaptor are securely connected, and turn on the Power Switch.

Possible cause 3:  The USB cable has not been connected correctly.

Solution:  Reinsert the USB cable.  Check the cable, the connector and whether the pins are dented, bent or damaged.

Symptom:  Nothing can be added to the SD card.

Possible cause 1:  There is not enough space on the SD card for added data.

Solution:  Free up some space on the SD card and try again.

Possible cause 2:  The SD card is locked.

Solution:  Eject the SD card, move the lock to the unlocked position, reinsert it and try again.

Appendix D -  Technical data about recording modes and sound qualities.

1.  Recording mode settings.

Each recording mode has predetermined settings for the following 4 parameters:

· Auto section setting; PLEXTALK can be set to automatically create a new section, in a DAISY recording, after a specified amount of time with no audio input, for example 2 seconds. Use the setting "No setting" to create a continuous recording with no new sections.

· Time to auto stop recording when no sound; This is the amount of time that PLEXTALK will continue to record with no audio input. After this time, recording stops automatically.

· Trigger of starting recording; Recording can be set to start either when you press the Record Key or when input audio is detected.

· Background noise level of input sound; If you are recording in a low noise environment, such as a studio, use the "low" setting. If your recording environment is noisy, such as a conference or lecture, use the "High" setting.

2.  Settings for preset recording modes.

The preset parameters for the 5 recording modes are as follows:

Default mode:

· Auto section setting:  Off

· Time to auto stop recording when no sound:  Off

· Trigger of starting recording:  When record key is pressed.

· Background noise level of input sound:  Normal (-28dB)

Conference mode:

· Auto section setting:  Off

· Time to auto stop recording when no sound:  Off

· Trigger of starting recording:  When record key is pressed.

· Background noise level of input sound:  High (-20 dB)

Audio mode:

· Auto section setting:  2 seconds

· Time to auto stop recording when no sound:  1 minute

· Trigger of starting recording:  When record key is pressed.

· Background noise level of input sound:  Low (-40 dB)

Analogue Cassette mode:

· Auto section setting:  4 seconds

· Time to auto stop recording when no sound:  1 minute

· Trigger of starting recording:  When sound is detected.

· Background noise level of input sound:  Normal (-28 dB)

Radio mode:

· Auto section setting:  Off

· Time to auto stop recording when no sound:  1 minute

· Trigger of starting recording:  When sound is detected.

· Background noise level of input sound:  Normal (-28 dB)

3.  Available settings for Custom mode.

In Custom mode you can set each parameter to the settings listed below:

· Auto section setting can be set to:  "Off", "1 second", "2 seconds", "3 seconds", "4 seconds" or "5 seconds".

· Time to auto stop recording when no sound can be set to:  "Off", "30 seconds", "1 minute", "2 minutes", "3 minutes", "4 minutes" or "5 minutes".

· Trigger of starting recording can be set to either "When record key is pressed" or "When sound is detected".

· Background noise level of input sound can be set to:  "Normal (-28 dB)", "Low (-40 dB)" or "High (-20 dB)".

Note:  The settings for the Default, Conference, Audio, Analogue cassette and Radio modes cannot be changed.  The settings for the Custom mode can be changed to suit your requirements.

Note:  When recording with the built in microphone, the "Background noise" setting will automatically be set to "High".

4.  Available recording time for the specified sound qualities.

The available recording time will depend on the sound quality setting you select. The higher the sound quality, the less recording time. Below are listed the approximate recording times you can expect to get with 4 media types.

PCM 44.1kHz stereo:

· SD card, 1GB:  90 minutes

· SD card, 2GB:  3 hours

· SD card, 4GB:  6 hours

· SD card, 8GB:  12 hours

PCM 22.05kHz mono:

· SD card, 1GB:  6 hours

· SD card, 2GB:  12 hours

· SD card, 4GB:  24 hours

· SD card, 8GB:  48 hours

MP3 256 k stereo:

· SD card, 1GB:  7.5 hours

· SD card, 2GB:  15 hours

· SD card, 4GB:  30 hours

· SD card, 8GB:  60 hours

MP3 128 k stereo:

· SD card, 1GB:  15 hours

· SD card, 2GB:  30 hours

· SD card, 4GB:  60 hours

· SD card, 8GB:  120 hours

MP3 64 k mono:

· SD card, 1GB:  30 hours

· SD card, 2GB:  60 hours

· SD card, 4GB:  120 hours

· SD card, 8GB:  240 hours

MP3 32 k mono:

· SD card, 1GB:  60 hours

· SD card, 2GB:  120 hours

· SD card, 4GB:  240 hours

· SD card, 8GB:  480 hours

Note:  The above recording times are approximations and may vary depending on the number of headings and other structure in the recording.
Appendix E - Questions and Answers on Network setting
	Trouble / Question
	Solution / Answer

	I want to know the IP address of PLEXTALK
	You can confirm the IP address from "Confirm network information" option of the network setting menu.

	PLEXALK does not select your desired access point.
	You can confirm list of available access point by wireless network setting option in the network setting.

When PLEXTALK cannot find access point, please confirm status of network power has been set ON.

If the access point is listed, please confirm the priority of the access point. You can set the highest priority from "Set highest priority and connect" option of the network setting menu.

	It failed to find access point.

I am using access point that support IEEE802.11a
	PLEXTALK only support IEEE802.11b/g. If your access point cannot use IEEE802.11a and IEEE802.11b/g in same time, please select IEEE802.11b/g.



	It failed to connect access point.

I am using non broadcasted SSID.
	Non broadcasted SSID require a long time to establish a connection between PLEXTALK. Please allow the PLEXTALK up to 5 minutes to see if it finally succeeds or not.

If possible, please try to configure your access point setting to enable broadcasting the SSID name. When it succeeds in connecting after broadcast setting, that means your network configuration is fine, but non-broadcast setting is problem. If it failed even with broadcasted SSID, please confirm wireless security setting.

	It failed to connect access point.

I am using DHCP.
	Even if PLEXTALK could establish the wireless connection between access point, but when it failed to retrieve IP address, connection failed message will show up. In this case, please try fixed IP address. If it succeeds in connecting, DHCP server is problem. Please confirm your DHCP server setting. If it still fails to connect, please confirm wireless security setting.

	It failed to connect access point.

I am using MAC address filtering.
	When MAC address filtering is enabled in access point security setting, it refuses a connection of un-configured network device. You need to configure your access point to add PLEXTALK MAC address. You can confirm MAC address of PLEXTALK by "Confirm network information" option of the network setting menu.

	PLEXTALK always tries connection to not available SSID.

I have configured Stealth SSID settings also.
	When you have configured a network setting with Stealth SSID enabled. PLEXTALK tries connection to stealth (= non broadcasted SSID) always.

Please set Stealth SSID access point to low priority in the list of wireless settings.



	Plextalk cannot connect to the network service.

I am using WEP security with Fixed IP address.
	When you are using WEP security with fixed IP address setting, PLEXTALK will succeed in connecting to the access point even it has a wrong password.

Please confirm WEP password again.

	Does PLEXTALK support WPS (WIFI Protected Setup)?
	WPS is not supported. You need to configure the network setting by yourself.

	It failed to connect to the access point. 
	Please try and restart the access point and PLEXTALK.


Appendix F - List of text input

ABC mode

	Key name
	Assignment

	#1
	Symbols

“.”, “@”, “/”, “:”, “1”

	#2
	a, b, c, 2, A, B, C,

	#3
	d, e, f, 3, D, E, F,

	#4
	g, h, i, 4, G, H, I,

	#5
	j, k, l, 5, J, K, L,

	#6
	m, n, o, 6, M, N, O,

	#7
	p, q, r, s, 7, P, Q, R, S,

	#8
	t, u, v, 8, T, U, V,

	#9
	w, x, y, z, 9, W, X, Y, Z,

	#0
	 “?”, “_”, “=”, “0”

	*
	Backspace

	* + #
	New line

	#
	Space

	Play/stop
	Play start from cursor position / pause

	Upper arrow
	Push: Previous line

Hold: Top of document

	Down arrow
	Push: Next line

Hold: End of document

	Left arrow
	Push: Previous character

Hold: Home

	Right arrow
	Push: Next character

Hold: End

	0 + upper arrow
	Previous paragraph

	0 + down arrow
	Next paragraph

	0 + left arrow
	Previous word

	0 + right arrow
	Next word

	* + 1
	Save and exit editing mode

	* + 2
	Tab

	* + 3
	Backspace

	* + 4
	Insert / Overwrite

	* + 5
	Home

	* + 6
	screen up (40 lines up)

	* + 7
	Delete

	* + 8
	End

	* + 9
	screen down (40 lines down)

	Bookmark
	Select input mode from

ABC mode / Number mode / Symbol table


Symbol table

	Key name
	Assignment

	#1
	Not used

	#2
	Up

Go to the uppermost part of the table

	#3
	Not used

	#4
	Left

Hold: Go to the leftmost part of the table

	#5
	Check currently selected symbol

	#6
	Right

Hold: Go to the rightmost part of the table

	#7
	Not used

	#8
	Down

Hold: Go to the lowermost part of the table

	#9
	Not used

	#0
	Not used

	#
	Input the Symbol

	(Symbols)
	Table has 8 Column * 4 row

. , ‘ “ : ; ? |

+ - = / \ _ [ ]  

! @ # $ % ` { }

^ & * ( ) ~ < >


* Other keys are same as ABC mode.

2011/03 UG- ENG0601-0






130
1

